3GPP TSG-RAN WG4 Meeting # 106bis-e                           R4-2305383

Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-18 NTN coexistence study assumption

Agenda Item:	5.27.2

Document for:	Approval

Introduction

In RAN4#106 meeting, a WF [1] for above 10GHz NTN-TN co-existence study was approved with a lot of agreements and some open issues for further discussion. In last meeting, the phased array antenna parameters were proposed by companies in contribution [2], but companies question whether these parameters can meet the link budget for LEO scenarios. In this paper, we’d like to compare these parameters of phased array antenna with the parameters specified in TR 38.821 and clarify the rationality for these phased array antenna parameters.

In addition, the relationship between NTN network and TN network drafted in Annex of WF [1] haven’t been dis

Discussion on SAN and UE with phased array antenna

The phased array antenna parameters

In last meeting, the following UE phased array antenna parameters were proposed. The maximum array gain is 10*log10 (1600) = 32dB. In addition to the maximum antenna element gain, the final antenna gain is 32+5.2dB = 37.2dB. As 8dBm conductive power per antenna element is assumed, with the consideration of 1600 elements, the total maximum output power is assumed as 40dBm.

Table 2-1 UE phased array antenna characteristics

| Ka band UE phased array antenna |  |  |
| --- | --- | --- |
| 1 | Ka band UE phased array antenna Characteristics |  |
| 1.1 | Antenna pattern | TR 38.921 |
| 1.2 | Element gain (dBi) (Note 2) | 5.2 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90° for H 90° for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 dB |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Row × Column)(Note 4) | 40*40 elements |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.5 of wavelength for V |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 8 dBm |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees |
| 1.13 | normal direction | Toward Z+ axis |

Note 1:	Void

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Row × Column ×Number of supported polarizations elements (i.e. power per H/V polarized element).

Note 4:	Row × Column means there are Row vertical and Column horizontal radiating elements.

Similarly, the following SAN phased array antenna parameters for LEO600 were proposed in last meeting. The maximum array gain is 10*log10 (441) = 26.4dB. In addition to the maximum antenna element gain, the final antenna gain is 26.4+4dB = 30.4dB. As 10dBm conductive power per antenna element is assumed, with the consideration of 441 elements, the total maximum output power is assumed as 36.4dBm.

Table 2-2 SAN phased array antenna characteristics for LEO600

| Ka band SAN phased array antenna |  |  |
| --- | --- | --- |
| 1 | Ka band SAN phased array antenna Characteristics |  |
| 1.1 | Antenna pattern | TR 38.921 |
| 1.2 | Element gain (dBi) (Note 2) | 4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90 for H 90 for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Row × Column)(Note 4) | 441 elements (21*21) |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 10dBm |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 54 degrees |
| 1.13 | normal direction | Toward Z- axis |

SAN LEO1200 antenna elements can be twice as many as SAN LEO configuration. Thus, the following SAN phased array antenna characteristics for LEO1200 were proposed. The maximum array gain is 10*log10 (900) = 29.5dB. In addition to the maximum antenna element gain, the final antenna gain is 29.5+4dB = 33.5dB. As 10dBm conductive power per antenna element is assumed, with the consideration of 900 elements, the total maximum output power is assumed as 39.5dBm.

Table 2-3 SAN phased array antenna characteristics for LEO1200

| Ka band SAN phased array antenna |  |  |
| --- | --- | --- |
| 1 | Ka band SAN phased array antenna Characteristics |  |
| 1.1 | Antenna pattern | TR 38.921 |
| 1.2 | Element gain (dBi) (Note 2) | 4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90 for H 90 for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Row × Column)(Note 4) | 900 elements (30*30) |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 10dBm |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 54 degrees |
| 1.13 | normal direction | Toward Z- axis |

Note 1:	Void

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Row × Column ×Number of supported polarizations elements (i.e. power per H/V polarized element).

Note 4: Row × Column means there are Row vertical and Column horizontal radiating elements.

Referring to TR 38.821, Set-1 satellite parameters and VSAT parameters for Ka band were defined in table 2-4 and table 2-5.

Table 2-4 SAN characteristics specified in TR 38.821

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| Note 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. Note 2: This beam size refers to the Nadir pointing of the satellite Note 3: All these satellite parameters are applied per beam. Note 4: The EIRP density values are considered identical for all frequency re-use factor options. Note 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Table 2-5 VSAT characteristics specified in TR 38.821

| Characteristics | VSAT (Note 2) | Other (Note 1) |
| --- | --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) | Ka band(i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | circular | Linear : +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | TBD dBi per element |
| Antenna temperature | 150 K | TBD K |
| Noise figure | 1.2 dB | TBD dB |
| Tx transmit power | 2 W (33 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | TBD dBi per element |
| Note 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. Note 2: VSAT terminal characteristics could be implemented with phased array antenna |  |  |

Based on the UL/DL link budget, we compared two sets of parameters, i.e. phased array antenna and parabolic antenna.

Table 2-6 Comparison between phased array antenna and parabolic antenna in DL link budget

| The proposed parameters based on phased array antenna | 600km case | 1200km case | TR 38.821 Ka band Parameters | 600km case | 1200km case |
| --- | --- | --- | --- | --- | --- |
| SAN TRP(dBm) | 36.4 | 39.4 | Satellite EIRP density (dBW/MHz) | 4 | 10 |
| SAN antenna array gain | 26.4 | 29.4 | SAN antenna array gain | NA | NA |
| SAN antenna element gain | 4 | 4 | SAN antenna element gain | NA | NA |
| SAN antenna gain | 30.4 | 33.4 | Satellite EIRP dBm | 57.01 | 63.01 |
| Channel Bandwidth(MHz) | 200 | 200 | Channel Bandwidth(MHz) | 200 | 200 |
| center frequency (GHz) | 17 | 17 | center frequency (GHz) | 17 | 17 |
| distance (m) | 600000 | 1200000 | distance (m) | 600000 | 1200000 |
| free space Pathloss (dB) | 172.62 | 178.64 | free space Pathloss (dB) | 172.62 | 178.64 |
| NTN UE antenna array gain | 32 | 32 | NTN UE antenna array gain | NA | NA |
| NTN UEantenna element gain | 5.2 | 5.2 | NTN UEantenna element gain | NA | NA |
| NTN UE antenna gain | 37.2 | 37.2 | NTN UE antenna gain | 45 | 45 |
| Received signal (dBm) | -68.62 | -68.64 | Received signal (dBm) | -70.61 | -70.63 |
| Noise figure (dB) | 5.9 | 5.9 | Noise figure (dB) | 5.9 | 5.9 |
| Thermal Noise (dBm) | -85.08 | -85.08 | Thermal Noise (dBm) | -85.08 | -85.08 |
| SNR (dB) | 16.46 | 16.44 | SNR (dB) | 14.47 | 14.45 |

Based on the comparison of DL link budget, the proposed phased array antenna parameters can achieve the similar DL performance as parabolic antenna parameters defined in TR 38.821.

Observation 1: Based on the comparison of DL link budget, the proposed phased array antenna parameters can achieve the similar DL performance as parabolic antenna parameters defined in TR 38.821.

Table 2-7 Comparison between phased array antenna and parabolic antenna in UL link budget

| The proposed parameters based on phased array antenna | 600km case | 1200km case | TR 38.821 Ka band | 600km case | 1200km case |
| --- | --- | --- | --- | --- | --- |
| NTN UE TRP(dBm) | 40 | 40 | NTN UE TRP | 33 | 33 |
| NTN UE antenna array gain | 32 | 32 | SAN antenna array gain | NA | NA |
| NTN UE antenna element gain | 5.2 | 5.2 | SAN antenna element gain | NA | NA |
| NTN UE antenna gain | 37.2 | 37.2 | NTN UE antenna gain | 43.2 | 43.2 |
| Channel Bandwidth(MHz) | 200 | 200 | Channel Bandwidth(MHz) | 200 | 200 |
| center frequency (GHz) | 27 | 27 | center frequency (GHz) | 27 | 27 |
| distance (m) | 600000 | 1200000 | distance (m) | 600000 | 1200000 |
| free space Pathloss (dB) | 176.64 | 182.66 | free space Pathloss (dB) | 176.64 | 182.66 |
| SAN antenna array gain | 26.4 | 29.4 | NTN UE antenna array gain | NA | NA |
| SAN antenna element gain | 4 | 4 | NTN UEantenna element gain | NA | NA |
| SAN antenna gain | 30.4 | 33.4 | NTN UE antenna gain | 38.5 | 38.5 |
| Received signal (dBm) | -69.04 | -72.06 | Received signal (dBm) | -61.94030029 | -67.9609002 |
| Noise figure (dB) | 5.9 | 5.9 | Noise figure (dB) | 5.9 | 5.9 |
| Thermal Noise (dBm) | -85.08 | -85.08 | Thermal Noise (dBm) | -85.08970004 | -85.08970004 |
| SNR (dB) | 16.04 | 13.02 | SNR (dB) | 23.14 | 17.12 |

Based on the comparison of UL link budget, the expected UL SNR for proposed phased array antenna parameters is lower than the parabolic antenna parameters defined in TR 38.821. However, due to smaller Rx antenna gain, the NTN UE with proposed phased array antenna parameters may transmit much higher output power. From this perspective, the UL simulation based on the proposed phased array antenna parameters can reflect a worse scenario.

Observation 2: Based on the comparison of UL link budget, the expected UL SNR for proposed phased array antenna parameters is lower than the parabolic antenna parameters defined in TR 38.821. However, due to smaller Rx antenna gain, the NTN UE with proposed phased array antenna parameters may transmit much higher output power. From this perspective, the UL simulation based on the proposed phased array antenna parameters can reflect a worse scenario.

Phased array antenna pattern

In clause 8.1.1 of TR 38.921, the phased array antenna pattern is copied as below. In Table 2-8, the parameters used by the parameterized array antenna model are described.

Table 2-8: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Horizontal HPBW | 3dB | Degrees |
| Vertical HPBW | 3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Number of radiating elements rows and columns | (M, N) | Integer |
| Horizontal element separation | dh | m |
| Vertical element separation | dv | m |
| Electrical down-tilt angle | etilt | Degrees |
| Electrical scan angle | escan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 2-9.

Table 2-9: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan | dBi |

And this model can meet the ITU-R recommendation [3] shown as below.

The following reference radiation patterns should be adopted for angles between the direction considered and the axis of the main beam for frequencies in the range from 2 to 31 GHz:

G    32  –  25 log 		dBi		for  min          48°

  –10				dBi		for    48°        180°

where:

min    1° or 100 /D degrees, whichever is the greater, for D/ ≥ 50.

min =  2° or 114 (D/)–1.09 degrees, whichever is the greater, for D/ < 50.

Observation 3: Phased array antenna model specified in clause 8.1.1 of TR 38.921 can meet the requirements defined in ITU-R S.465-6.

Proposal

Proposal 1: To assume the following phased array antenna parameters for SAN and UE.

Table 2-1 UE phased array antenna characteristics

| Ka band UE phased array antenna |  |  |
| --- | --- | --- |
| 1 | Ka band UE phased array antenna Characteristics |  |
| 1.1 | Antenna pattern | TR 38.921 |
| 1.2 | Element gain (dBi) (Note 2) | 5.2 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90° for H 90° for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 dB |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Row × Column)(Note 4) | 40*40 elements |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.5 of wavelength for V |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 8 dBm |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees |
| 1.13 | normal direction | Toward Z+ axis |

Table 2-2 SAN phased array antenna characteristics for LEO

| Ka band SAN phased array antenna |  |  |  |
| --- | --- | --- | --- |
| 1 | Ka band SAN phased array antenna Characteristics | LEO600 | LEO1200 |
| 1.1 | Antenna pattern | TR 38.921 | TR 38.921 |
| 1.2 | Element gain (dBi) (Note 2) | 4 | 4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90 for H 90 for V | 90 for H 90 for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 | 30 |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Row × Column)(Note 4) | 441 elements (21*21) | 900 elements (30*30) |
| 1.7 | Number of supported polarizations, P | 1 | 1 |
| 1.8 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V | 0.5 of wavelength for H 0.5 of wavelength for V |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 10dBm | 10dBm |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 360 degrees | 360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 54 degrees | 54 degrees |
| 1.13 | normal direction | Toward Z- axis | Toward Z- axis |

Note 1:	Void

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Row × Column ×Number of supported polarizations elements (i.e. power per H/V polarized element).

Note 4: Row × Column means there are Row vertical and Column horizontal radiating elements.

Discussion on the relationship between NTN network and TN network

In the previous meetings, RAN4 hasn’t discussed the relationship between NTN network and TN network. In the WF [1], the outcomes in Rel-17 were listed in Annex as reference. Since the order of scenarios is different between Rel-17 and Rel-18. Before discussing, it’s better to align the order of Rel-18 scenarios. It’s recommended to use the approved WF for the scenarios to discuss this issue. The approved scenarios in Rel-18 are listed as below.

Study NTN-TN coexistence by assuming a reference frequency of 17 GHz for NTN DL cases and 27 GHz NTN UL cases, as well as the consideration of ACLR and ACS assumptions as following:

| No. | Combination | Aggressor | Victim | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2 | TN with NTN | TN UL | NTN UL | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3 | TN with NTN | NTN UL | TN DL | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4 | TN with NTN | TN DL | NTN UL | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5 | TN with NTN | TN DL | NTN DL | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6 | TN with NTN | NTN DL | TN DL | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7 | TN with NTN | NTN DL | TN UL | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8 | TN with NTN | TN UL | NTN DL | ACLR TN UE fixed ACS NTN UE to be varied/defined |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |

Proposal 2: After aligning the order of Rel-18 scenarios, it’s recommended to discuss the following relationship between TN and NTN for Ka band coexistence study.

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: Option 1: NTN UEs dropped at the edge of TN clusters (NTN UE outside of the TN cluster) Option 2: NTN UEs dropped in TN clusters (NTN UE inside TN cluster) Option 3: NTN UEs dropped in or at the edge of TN clusters with a possibility (Random number). FFS on possibility. | TN randomly placed in this NTN beam | Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the active TN clusters which contain the NTN UE(s). Option 3: All active TN clusters no matter whether the NTN UE(s) is (are) included in the TN cluster or at the edge of TN clusters. |
| 2 | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Follow case 1. | Consider an active rate of [20%] for TN. | All active TN cells in central NTN beam |
| 3 | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: Follow case 1. | TN clusters randomly placed in this NTN beam | Follow case 1. |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: Follow case 1. | TN clusters randomly placed in this NTN beam | Follow case 1. |
| 4 | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Follow case 1. | Consider the active rate of [20%] for TN. | Follow case 1. |
| 5 | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Option 1: NTN UEs dropped at the edge of TN clusters (NTN UE outside of the TN cluster) Option 2: NTN UEs dropped in TN clusters (NTN UE inside TN cluster) Option 3: NTN UEs dropped in or at the edge of TN clusters with a possibility (Random number). FFS on possibility. | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the active TN clusters which contain the NTN UE(s). Option 3: All active TN clusters no matter whether the NTN UE(s) is (are) included in the TN cluster or at the edge of TN clusters. |
| 6 | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: Follow case 5 conclusion | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values) NTN UE: Follow case 5 conclusion |  |  |
| 7 | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: Follow case 5 conclusion | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: Follow case 5 conclusion |  |  |
| 8 | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Follow case 5 conclusion. | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Follow case 5 conclusion |

Discussion on ACLR1 and ACLR2 model for NTN UE.

In last meeting, RAN4 has the following conclusion for the number of active NTN UE (UL).

- For calibration, the number of active NTN UE (UL) is 10 UEs with divided RBs per UE by channel bandwidth.

However, it seems that RAN4 never discuss the ACLR1/ACLR2 model for Ka band UE. Generally, the ACLR2 value is higher than ACLR1 since the 2nd adjacent channel is far away from 1st adjacent channel.

Proposal 3: It’s proposed that RAN4 further discuss how much value the ACLR2 for Ka band UE is higher than ACLR1.

Summary

Based on the discussion above, the following observations and proposals are listed.

Observation 1: Based on the comparison of DL link budget, the proposed phased array antenna parameters can achieve the similar DL performance as parabolic antenna parameters defined in TR 38.821.

Observation 2: Based on the comparison of UL link budget, the expected UL SNR for proposed phased array antenna parameters is lower than the parabolic antenna parameters defined in TR 38.821. However, due to smaller Rx antenna gain, the NTN UE with proposed phased array antenna parameters may transmit much higher output power. From this perspective, the UL simulation based on the proposed phased array antenna parameters can reflect a worse scenario.

Observation 3: Phased array antenna model specified in clause 8.1.1 of TR 38.921 can meet the requirements defined in ITU-R S.465-6.

Proposal 1: To assume the following phased array antenna parameters for SAN and UE.

Table 2-1 UE phased array antenna characteristics

| Ka band UE phased array antenna |  |  |
| --- | --- | --- |
| 1 | Ka band UE phased array antenna Characteristics |  |
| 1.1 | Antenna pattern | TR 38.921 |
| 1.2 | Element gain (dBi) (Note 2) | 5.2 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90° for H 90° for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 dB |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Row × Column)(Note 4) | 40*40 elements |
| 1.7 | Number of supported polarizations, P | 1 |
| 1.8 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.5 of wavelength for V |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 8 dBm |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 0~360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 0~60 degrees |
| 1.13 | normal direction | Toward Z+ axis |

Table 2-2 SAN phased array antenna characteristics for LEO

| Ka band SAN phased array antenna |  |  |  |
| --- | --- | --- | --- |
| 1 | Ka band SAN phased array antenna Characteristics | LEO600 | LEO1200 |
| 1.1 | Antenna pattern | TR 38.921 | TR 38.921 |
| 1.2 | Element gain (dBi) (Note 2) | 4 | 4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90 for H 90 for V | 90 for H 90 for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 | 30 |
| 1.5 | Antenna polarization | Circular（RHCP or LHCP） | Circular（RHCP or LHCP） |
| 1.6 | Antenna array configuration (Row × Column)(Note 4) | 441 elements (21*21) | 900 elements (30*30) |
| 1.7 | Number of supported polarizations, P | 1 | 1 |
| 1.8 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V | 0.5 of wavelength for H 0.5 of wavelength for V |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 10dBm | 10dBm |
| 1.11 | maximum coverage angle in the horizontal plane (degrees) | 360 degrees | 360 degrees |
| 1.12 | vertical coverage range (degrees) between beam direction and normal direction | 54 degrees | 54 degrees |
| 1.13 | normal direction | Toward Z- axis | Toward Z- axis |

Note 1:	Void

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Row × Column ×Number of supported polarizations elements (i.e. power per H/V polarized element).

Note 4: Row × Column means there are Row vertical and Column horizontal radiating elements.

Proposal 2: After aligning the order of Rel-18 scenarios, it’s recommended to discuss the following relationship between TN and NTN for Ka band coexistence study.

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: Option 1: NTN UEs dropped at the edge of TN clusters (NTN UE outside of the TN cluster) Option 2: NTN UEs dropped in TN clusters (NTN UE inside TN cluster) Option 3: NTN UEs dropped in or at the edge of TN clusters with a possibility (Random number). FFS on possibility. | TN randomly placed in this NTN beam | Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the active TN clusters which contain the NTN UE(s). Option 3: All active TN clusters no matter whether the NTN UE(s) is (are) included in the TN cluster or at the edge of TN clusters. |
| 2 | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Follow case 1. | Consider an active rate of [20%] for TN. | All active TN cells in central NTN beam |
| 3 | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: Follow case 1. | TN clusters randomly placed in this NTN beam | Follow case 1. |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: Follow case 1. | TN clusters randomly placed in this NTN beam | Follow case 1. |
| 4 | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Follow case 1. | Consider the active rate of [20%] for TN. | Follow case 1. |
| 5 | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Option 1: NTN UEs dropped at the edge of TN clusters (NTN UE outside of the TN cluster) Option 2: NTN UEs dropped in TN clusters (NTN UE inside TN cluster) Option 3: NTN UEs dropped in or at the edge of TN clusters with a possibility (Random number). FFS on possibility. | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Option 1: All active TN clusters which has the NTN UE(s) at its edge. Option 2: Only the active TN clusters which contain the NTN UE(s). Option 3: All active TN clusters no matter whether the NTN UE(s) is (are) included in the TN cluster or at the edge of TN clusters. |
| 6 | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: Follow case 5 conclusion | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values) NTN UE: Follow case 5 conclusion |  |  |
| 7 | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: Follow case 5 conclusion | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (45° for GEO and LEO，Interested companies can bring analysis and results for other values). NTN UE: Follow case 5 conclusion |  |  |
| 8 | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: Follow case 5 conclusion. | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Follow case 5 conclusion |

Proposal 3: It’s proposed that RAN4 further discuss how much value the ACLR2 for Ka band UE is higher than ACLR1.

- References

[1] R4-2302878, WF for above 10GHz NTN-TN co-existence study, Samsung

[2] R4-2302108, Discussion on Rel-18 NTN coexistence study assumption, Huawei, HiSilicon

[3] ITU-R S.465-6, Study on new radio access technology: Radio Frequency (RF) and co-existence aspects