3GPP TSG-RAN WG4 Meeting # 106bis-e                           R4-2305390

Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on HD-FDD REFSENS for RedCap UE

Agenda Item:	4.25.2

Document for:	Approval

Introduction

In RAN#99 meeting, a WID [1] on adding new NR FDD bands for RedCap (Reduced Capability) was approved. The objectives are provided as below.

- Specify HD-FDD reference sensitivity requirements for the NR FDD bands listed in the table below to support Redcap UE a release-independent way (from Rel-17 onwards).

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n105 | 663 MHz – 703 MHz | 612 MHz – 652 MHz | FDD |

In this paper, we’d like to analyse the RedCap UE HD-FDD REFSENS for band n105.

Discussion

Based on the previous agreement, 0.8dB scale factor is used to tight the HD-FDD RedCap UE REFSENS with 2 Rx antenna port and there is no need to consider the impacts from Tx chain. So, HD-FDD REFSENS for band n105 is proposed as below.

Proposal 1: HD-FDD REFSENS for band n105 is proposed as below for 2 Rx antenna port.

Table 7.3I.2-2: HD-FDD RedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n105 | 15 | -98 | -94.8 | -93.0 | -91.8 |
|  | 30 |  | -95.1 | -93.1 | -92.0 |
|  | 60 |  | -95.5 | -93.4 | -92.2 |

Based on the previous agreement, 2.5dB scale factor is used to relax HD-FDD RedCap UE REFSENS with 2 Rx antenna port compared to 1 Rx antenna port.

Proposal 2: HD-FDD REFSENS for band n105 is proposed as below for 1 Rx antenna port.

Table 7.3I.2-2: HD-FDD RedCap UE with 1 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n105 | 15 | -95.5 | -92.3 | -90.5 | -89.3 |
|  | 30 |  | -92.6 | -90.6 | -89.5 |
|  | 60 |  | -93.0 | -90.9 | -89.7 |

Proposal 3: HD-FDD UL RB configuration for band n105 REFSENS is proposed as below

Table 7.3I.2-4: Uplink configuration for HD-FDD reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n105 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |

Since RF specifications’ impacts are very limited when introducing HD-FDD mode for new FDD bands, in order to reduce RAN or RAN4’s workload, the following improvements can be considered in RAN4’s future work.

1) HD-FDD REFSENS for new FDD bands can be introduced under TEI based on operators’ demands.

2) When one WI on introducing new FDD band is established, HD-FDD REFSENS objective can be included together.

Proposal 4: Since RF specifications’ impacts are very limited when introducing HD-FDD mode for new FDD bands, in order to reduce RAN and RAN4’s workloads, the following improvements can be considered in RAN4’s future work.

1) HD-FDD REFSENS for new FDD bands can be introduced under TEI based on operators’ demands.

2) When one WI on introducing new FDD band is established, HD-FDD REFSENS objective can be included together.

Summary

Proposal 1: HD-FDD REFSENS for band n105 is proposed as below for 2 Rx antenna port.

Table 7.3I.2-2: HD-FDD RedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n105 | 15 | -98 | -94.8 | -93.0 | -91.8 |
|  | 30 |  | -95.1 | -93.1 | -92.0 |
|  | 60 |  | -95.5 | -93.4 | -92.2 |

Proposal 2: HD-FDD REFSENS for band n105 is proposed as below for 1 Rx antenna port.

Table 7.3I.2-2: HD-FDD RedCap UE with 1 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n105 | 15 | -95.5 | -92.3 | -90.5 | -89.3 |
|  | 30 |  | -92.6 | -90.6 | -89.5 |
|  | 60 |  | -93.0 | -90.9 | -89.7 |

Proposal 3: HD-FDD UL RB configuration for band n105 REFSENS is proposed as below

Table 7.3I.2-4: Uplink configuration for HD-FDD reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n105 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |

Proposal 4: Since RF specifications’ impacts are very limited when introducing HD-FDD mode for new FDD bands, in order to reduce RAN and RAN4’s workloads, the following improvements can be considered in RAN4’s future work.

1) HD-FDD REFSENS for new FDD bands can be introduced under TEI based on operators’ demands.

2) When one WI on introducing new FDD band is established, HD-FDD REFSENS objective can be included together.

- References

[1] RP-230743, New WID on adding new NR FDD bands for RedCap (Reduced Capability) in Rel-18, Ericsson