3GPP TSG-RAN WG4 Meeting # 106bis-e                           R4-2305392

Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on UE RF requirements for NTN LS bands

Agenda Item:	4.32.3

Document for:	Approval

Introduction

In last meeting, NTN L-/S- band was discussed. For some UE requirements, rapporteur provided a detailed analysis in [1]. In this paper, we’d like to provide our views on some aspects.

Discussion on duplexer performance

In last meeting, the following requirements were analysed but without considering the duplexer performance for new band.

Maximum output power

MPR and A-MPR

Emission requirements

UE co-existence emission requirements

Reference sensitivity level

In-band and out-of-band blocking requirements

Firstly, additional emission requirements based on ETSI and FCC regulations were identified. As the protected regions are very close to the UL frequency range 1610~1626.5MHz, it’s better to provide the technical performance of duplexer to further analyse the attenuation when AMPR requirements are derived.

Secondly, DL frequency range of n255 and n256 are listed to be protected for new [n254] band coexistence requirements. Since the DL frequency range of band n255 is 1525 MHz – 1559 MHz, not sure whether -50dBm/MHz requirement can be met without the assumption of Tx filter’s performance.

Thirdly, the following REFSENS requirements were proposed just referring to band n256. However, larger duplex distance make this duplexer easier to reach a better noise figure performance.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n256 | 15 | -99.5 | -96.3 | -94.5 | -93.8 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -94.2 |  |  |  |  |  |  |
| n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
| [n254] | 15 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

It’s better to follow the normal RAN4 procedure to derive the REFSENS requirements based on the following formula.

REFSENS= -174+NF+10*log10(BW)+60-diversity gain+IM+SNR

BW(MHz) = (5, 10, 15)*SU

diversity gain = 3dB for 2Rx.

IM=2.5dB.

SNR= -1dB minimum SNR for detection

9dB Noise figure can be assumed as this duplexer is not hard to be implemented especially for DL part.

Proposal 1: to derive REFSENS requirements based on 9dB noise figure assumption.

Finally, a relaxed Out-of-band blocking requirements (-20dBm) between 2580 MHz and 2775 MHz were proposed. However, without the duplexer performance, it’s very hard to make such technical judgement.

Proposal 2: The duplexer performance need to be analysed and investigated before RAN4 specify the additional emission requirements, -50dBm/MHz coexistence requirements to protect band n255 and relax -20dBm out-of-band blocking requirements between 2580 MHz and 2775 MHz.

Summary

Proposal 1: to derive REFSENS requirements based on 9dB noise figure assumption.

Proposal 2: The duplexer performance need to be analysed and investigated before RAN4 specify the additional emission requirements, -50dBm/MHz coexistence requirements to protect band n255 and relax -20dBm out-of-band blocking requirements between 2580 MHz and 2775 MHz.

- References

[1] R4-2300302, RF requirements for the NTN L-/S-band, Apple, Globalstar