3GPP TSG-RAN WG4 Meeting#106bis                              R4-2305395

E-meeting, April 17 – April 26,2023

Agenda item:	5.14.2.3

Source: 	ZTE Corporation

Title: 	Further discussion on ATG UE Rx RF requirements

Document for:	Approval

Introduction

In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package. During the previous RAN4 meetings, we have reached some initial progress for ATG UE in [6], however there are still lots of open issues left for further discussions. For the ACLR/SEM and ACS requirement of ATG UE, it should be dependent on the outcome of ATG coexistence study as following. In this contribution, we want to share further views on ATG UE RF requirements based on the initial evaluation results obtained so far.

Table 6.1-1: Simulation scenarios for ATG coexistence study

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 1 | TN with ATG | ATG DL | 100MHz TDD | TN rural DL | 100MHz /TDD | 3.5 GHz |  | Phase 1 |
| 2 | TN with ATG | ATG UL | 100MHz TDD | TN rural UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 3 | TN with ATG | TN rural DL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 4 | TN with ATG | TN rural UL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 3.5GHz |  | FFS |
| 6 | TN with ATG | ATG UL | 100MHz TDD | TN rural DL | 100MHz TDD | 3.5GHz |  | FFS |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | FFS |
| 8 | TN with ATG | TN rural UL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | FFS |
| 9 | TN with ATG | ATG DL | 20MHz FDD | TN rural DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 10 | TN with ATG | ATG UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 11 | TN with ATG | TN rural DL | 20MHz FDD | ATG DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 12 | TN with ATG | TN rural UL | 20MHz FDD | ATG UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 13 | TN with ATG | ATG UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | FFS |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 | FFS |

Discussion

## 2.1. ACS requirement of ATG UE

Based on the simulation results obtained so far for Case 3 and Case 11 as shown in Figure 1 and Figure 3, it should be sufficient to reuse the legacy FR1 UE PC3 ACS requirement 33dBc requirement for ATG UE.

Figure 1a. simulation results for Case 3: TN DL interfering ATG DL @4GHz [non co-located deployment]

Figure 1b. simulation results for Case 3: TN DL interfering ATG DL @4GHz [co-located deployment]

Figure 2. the TN cells placed underneath of Aircraft [the figure is for informative with one more ring of 57 sectors]

Figure 3. simulation results for Case 10: TN DL interfering ATG DL @2GHz [non co-located deployment]

Observation 1: based on the initial simulation results in Case 3 and Case 11, it should be sufficient to reuse the legacy FR1 UE PC3 ACS requirement 33dBc requirement for ATG UE.

## 2.2. Other Rx RF requirements for ATG UE

In addition, in the following table 1, we share some further views on the ATG UE Rx RF requirement and its details could be found as following:

Table 1. Summary of the proposals for ATG UE requirements

| Requirement | Band-specific | Applicability |
| --- | --- | --- |
| Receiver characteristics |  |  |
| General | No |  |
| Diversity characteristics | No | Status: completed |
| Reference sensitivity | Yes | - Status: completed |
| Maximum input level | No | Status: ongoing This depend on the simulation results input to verify whether this requirement could be further relaxed similar as NR over NTN in Rel-17. Based on the coupling loss shown as following, the maximum input power could be further relaxed around 30dB compared with MCL 70dBc for Wide area BS class. Proposal 1: the maximum input power could be relaxed around 30dBc. |
| ACS | No | Status: ongoing Please see the simulation results in section 2.1 and its observations. |
| Blocking characteristics |  |  |
| - In-band | No | Status: ongoing To use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be lower compared with the handheld smartphone |
| - Out-of-band | Partially Yes | Status: ongoing To use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be lower compared with the handheld smartphone |
| - Narrow band | No | Status: completed |
| Spurious response | No | Status: ongoing Expect the same as TN and its power level of interfering signal could be further investigated similar as in-band blocking and out of band blocking. |
| Intermodulation | No | Status: ongoing Not applicable |
| Spurious emissions | No | Status: completed |

Conclusions

In this contribution, we shared some further views for ATG UE Rx RF requirements and proposals are made as following:

Observation 1: based on the initial simulation results in Case 3 and Case 11, it should be sufficient to reuse the legacy FR1 UE PC3 ACS requirement 33dBc requirement for ATG UE.

Proposal 1: the maximum input power could be relaxed around 30dBc.

Proposal 2: for in-band blocking requirement of ATG CPE, propose to use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be lower compared with the handheld smartphone.

Proposal 3: for OOBB requirement of ATG CPE, propose to use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be lower compared with the handheld smartphone.

Proposal 4: for receiver spurious response requirement, propose to the same as TN and its power level of interfering signal could be further investigated similar as in-band blocking and out of band blocking.

Proposal 5: for Rx IMD requirement of ATG CPE, propose not to define the requirement.

References

- RP-221369, Core part: Air-to-ground network for NR, CMCC, Approved.

- R4-2214461,WF on BS requirements for ATG, ZTE, Approved.

- R4-2215147,WF on UE requirements for ATG, Apple, Approved.

- R4-2214247,	Email Discussion Summary for [104-e][136] NR_ATG_UERF, CMCC, Noted

- R4-2217738	WF on ATG terminal RF requirement, Huawei, HiSilicon, Approved.