3GPP TSG-RAN WG4 Meeting#106bis                              R4-2305397

E-meeting, April 17 – April 26,2023

Agenda item:	5.14.1.4

Source: 	ZTE Corporation

Title: 	The simulation results for ATG coexistence study

Document for:	Discussion

Introduction

- In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package. In this contribution, we want to share some further updated simulation results for it based on the agreed simulation assumption so far.

- Table 6.1-1: Simulation scenarios for ATG coexistence study

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 1 | TN with ATG | ATG DL | 100MHz TDD | TN rural DL | 100MHz /TDD | 3.5 GHz |  | Phase 1 |
| 2 | TN with ATG | ATG UL | 100MHz TDD | TN rural UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 3 | TN with ATG | TN rural DL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 4 | TN with ATG | TN rural UL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 3.5GHz |  | FFS |
| 6 | TN with ATG | ATG UL | 100MHz TDD | TN rural DL | 100MHz TDD | 3.5GHz |  | FFS |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | FFS |
| 8 | TN with ATG | TN rural UL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | FFS |
| 9 | TN with ATG | ATG DL | 20MHz FDD | TN rural DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 10 | TN with ATG | ATG UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 11 | TN with ATG | TN rural DL | 20MHz FDD | ATG DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 12 | TN with ATG | TN rural UL | 20MHz FDD | ATG UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 13 | TN with ATG | ATG UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | FFS |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 | FFS |

Discussion

## 2.1 Sync scenario

### 2.1.1 Case 1,9 for ACLR requirement of ATG BS

In the following Figure 1, we shared the concerned TN victim cells for the coexistence study in Case 1 and Case 9 which is supposed to be most affected cells.

Figure 1. the TN victim cells considered in the throughput loss

Based on the simulation results obtained so far for Case 1 and Case 9 as shown in Figure 2 and Figure 3, ACI caused by ATG DL is still acceptable due to both the sparse deployment sites of ATG BS and its uptilt beamforming towards the aircraft in the sky. In short, based on the simulation results in Case 1 and Case 9 dominating the ATG BS DL ACLR requirement, it should be reasonable to reuse the legacy FR1 ACLR 45dBc requirement for ATG BS.

Figure 2. simulation results for Case 1: ATG DL interfering TN DL @4GHz

Figure 3. simulation results for Case 9: ATG DL interfering TN DL @2GHz

Observation 1: based on the simulation results in Case 1 and Case 9, it’s reasonable to reuse the legacy FR1 ACLR 45dBc requirement for ATG BS.

### 2.1.2. Case 4, 12 for ACS requirement of ATG BS

First of all, from our understanding, for Case 4 and Case 12, the TN BS and ATG BS are dropped in the same cluster should be worst case.

Based on the simulation results obtained so far for Case 4 and Case 12 as shown in Figure 4 and Figure 5, it is be sufficient to reuse the legacy FR1 ACS 46dBc requirement for ATG BS.

Figure 4. simulation results for Case 4: TN UL interfering ATG UL @4GHz [co-located deployment]

- Figure 5. simulation results for Case 12: TN UL interfering ATG UL @2GHz [co-located deployment]

Observation 2: based on the initial simulation results in Case 4 and Case 12, it’s sufficient to reuse the legacy FR1 ACS 46dBc requirement for ATG BS.

### 2.1.3. Case 2,10 for ACLR requirement of ATG UE

Based on the simulation results obtained so far for Case 2 and Case 10 as shown in Figure 6 and Figure 7, ACI caused by ATG UL towards TN UL are still quite limited. In short, based on the simulation results in Case 2 and Case 10 dominating the ATG UE UL ACLR requirement, it should be sufficient to reuse the legacy FR1 UE PC3 ACLR requirement 30dBc requirement for ATG UE.

Figure 6a. simulation results for Case 2: ATG UL interfering TN UL @4GHz [non co-located deployment]

Figure 6b. simulation results for Case 2: ATG UL interfering TN UL @4GHz [co-located deployment]

Figure 7a. simulation results for Case 10: ATG UL interfering TN UL @2GHz [non co-located deployment]

Figure 7b. simulation results for Case 10: ATG DL interfering TN UL @2GHz [co-located deployment]

Observation 3: based on the initial simulation results in Case 2 and Case 10, it should be sufficient to reuse the legacy FR1 UE PC3 ACLR requirement 30dBc requirement for ATG UE.

### 2.1.4. Case 3 and 11 for ACS requirement of ATG UE

Based on the simulation results obtained so far for Case 3 and Case 11 as shown in Figure 8 and Figure 10, it should be sufficient to reuse the legacy FR1 UE PC3 ACS requirement 33dBc requirement for ATG UE.

Figure 8a. simulation results for Case 3: TN DL interfering ATG DL @4GHz [non co-located deployment]

Figure 8b. simulation results for Case 3: TN DL interfering ATG DL @4GHz [co-located deployment]

Figure 9. the TN cells placed underneath of Aircraft [the figure is for informative with one more ring of 57 sectors]

Figure 10. simulation results for Case 10: TN DL interfering ATG DL @2GHz [non co-located deployment]

Observation 4: based on the initial simulation results in Case 3 and Case 11, it should be sufficient to reuse the legacy FR1 UE PC3 ACS requirement 33dBc requirement for ATG UE.

## 2.2 Unsync scenario

In the following section, we provided some initial simulation results for ATG and TN CLI analysis to figure out the appropriate isolation distance.

Figure 11. illustration of isolation distance between ATG BS and TN cluster

### 2.2.1 Case 5

In the following figure 12a, the simulation results for ATG BS and TN BS with 100% grid shift considered, it could found that about 20% throughput loss of cell edge users. It should be noted that all sectors considered in this evaluation scenario, if only most affected cells to be considered, then more throughput loss could be expected.

- Figure 12a simulation results for Case 5 [all TN sectors considered]

In the following figure 12b, the simulation results for ATG BS and TN BS with isolation distance as shown in Figure11, it could found that about throughput loss of cell edge users would be down to 5% which is usually acceptable from RAN4 perspective. It should be noted that all sectors considered in this evaluation scenario, if only most affected cells to be considered, then more throughput loss could be expected and more isolation distance are needed..

- Figure 12b simulation results for Case 5 [all TN sectors considered]

### 2.2.2 Case 6

In the following figure 13a and 13b, ATG UL interfering TN DL, the interference are quite limited due to far distance between TN UEs and ATG CPE.

- Figure 13a simulation results for Case 6 [non co-located]

- Figure 13b simulation results for Case 6 [co-located]

### 2.2.3 Case 7

In the following figure 14a and 14b, TN DL interfering ATG UL, there are relative high interference, however due to the high SINR within the ATG uplink carrier, therefore the tolerance for ACI is still okay. If with 3*ISD isolation as shown in Figure 14b, the ACI from TN DL would be further reduced and there would be no coexistence problems anymore.

- Figure 14a simulation results for Case 7

- Figure 14b simulation results for Case 7

### 2.2.4 Case 8

In the following figure 15a and 15b, TN UL interfering ATG DL, the interference are quite limited due to far distance between TN UEs and ATG CPE.

- Figure 15a. simulation results for Case 8 [should be ATG DL]

- Figure 15b. simulation results for Case 8 [should be ATG DL]

- 2.2.6 Case 13

In the following figure 16a and 16b, ATG UL interfering TN DL, the interference are quite limited due to far distance between TN UEs and ATG CPE.

- Figure 16a simulation results for Case 13 [non-colocated]

- Figure 16b simulation results for Case 13 [colocated]

- 2.2.6 Case 14

In the following figure 17b, TN DL interfering ATG UL, there are relative high interference, however due to the high SINR within the ATG uplink carrier, therefore the tolerance for ACI is still okay. If with 3*ISD isolation, the ACI from TN DL would be further reduced and there would be no coexistence problems anymore.

- Figure 17 simulation results for Case 14

Conclusions

In this contribution, we shared some further views on coexistence study for ATG coexistence and proposals are made as following:

Observation 1: based on the simulation results in Case 1 and Case 9, it’s reasonable to reuse the legacy FR1 ACLR 45dBc requirement for ATG BS.

Observation 2: based on the initial simulation results in Case 4 and Case 12, it’s sufficient to reuse the legacy FR1 ACS 46dBc requirement for ATG BS.

Observation 3: based on the initial simulation results in Case 2 and Case 10, it should be sufficient to reuse the legacy FR1 UE PC3 ACLR requirement 30dBc requirement for ATG UE.

Observation 4: based on the initial simulation results in Case 3 and Case 11, it should be sufficient to reuse the legacy FR1 UE PC3 ACS requirement 33dBc requirement for ATG UE.

References

- RP-221369, Core part: Air-to-ground network for NR, CMCC, Approved.

- R4-2214459,WF on co-existence evaluation for ATG, Huawei, Approved.

- R4-2217736, WF on FR1 co-existence evaluation for ATG, CMCC, Approved.

- R4-2303573, WF on ATG co-existence evaluation, Approved.