3GPP TSG-RAN WG4 Meeting#106bis                               R4-2305402

E-meeting,April 17 – April 26,2023

Agenda item:	5.20.2.3

Source: 	ZTE Corporation

Title: 	Discussion on BS RF requirement impacts from SBFD perspective

Document for:	Approval

Introduction

In the previous RAN4 meetings, there were extensive discussions on BS RF impacts from SBFD perspective and we have reached the following agreements for the impacted RF requirements for SBFD time slots, however there are still some open issues left for further discussions. In this contribution, we want to share some further analysis on these remaining issues.

Agreement:

The existing RF requirements with respect to wanted signal as below are still applicable within SBFD time slot:

| Conducted RF requirement | Radiated RF requirement |
| --- | --- |
| BS output power Output power dynamics Transmitted signal quality Occupied bandwidth Dynamic range | Radiated transmit power OTA base station output power OTA output power dynamics OTA transmitted signal quality OTA occupied bandwidth OTA dynamic range |

The following requirements are new requirements,

- OTA sensitivity within SBFD time slot

- Other requirements not precluded

The following requirements may be impacted,

- Transmitter intermodulation

- Out of band blocking

The following requirements are not applicable,

- Transmit ON/OFF power requirement within SBFD time slot

- OTA receiver spurious emissions

Discussion

First of all, based on the conclusion reached during RAN#96e meeting, SBFD time slots are prioritized to be configured on top of the existing downlink symbols and de-prioritized to be configured in the uplink symbols.

Endorsed proposal [RAN#96e]

Conclusion: UL symbol as 2nd priority is accepted, no intended suspension of continuation of work in WGs.

In the following figure 1, the generic diagram for the configuration of SBFD on top of the existing TDD configuration is shown for the convenience of further analysis in the following section.

Figure 2.1. the illustration of SBFD configuration

| Company | Comments |
| --- | --- |

## 2.1. Conducted Tx requirements

### 2.1.1. BS station output power

For the legacy/normal slot/symbols, the existing output power accuracy requirement and the rated power limits in section 6.2 of TS 38.104 are still applicable.

For the SBFD slot/symbols, the existing output power accuracy requirement and the rated power limits in section 6.2 of TS 38.104 could be still applied. However it should be noted that rated output power limits within SBFD time slot could be different from the normal time slots. The exact difference between should be up to the exact SBFD downlink bandwidth configuration, antenna configuration, etc.

| Company | Comments |
| --- | --- |

### 2.1.2. Output power dynamics

For the legacy/normal slot/symbols, the existing RE power control dynamic range and total power dynamic range requirement in section 6.3 of TS 38.104 are still applicable.

For the SBFD slot/symbols, the existing RE power control dynamic range and total power dynamic range requirement in section 6.3 of TS 38.104 could be still applied. However it should be noted that total power dynamic range requirement within SBFD time slot could be different from the normal time slots. The exact difference between is up to the SBFD downlink bandwidth configuration.

| Company | Comments |
| --- | --- |

### 2.1.3. Transmit ON/OFF power

For the legacy/normal slot/symbols, the existing transmitter OFF power and transmitter transient period requirement in section 6.4 of TS 38.104 are still applicable for DL transmission-UL transmission/GP switching scenario and UL transmission -DL transmission switching scenario.

Transmitter output power

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transmitter transient

period

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 2.1.3-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period [extracted from TS38.104]

For DL transmission and SBFD slot switching scenario as shown in the following guard period 1 and SBFD slots switching scenario as shown in the following guard period 2, we didn’t discuss whether it’s necessary to specify the ON-OFF transition related requirements yet. It should be well known that ON-OFF transmission requirement is mainly defined to specify the BS Tx-Rx ON-OFF and OFF-ON behavior, however in guard period 1 and guard period 2, the Tx chain are just reconfigured instead of switching ON-OFF behavior. Similar as consecutive SRS time mask for the case when power change is required in UE specification TS 38.101-1, the transition period in guard period 1 and guard period 2 in the following figure might be still necessary. The exact transition period could be 10us for FR1 and 5us for FR2 similar as UE requirements.

Figure 2.1.3-2: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period

In addition, transmit ON/OFF power requirement within SBFD time slot is not applicable since both transmitter and receiver is turn on during the SBFD time slot.

Proposal 1: to further discuss the necessity of transition requirement for DL transmission and SBFD slot switching scenario and SBFD slots switching scenario and its exact requirement if necessary.

| Company | Comments |
| --- | --- |

### 2.1.4. Transmitted signal quality

For the legacy/normal slot/symbols, the existing frequency error, EVM requirement and TAE requirement in section 6.5 of TS 38.104 are still applicable.

For the SBFD slot/symbols, in principle, the existing frequency error, EVM requirement and TAE requirement in section 6.5 of TS 38.104 are still applicable. However the CA related TAE requirement is not applicable for SBFD slots since single carrier operation is prioritized in Rel-18 SI.

In addition, if BS is configured with SBFD slots, from our understanding, CA is also not supposed to be supported in the normal slot.

Besides, for the freq error measurement and EVM measurement, whether measurement for normal slots and SBFD slots are conducted together or separately, this also need more discussion.

If measurement for normal slots and SBFD slots are conducted together, then measurement time for freq error and EVM measurement could be kept as the same as the existing requirements.

If measurement for normal slots and SBFD slots are conducted separately, then measurement time for freq error and EVM measurement could be further extended due to the limited measurement resource in the time domain.

| Company | Comments |
| --- | --- |

### 2.1.5. OBW

For the legacy/normal slot/symbols, the existing OBW requirement in section 6.6.2.1 of TS 38.104 are still applicable.

For the SBFD slot/symbols, the existing OBW requirement in section 6.6.2.1 of TS 38.104 are still applicable since this is inherited from ITU-R recommendation.

| Company | Comments |
| --- | --- |

### 2.1.6. ACLR

For the legacy/normal slot/symbols, the existing ACLR requirement in section 6.6.3 of TS 38.104 are still applicable.

For the SBFD slot/symbols, for the adjacent channel leakage within carrier uplink spectrum part , this could be implicitly tested by SBFD reference sensitivity requirement if downlink transmission is also configured to be transmitted during the SBFD slots/symbols.

For the adjacent channel leakage outside of the carrier, then legacy requirement of normal downlink slots should be still applied for SBFD slots/symbols. In other words, even though sub-band configuration is 20MHz and normal configuration is 100MHz, then ACLR measurement of sub-band configuration is still conducted over adjacent 100MHz carrier. The main reason is that sub-band configuration could be in any PRB levels similar as UE BWP configuration, it’s not possible to test any PRB configurations within the adjacent channel which will lots of conformance testing burden and increase the implementation complexity. In addition, from network deployment perspective, the if ACLR requirement of SBFD slots/symbols could be guaranteed as the same as that of normal slots, there would be also no coexistence issues in the practice. From this perspective, it also make sense to reuse the ACLR requirement for normal slots for sub-band slots/symbols.

Figure 2.1.4-1: Example of ACLR requirement of SBFD slots

| Company | Comments |
| --- | --- |

### 2.1.7. Unwanted emission mask

For the legacy/normal slot/symbols, the existing OBUE requirement in section 6.6.4 of TS 38.104 are still applicable.

For the SBFD slot/symbols, for the OBUE within carrier uplink spectrum part , this could be implicitly tested by SBFD reference sensitivity requirement if downlink transmission is also configured to be transmitted during the SBFD slots/symbols.

For OBUE outside of the carrier, then legacy requirement of normal downlink slots should be still applied for SBFD slots/symbols similar as ACLR requirement for sub-band slots/symbols.

| Company | Comments |
| --- | --- |

### 2.1.8. Transmitter spurious emission

General spurious emission requirement:

For the legacy/normal slot/symbols, the existing general spurious emission requirement in section 6.6.5.2.1 of TS 38.104 are still applicable.

For the SBFD slot/symbols, the existing general spurious emission requirement in section 6.6.5.2.1 of TS 38.104 are still applicable since these requirements are inherited from ITU-R SM.329.

Protection of the BS receiver of own or different BS:

For the legacy/normal slot/symbols, the existing protection requirement in section 6.6.5.2.2 of TS 38.104 are not applicable since this is only targeted for FDD operation;

For the SBFD slots/symbols, it work similar as FDD operation indeed, however the existing requirement might be too stringent in the practice based on the assumption of 30dB isolation between Tx and Rx chain, however for SBFD BS, much larger isolation between Tx and Rx chain could be assumed, therefore the protection of BS receiver of own is not applied here. In addition, for the protection of other BS receiver, this requirement might be also not applicable similar as inter-sector coexistence analysis.

Additional spurious emissions requirements

For the legacy/normal slot/symbols, the existing additional spurious emission requirement in section 6.6.5.2.3 of TS 38.104.

For the SBFD slots/symbols, the existing additional spurious emission requirement in section 6.6.5.2.3 of TS 38.104.

Co-location with other base stations

For the legacy/normal slot/symbols, the existing co-location requirement in section 6.6.5.2.4 of TS 38.104.

For the SBFD slots/symbols, the existing existing co-location requirement in section 6.6.5.2.4 of TS 38.104.

| Company | Comments |
| --- | --- |

2.1.9. Tx intermodulation

For the SBFD slots/symbols, in the principle, the existing co-location transmitter intermodulation requirement should be also applied for SBFD BS, however co-location interfering signal is not only injected into the transmitter side, but also injected into receiver side during SBFD slots/symbols, this will cause significant impacts on receiver performance if possible.

In addition, based on the coexistence study of SBFD scenario, 100% grid shift is assumed as baseline. For other grid shift, e.g. 10% grid shift could be treated as 2nd priority. In other words, the coupling loss between aggressor BS and victim SBFD DL should be much larger than the legacy 30dB in the practice.

In short, the existing co-location transmitter intermodulation requirement might be too big for the receiver during SBFD slots/symbols.

- Agreements: Take 100% grid shift as baseline assumptions, companies are also encouraged to bring evaluation simulations with other values besides 100% grid shift e.g. 10% grid shift as 2nd priority

Figure 2.1.9-1: Example of Tx intermodulation

For the legacy/normal slot/symbols, in principle, the existing co-location transmitter intermodulation requirement requirement in section 6.7.2.1 of TS 38.104 could be applied here. However based on the analysis from receiver during SBFD slots/symbols, the existing transmitter intermodulation requirement might be not applicable for SBFD BS.

| Company | Comments |
| --- | --- |

## 2.2. Conducted Rx requirements

### 2.2.1. Reference sensitivity level

For the legacy/normal slot/symbols, the existing REFSENS requirement in section 7.2 of TS 38.104 are still applicable.

For the SBFD slot/symbols, the new REFSENS requirement should be defined considering the downlink transmission during the receiver refense conformance testing. It should noted that radiated antenna should be also included during the conducted conformance testing, otherwise it would be self interference cannot been rejected into the SBFD receiver side.  The self interference caused by PCB coupling loss between transmitter and receiver of SBFD might be limited compared with self interference experienced over the air.

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ 1dB desens target]

In addition, it should noted that FRCs for sub-band configuration should be left for further discussions in RAN1 since the PRB configuration and symbol level or slot level granularity is still not clear in RAN1. The same story also applies for downlink configuration during the SBFD slots/symbols. All these issues could be left for further discussion in the WID phase.

Figure 2.2.1-1: Example of REFSENS requirement for SBFD BS

Figure 2.2.1-2: Example of REFSENS requirement for SBFD BS

| Company | Comments |
| --- | --- |

### 2.2.2. Dynamic range

For the legacy/normal slot/symbols, the existing dynamic range requirement in section 7.3 of TS 38.104 are still applicable.

For the SBFD slot/symbols, dynamic range requirement should be further considered to configure both downlink transmission signal and wanted signal, interfering signal of dynamic range within the carrier. Self interference caused by the downlink transmission is much lower than interference signal power level and wanted signal power level, therefore from our understanding, additional desens for dynamic range requirement should be marginal and could be ignored.

The fundamental issues is how to design the FRCs for downlink signal and uplink signal in the dynamic range requirements.

PIntf = -174dBm/Hz+10*log10(NRB*SCS*12) + NF + 20

Pwanted = -174dBm/Hz+10*log10(NRB*SCS*12) + NF + 20+SNR+IM

Figure 2.2.2-1: Example of Dynamic range requirement for SBFD BS

| Company | Comments |
| --- | --- |

### 2.2.3 In-band selectivity and blocking

ACS requirement

For the legacy/normal slot/symbols, the existing dynamic range requirement in section 7.4.1 of TS 38.104 are still applicable.

For the SBFD slot/symbols, wanted signal power level of ACS requirement could be defined as shown in the following equation. In addition, self interference caused by the downlink transmission is somehow close to refense degradation caused by ACS requirement, therefore self interference cannot been ignored as dynamic range requirement mentioned before.

Wanted signal power level = REFSENS +10*log10(10^6/10+10^[1dB desens target/10])

The fundamental issues is how to design the FRCs for downlink signal and uplink signal as clarified in the REFSENS part.

Figure 2.2.2-2: Example of ACS requirement for SBFD BS

In-band blocking

For the legacy/normal slot/symbols, the existing in-band blocking requirement in section 7.4.2.2 of TS 38.104 are still applicable.

For the SBFD slot/symbols, wanted signal power level for IBB requirement could be defined in the following equation similar as that of ACS requirement . In addition, self interference caused by the downlink transmission is somehow close to IBB interference, therefore self interference cannot been ignored as dynamic range requirement.

Wanted signal power level = REFSENS +10*log10(10^6/10+10^[1dB desens target/10])

The fundamental issues is how to design the FRCs for downlink signal and uplink signal as clarified in the REFSENS part.

Figure 2.2.2-3: Example of IBB and NBB requirement for SBFD BS

Narrow band blocking requirement

For the legacy/normal slot/symbols, the existing NBB requirement in section 7.4.2.2 of TS 38.104 are still applicable.

For the SBFD slot/symbols, wanted signal power level for NBB requirement could be defined in the following equation similar as that of ACS requirement and IBB requirement.

Wanted signal power level = REFSENS +10*log10(10^6/10+10^[1dB desens target/10]) for other 5MHz channel bandwidth

Wanted signal power level = REFSENS +10*log10(10^8/10+10^[1dB desens target/10]) for 5MHz channel bandwidth

| Company | Comments |
| --- | --- |

### 2.2.4. Out-of-band blocking

General OOBB requirement

For the legacy/normal slot/symbols, the existing NBB requirement in section 7.5.2 of TS 38.104 are still applicable.

For the SBFD slot/symbols, wanted signal power level for OOBB requirement could be defined in the following equation similar as that of ACS requirement and IBB requirement.

Wanted signal power level = REFSENS +10*log10(10^6/10+10^[1dB desens target/10])

Figure 2.2.4-1: Example of OOBB requirement for SBFD BS

Co-location minimum requirements

For the legacy/normal slot/symbols, the existing co-location blocking requirement in section 7.5.2.3 of TS 38.104 are still applicable.

For the SBFD slot/symbols, wanted signal power level for co-location blocking requirement could be defined in the following equation similar as that of ACS requirement and IBB requirement.

Wanted signal power level = REFSENS +10*log10(10^6/10+10^[1dB desens target/10])

| Company | Comments |
| --- | --- |

### 2.2.5. Receiver spurious emissions

For the legacy/normal slot/symbols, the existing receiver spurious emission requirement in section 7.6.3 and 7.6.4 of TS 38.104 are still applicable.

For the SBFD slot/symbols, as mentioned in the refense conformance testing, the radiated antenna should be also included during the conducted conformance testing, otherwise it would be self interference cannot been rejected into the SBFD receiver side. The self interference caused by PCB coupling loss between transmitter and receiver of SBFD might be limited compared with self interference experienced over the air. In other words, the receiver spurious emissions will be also overwhelmed by transmitter spurious emission at the end which is somehow similar as OTA receiver spurious emission at the end. In other words, the conducted receiver spurious emission requirement is also not applicable if the radiated antenna of Tx chain and Rx chain are be also included during the conducted conformance testing.

| Company | Comments |
| --- | --- |

### 2.2.6. Receiver intermodulation

For the legacy/normal slot/symbols, the existing receiver intermodulation requirement in section 7.7.2 of TS 38.104 are still applicable.

For the SBFD slot/symbols, there might be some self interference and additional Rx intermodulation caused by downlink part especially when considering with CW signal or NBB or general Rx intermodulation signals configured during the Rx IMD test, therefore it’s suggested to have further study for it.

Figure 2.2.6-1: Example of Rx intermodulation requirement for SBFD BS

| Company | Comments |
| --- | --- |

### 2.2.7. In channel selectivity

For the legacy/normal slot/symbols, the existing in-channel selectivity requirement in section 7.8.1 of TS 38.104 are still applicable.

For the SBFD slot/symbols, there might be some additional self caused by downlink part in addition to the I/Q imbalance or image interference, therefore it should be similar as other ACS and IBB requirement, the wanted signal should be updated as following to accommodate the self interference from downlink part.

Wanted signal power level = REFSENS +10*log10(10^6/10+10^[1dB desens target/10])

Figure 2.2.7-1: Example of Rx intermodulation requirement for SBFD BS

| Company | Comments |
| --- | --- |

## 2.3. Radiated Tx requirements

### 2.3.1 Radiated transmit power

For the legacy/normal slot/symbols, the existing EIRP accuracy requirement in section 9.2 of TS 38.104 are still applicable.

For the SBFD slot/symbols, the existing EIRP accuracy requirement in section 9.2 of TS 38.104 could be still applied. However it should be noted that the manufacturer claimed EIRP level within SBFD time slot could be different from the normal time slots. The exact difference between should be up to the exact SBFD downlink bandwidth configuration, antenna configuration, etc.

In addition, for the SBFD DL part, whether uplink testing signals should be also injected into the chamber, this still more discussions. From our understanding, if there are no sufficient isolation method between transmitter and receiver of measurement equipment as shown in following Figure 2.3.1-1, the measurement accuracy of downlink EIRP would be impacted indeed.

In short, during the SBFD downlink conformance testing, we propose not to add any uplink testing signal within the chamber.

Figure 2.3.1-1: Example of test setup for SBFD BS in OTA scenario

NOTE: during the SBFD downlink conformance testing, it is not allowed to add any uplink testing signal within the chamber.

| Company | Comments |
| --- | --- |

### 2.3.2 OTA base station output power

For the legacy/normal slot/symbols, the existing TRP accuracy requirement and the rated TRP output power limits in section 9.3 of TS 38.104 are still applicable.

For the SBFD slot/symbols, the existing TRP accuracy requirement and the rated TRP output power limits in section 9.3 of TS 38.104 could be still applied. However it should be noted the rated TRP output power limits within SBFD time slot could be different from the normal time slots. The exact difference between should be up to the exact SBFD downlink bandwidth configuration, antenna configuration, etc.

| Company | Comments |
| --- | --- |

### 2.3.3. OTA output power dynamics

For the legacy/normal slot/symbols, the existing RE power control dynamic range and total power dynamic range requirement in section 6.3 of TS 38.104 are still applicable for FR1 SBFD.

For the SBFD slot/symbols, the existing RE power control dynamic range and total power dynamic range requirement in section 6.3 of TS 38.104 could be still applied. However it should be noted that total power dynamic range requirement within SBFD time slot could be different from the normal time slots. The exact difference between is up to the SBFD downlink bandwidth configuration.

| Company | Comments |
| --- | --- |

### 2.3.4. OTA transmitted signal quality

The conclusion for conducted transmit transmitted signal quality could be also reused here.

| Company | Comments |
| --- | --- |

### 2.3.5.OTA unwanted emissions (OBW, ACLR, OBUE, spurious emission)

The conclusion for conducted unwanted emission requirement could be also reused here.

| Company | Comments |
| --- | --- |

### 2.3.6.OTA Tx intermodulation requirement

The conclusion for conducted Tx intermodulation requirements could be also reused here.

| Company | Comments |
| --- | --- |

## 2.4. Radiated Rx requirements

In the current specification, it is only defined that transmitter should be turn on when this is FDD operation. For SBFD operation, indeed this is also FDD operation in TDD spectrum, therefore the following restriction note should be updated for SBFD in TDD bands;

-	For FDD operation the requirements shall be met with the transmitter unit(s) ON.

| Company | Comments |
| --- | --- |

### 2.4.1.OTA sensitivity

This requirement is not related with conducted requirement at all, however similar as conducted refsense requirement definition, the self interference should be also considered, in other words, the declared EIS level for SBFD slots and normal slots could be different.

| Company | Comments |
| --- | --- |

### 2.4.2.OTA reference sensitivity level

The conclusion for conducted Reference sensitivity could be also reused here.

| Company | Comments |
| --- | --- |

### 2.4.3.OTA dynamic range

The conclusion for conducted dynamic Reference sensitivity could be also reused here.

| Company | Comments |
| --- | --- |

### 2.4.4 OTA In-band selectivity and blocking

The conclusion for conducted in-band selectivity and blocking could be also reused here.

| Company | Comments |
| --- | --- |

2.4.5. OTA Out-of-band blocking

The conclusion for conducted OOBB requirements could be also reused here.

| Company | Comments |
| --- | --- |

2.4.6. OTA Receiver spurious emissions

The conclusion for conducted receiver spurious emission requirement could be also reused here. It should be not applicable for SBFD slots/symbols.

| Company | Comments |
| --- | --- |

2.4.7. OTA Receiver intermodulation

The conclusion for conducted Rx IMD requirements could be also reused here.

| Company | Comments |
| --- | --- |

2.4.8. OTA In channel selectivity

The conclusion for conducted Rx ICS requirements could be also reused here.

| Company | Comments |
| --- | --- |

Conclusions

In this contribution, we want to share further feasibility analysis on the MR SBFD and LA SBFD BS and proposals are made as following:

References

- R4-2302969, WF for BS RF requirements impact for SBFD operation, CATT, Approved.

- TS 38.104 18.0.0

TP

- TBD