3GPP TSG-RAN WG4 Meeting#106bis                              R4-2305416

E-meeting, April 17 – April 26,2023

Agenda item:	5.29.5

Source: 	ZTE Corporation

Title: 	Further discussion on RRM requirements for NCR-MT

Document for:	Approval

Introduction

In the latest RAN#97e meeting, the work item [RP-222673] on NR network-controlled repeater was approved after the completion of study item phase led by RAN1. In this contribution, we want to share further views on RRM requirement impacts from RAN4 perspective based on the latest RAN1/RAN2/4 agreement reached.

RAN1 agreement in RAN1#110-bis-e meeting

Agreement: The following information can be used to characterize the physical beam(s) supported by NCR-Fwd for access link: - Number of beams supported for access link - FFS: How to define the detailed value (e.g., per beam type) - FFS: Whether the number of beam can be derived by beam layout - Spatial relationship between different beams - FFS: Beam types defined by the beam width (e.g. two types as wide beam and narrow beam type) - FFS: Beam direction defined by the boresight of beam - FFS: Whether/How to deliver this information to gNB - FFS: Coverage area for each beam type - FFS: Beam ID (via explicit or implicit means) Agreement: The following methods are supported for access link beam indication: - Single beam index per indication is supported to indicate one beam. - The indication of multiple beams in one indication is supported - Note: The multiple beams are applied in TDMed over same frequency resource Agreement Confirm the WA that in access link, a DL beam and a UL beam which are correspondent with each other have the same beam index. Agreement The following aspects should be NCR capability: - Simultaneous UL transmission of C-link and backhaul link - Adaptive beam for C-link/backhaul-link - Note-1: Fixed beam for C-link/backhaul link is default capability - Note-2: TDMed UL transmission of C-link and backhaul link is default capability. - FFS: How to define the capability for adaptive beam for C-link/backhaul-link Conclusion An NCR is not expected to perform forwarding in “OFF” state. Agreement For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the default behavior of the NCR-Fwd is expected to be OFF or not forwarding over these symbols - FFS: The behavior over these symbol if dynamic DL/UL operation is supported by NCR-MT and/or NCR-Fwd. Agreement Following parameters should be supported to define the time resource: - For a periodic and/or semi-persistent configured time resource, starting time, duration per beam(s) and periodicity is needed. - For aperiodic indication of time resource, starting time and duration per beam(s) is needed. - FFS: The SCS for starting time, duration, and periodicity - FFS: How to define the duration, e.g., via the length of time resource or resource index(es) - FFS: Whether indication of starting time can be implicit (e.g., the first slot after the time to apply the received beam indication and the first OFDM symbol in the slot) Agreement If adaptive beams are adopted for C-link and backhaul link, new signaling is supported to indicate a beam(s) used for backhaul link from the set of beams for C-link. - Predefined rule is used to define the beam in case there is no indication via the new signalling - FFS: Details of the predefined rule - FFS: Application of predefined rule for other cases - Note: The beam(s) used for backhaul link should be from the RRC-configured list of beams for C-link. - The new signalling, if needed, is an optional NCR capability Agreement For the ON/OFF information indication, at least one of following options is supported to indicate the ON state of NCR - Alt-1: Explicit indication with dedicated field to indicate ON state - Note: At least it’s supported when the beam indication is not applicable - Alt-2: Implicit indication via the beam indication - Alt-3: Indication via the time domain resource indication (i.e., the NCR is assumed to be ON over the indicated time domain resource)

Agreement: For NCR-MT which can support adaptive beams in C link, - Rel-15 beam indication framework can be reused. - Rel-17 beam indication framework (i.e., the unified TCI) can be reused as well, if the NCR supports. The gNB can configure the unified TCI for the NCR-MT, if the NCR-MT supports. Agreement: To support CSI measurement/reporting mechanisms for NCR-MT in C-link - The necessary legacy mechanism for receiving CSI-RS is reused for NCR-MT. - The necessary legacy mechanism for reporting CSI is reused for NCR-MT. - FFS: The details of the necessary mechanisms will be further discussed and decided. - Note: this does not mean all the legacy procedures for receiving CSI-RS and reporting CSI will be supported. Agreement: HARQ-ACK feedback for PDSCH carrying the side control information from higher layer (e.g., MAC-CE, RRC) is supported. The legacy HARQ-ACK feedback mechanism is reused. - FFS: Whether HARQ-ACK feedback for PDCCH carrying side control information is supported - Note: This does not mean all legacy HARQ-ACK feedback mechanism will be supported. Agreement: PUCCH and PUSCH are supported for NCR-MT. Agreement: The TA adjustment mechanism of legacy UEs is supported for NCR-MT in C link. Agreement: To support the sounding procedure for NCR-MT in C link, the necessary mechanism of legacy UE sounding procedure is supported. - FFS: The details of the necessary mechanism of legacy UE sounding procedure. - Note: This does not mean all legacy UE sounding procedure will be supported.

RAN2 agreement in RAN1#119-bis-e meeting

Agreement: RAN2 confirms to use RRC signalling to configure NCR-MT to receive side control information. How the side control information itself is transmitted (i.e. via RRC or DCI or MAC CE) is up to RAN1 (RAN2 may discussion the initial RAN1 decision and revisit if needed).

Agreement: NCR-MT supports RRC_CONNECTED and RRC_IDLE states, FFS on RRC_INACTIVE state (e.g. optional support or not support)

Agreement NCR-MT supports SRB0/1/2 and DRB is optional. FFS on maximum number of DRBs.

Agreements RRM functions supported by NCR-MR: - Cell selection is mandatory - Cell reselection, RLM, BFD, BFR are FFS

Discussion

## 2.1. RRC Connection Mobility Control

### 2.1.1. RRC re-establishment

During the last RAN4 meeting, we reached good consensus on RRC re-establishment, however there are no clear agreement whether this requirement could be applicable for both WA NCR-MT and LA NCR-MT. From our understanding, this should be similar as other RRM requirement, this should be only applicable for LA NCR-MT.

- Agreement - Define the RRC re-establishment requirement for NCR-MT in Rel-18 and reuse the existing IAB-MT RRM requirement from TS38.174 clause 12.1.1.1 as baseline

Proposal 1: to define the RRC re-establishment requirement for LA NCR-MT in Rel-18 only.

### 2.1.2. Random access

During the last RAN4 meeting, we reached good consensus on random access, however it is still not clear whether 2 step rach is needed or not. At least from our understanding, this is not needed since 2 step rach is mainly used for mMTC, URLLC and eMBB to improve the overall latency of RACH procedure, however for NCR-MT, we don’t see much necessity for this feature or this could left up to the implementation.

- Agreement - Wide area NCR - Do not define random access requirements - Local area NCR - Define the 4-step RACH requirement for NCR-MT and reuse the existing UE requirement in clause 6.2.2 of TS38.133 - FFS for 2-step RACH

Proposal 2: to only define the 4 step RACH requirement for LA NCR-MT and leave 2 step RACH by the implementation.

## 2.2. Transmit timing requirement for NCR-MT

During the last RAN4 meeting, we reached good consensus on initial transmit timing requirement Te, however there are no clear agreement whether this requirement could be applicable for both WA NCR-MT and LA NCR-MT. From our understanding, this should be similar as other RRM requirement, this should be only applicable for LA NCR-MT.

· Agreement - To define initial transmit timing requirement Te for NCR-MT and to reuse the requirement in clause 12.2.1 of Rel-16 TS 38.174. - No RRM requirements need to be specified for the transmit timings of backhaul and access link of NCR-Fwd;

Proposal 3: to define initial transmit timing requirement Te for LA NCR-MT in Rel-18 only.

During the last RAN4 meeting, we reached good consensus on gradual timing adjustment for LA NCR-MT, however there are no clear agreement for the requirement of gradual timing adjustment itself. From our understanding, it is reasonable to keep alignment between Rel-16 IAB-MT and Rel-18 NCR-MT.

- Agreement - Wide area NCR - Do not define gradual timing adjustment requirement Tp and Tq - Local area NCR - Define gradual timing adjustment requirement Tp and Tq

Proposal 4: For the gradual timing adjustment requirement Tp and Tq for LA NCR-MT, propose to reuse the requirement defined for IAB-MT in Rel-16.

For NCR-MT timer requirement, we don’t see the reasons to preclude it since there ares still timer used in the different protocol entities to control the NCR-MT behaviour. It should be okay to reuse the existing UE timer requirement defined in clause 7.2.1 of TS38.133.

Proposal 5: to define the timer accuracy requirement for LA NCR-MT and reuse the requirement in clause 7.2.1 of TS38.133.

For NCR-MT TA adjustment accuracy, based on the following agreement reached in RAN1, TA adjustment accuracy requirement are still needed since network will send the TA command to NCR-MT and NCR-MT should execute the TA command in the uplink. Without its TA adjustment accuracy requirement, it might be hard to ensure its performance. Regarding the reasons why TA command is needed, please check the above clarifications for gradual timing adjustment part.

Agreement:

The TA adjustment mechanism of legacy UEs is supported for NCR-MT in C link.

Proposal 6: to define TA adjustment accuracy requirement for LA NCR-MT and reuse the same requirement as Rel-16 IAB-MT.

## 2.3. Active BWP switching

BWP switching for Rel-18 NCR-MT is not expected to happen frequently and don’t see its strong necessity similar as Rel-16 IAB-MT.

Proposal 7: not to define the active BWP switching requirement for WA and LA NCR-MT in Rel-18.

## 2.4. Adaptive beamforming for NCR-MT

RAN1#110bis meeting agreement:

Agreement:

For NCR-MT which can support adaptive beams in C link,

- Rel-15 beam indication framework can be reused.

- Rel-17 beam indication framework (i.e., the unified TCI) can be reused as well, if the NCR supports. The gNB can configure the unified TCI for the NCR-MT, if the NCR-MT supports.

RAN4#105 meeting agreement:

Issue 3-1: adaptive beamforming for NCR-MT

- Agreements

  - Further discuss and define, if necessary, NCR-MT beam management related requirements based on further RAN1/RAN2 progress.

  - Candidate requirements include

    - Active TCI state switching delay

    - Uplink spatial relation switch delay

    - Active downlink TCI switching delay for unified TCI

    - Active uplink TCI switching delay for unified TCI

    - L1-RSRP measurement and reporting

  - FFS on requirements applicability to different NCR classes

Based on the previous agreement for Rel-18 NCR-MT in RAN1, NCR-MT at least need to support the Rel-15 TCI switching and Rel-17 unified TCI if supported. However compared with IAB RRM requirement, it should be okay not to define the TCI switching requirement for NCR-MT unless there are any problems identified for IAB-MT in the past.

Proposal 8: not to define the active TCI switching requirements for WA and LA NCR-MT in Rel-18.

In addition, regarding the beam information for NCR-Fwd access link, beam information for access link are informed to gNB and NCR via OAM agreed in last RAN1#111 meeting and network could further indicate the beam index of access link via the DCI indication (periodic beam indication) or RRC indication (aperiodic beam indication), however this is somehow quite different from the legacy TCI switching delay based on the previous L1-RSRP measurement report and further DCI or RRC indication for TCI state switch. For beam switching of NCR-Fwd access link, it could be done by the implementation without waiting for first SSB transmission after reception of RRC configuration.

In short, the beam switching interruption could be left up to the implementation and its beam switching delay from the hardware perspective is also quite limited (e.g. down to 100ns or even much less than it in the practice.)

Proposal 9: not to define the RRM requirement for WA and LA NCR-MT dynamic beam indication on the access link.

Figure 2. the legacy RRC based TCI state switch delay if the target TCI state is known

Agreement The following is supported to deliver the information to characterize the supported physical beam of NCR-Fwd for access link: Option-2: The information is informed to gNB and NCR via OAM - Note-1: In this option, how to characterize the beam information is based on implementation (e.g., declaration from NCR vendor). - Note-2: In this option, the beam(s) used by NCR-Fwd for access link is configured for gNB and NCR by OAM based on implementation. - The beam index in SCI corresponds to the configured beam(s) sequentially. Agreement For each periodic beam indication for access link, one RRC signalling is used with the information defined by the following: Option-2: - A list of X(1≤X≤Xmax) forwarding resource, each is defined as {Beam index, time resource} - FFS: The value of Xmax Each time resource is defined by {Starting slot defined as the slot offset in one period, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols} with dedicated field. - The periodicity is configured as part of the RRC signaling for periodic beam indication - The same periodicity is assumed for all time resource(s) in one periodic beam indication. - The reference SCS is configured as part of the RRC signaling for periodic beam indication - The same reference SCS is assumed for all time resource(s) in one periodic beam indication. Agreement For each aperiodic beam indication for access link, one DCI is used with the information defined by Option-1: - Lmax fields are used to indicate the beam information and each field refers to one beam index ; - Note: The bitwidth of this field is determined by the number of beams used for access link. - Tmax fields to indicate the time resource; - Note: A list of time resource is pre-defined by RRC signalling. The bitwidth of this field for time resource indication is determined by the length of list. - FFS: The value of Tmax - Down-select between Tmax=1 or Tmax=Lmax. - FFS: How to define the association between time indication and beam indication Each time resource is defined by {Starting slot defined as the slot offset, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols} with dedicated field.

## 2.5. The support of BFD/BFR/RLM for NCR-MT

RAN1#111 meeting agreement:

As optional functionalities for the NCR-MT, at least Rel-15 legacy BFD/BFR/RLM mechanisms are supported

- FFS: The behavior of NCR-Fwd when BFR/RLF happen in C link.

In last RAN4 meeting, RAN1 reached the above agreement on BFD/BFR/RLM for NCR-MT. In other words, from our understanding, this Rel-15 functionality should be supported for NCR-MT. Compared with Rel-16 IAB-MT, RLM/BFD/BFR requirement was defined corresponding in clause 12.3 of TS38.174 without considering DRX configuration. It is also reasonable to define the RLM/BFD/RLM requirement For NCR-MT. In addition, from our understanding, DRX configuration for NCR-MT could be still allowed in practice which is slightly different from IAB-MT for backhaul link transmission.

Proposal 10: to define the BFD/BFR/RLM requirement for LA NCR-MT and reuse the existing IAB-MT RRM requirement in T38.174 clause 12.3 as baseline and further consider the DRX configuration for NCR-MT.

## 2.6. The support of CA/DC for NCR-MT

For Rel-18 NCR-MT, there are no conclusion for the support of CA/DC for NCR-MT in other WGs, however from our understanding, single carrier operation should be sufficient for NCR-MT to the receive of the side control information defined by RAN1/2/3 and provide some necessary feedback to donor gNB. In short, we don’t see the strong necessity to support the CA and DC, therefore we don’t need to define the following RRM requirements for NCR-MT.

| Requirements | Detailed RRM Scope | Is requirement needed for Rel-18 NCR- MT? | Technical justifications |
| --- | --- | --- | --- |
| SCell activation/deactivation delays |  | No | For Rel-18 NCR-MT, single carrier operation should be sufficient for the reception of the side control information defined by RAN1/2/3. Don’t see the strong necessity to have Scell configuration and activation. |
| Interruptions related to SCells addition, release, activation, deactivation | Interruptions on serving cells due to any of the addition, release, activation, or deactivation procedures | No | Similar reason as Scell activation/deactivation of CA. |
| PSCell addition/release delays |  | No | NR-DC is not expected to be supported for Rel-18 NCR-MT. |
| MT timing related requirements | - MT MRTD/MTTD | No | MT MRTD and MTTD is not applicable for NCR-MT. |

Proposal 11: don’t define the RRM requirement with related CA/DC for NCR-MT.

Conclusions

In this contribution, we want to share further views on NCR-MT from the RRM perspective and proposals are made as following:

Proposal 1: to define the RRC re-establishment requirement for LA NCR-MT in Rel-18 only.

Proposal 2: to only define the 4 step RACH requirement for LA NCR-MT and leave 2 step RACH by the implementation.

Proposal 3: to define initial transmit timing requirement Te for LA NCR-MT in Rel-18 only.

Proposal 4: For the gradual timing adjustment requirement Tp and Tq for LA NCR-MT, propose to reuse the requirement defined for IAB-MT in Rel-16.

Proposal 5: to define the timer accuracy requirement for LA NCR-MT and reuse the requirement in clause 7.2.1 of TS38.133.

Proposal 6: to define TA adjustment accuracy requirement for LA NCR-MT and reuse the same requirement as Rel-16 IAB-MT.

Proposal 7: not to define the active BWP switching requirement for WA and LA NCR-MT in Rel-18.

Proposal 8: not to define the active TCI switching requirements for WA and LA NCR-MT in Rel-18.

Proposal 9: not to define the RRM requirement for WA and LA NCR-MT dynamic beam indication on the access link.

Proposal 10: to define the BFD/BFR/RLM requirement for LA NCR-MT and reuse the existing IAB-MT RRM requirement in T38.174 clause 12.3 as baseline and further consider the DRX configuration for NCR-MT.

Proposal 11: don’t define the RRM requirement with related CA/DC for NCR-MT.

References

- RP-222673, New WID on NR network-controlled repeater, ZTE, Sanechips, Approved

- RP-222323, TR 38.867 for NR Network-Controlled repeater, RAN#97-e

- R4-2217263,WF on RRM requirements for NCR, ZTE, approved.

- R4-2217471,WF on NCR RF requirements, ZTE, approved.

- R4-2220708,WF on NR Network-controlled Repeaters RRM requirements, ZTE

- R4-2220076,Topic summary for [105][230] NR_netcon_repeater_RRM

- R4-2219845,Discussion on RRM impacts for NR network-controlled repeaters, Huawei.

- R4-2303257, WF on NCR-MT RRM requirements, ZTE, approved.