3GPP TSG-RAN WG4 Meeting#106bis                               R4-2305417

E-meeting,April 17 – April 26,2023

Agenda item:	5.27.1.1

Source: 	ZTE Corporation

Title: 	Further discussion on system parameter for NTN in Ka band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. In this contribution, we want to share further views for system parameters for NTN in Ka-band.

Agreement:

- FSS spectrum should be included in 3GPP band definition for the purpose of enabling ESIM/moving VSAT moveable NTN terminal with priority.  [Fixed VSAT in FSS allocation – Companies to check RAN4 ToR to see if there is a restriction on fixed device in fixed spectrum]

- Define several NTN Ka-bands to address the diversity of spectrum allocation to ESIM by national regulators, RAN4 should consider specifying several “NTN Ka-bands”

Discussion

## 2.1.	Asymmetric channel bandwidths

The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth.

In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center frequency separation (defined in table 5.4.4-1) as following:

ΔFTX-RX = | (BWDL – BWUL)/2 |

The operating bands and supported asymmetric channel bandwidth combinations are defined in table 5.3.6-1.

Observation 1: the legacy restriction for Tx-Rx carrier center frequency separation might be not needed if there are no filtering or duplex between Ka-band DL and UL.

## 2.2. Channel raster and sync raster

The channel raster for NTN Ka-band could be be the same as band n257, i.e. 60/120kHz to the simplify the VSAT /ESIM/MES implementation complexity for initial access. In addition, sync raster could use similar approach as band n257. It should be noted that range of NREF should fall into 3GHz to 24.25GHz instead of 24.25-52.6GHz. Therefore global raster should be still 15kHz while with step size as 4 and 8 for 60kHz and 120kHz ΔFRaster.

Proposal 1: to use following NR-ARFCN in the Table 2.3-1/2.3-2 for NTN Ka-band.

Table 2.2-1: NR-ARFCN parameters for the global frequency raster [TS 38.101-5]

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs [MHz] | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 17300 – 20200 | 15 | 17300 | 1580000 | 1553336 – 1746664 |

Table 2.2.2. NR-ARFCN for NTN Ka-band

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n512, n511,n510 | 60 | 1553336 – <4> – 1746664 |
|  | 120 | 1553336 – <8> – 1746664 |

Similar as NR-ARFCN, it should be noted that range of GSCN should fall into 3GHz to 24.25GHz instead of 24.25-52.6GHz. Therefore sync raster should be still 1.44MHz while with step size as 12 (17.28MHz) and 24 (34.56MHz) for 120kHz and 240kHz SSB.

Table 2.2-3: GSCN parameters for the global frequency raster

| 3000 – 24250 MHz | 3000 MHz + N * 1.44 MHz N = 0:14756 | 7499 + N | 7499 – 22255 |
| --- | --- | --- | --- |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. |  |  |  |

| 17700 – 20200 MHz | 3000MHz MHz + N *1.44 MHz N = 10209:11944 | 7499 + N | 17430 – 19443 |
| --- | --- | --- | --- |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. |  |  |  |

Table 2.2.4. GSCN for NTN Ka-band

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n512] | 120 kHz | Case D | 17448- <12> - 19428 |
|  | 240 kHz | Case E | 17472 - <24> - 19416 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [10]. NOTE 2: Step size 12 equal to 17.28MHz by 1.44MHz*12 and step size 24 equals to 34.56MHz by 1.44MHz*24. |  |  |  |

Proposal 2: to use following GSCN in the Table 2.2-3/2.2-4 for NTN Ka-band MSS-band.

Conclusions

In this contribution, we want to share some further views from system parameter perspective for NTN Ka-band.and proposals are made as following:

Observation 1: the legacy restriction for Tx-Rx carrier center frequency separation might be not needed if there are no filtering or duplex between Ka-band DL and UL.

Proposal 1: to use following NR-ARFCN in the Table 2.3-1/2.3-2 for NTN Ka-band.

Table 2.2-1: NR-ARFCN parameters for the global frequency raster [TS 38.101-5]

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs [MHz] | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 17300 – 20200 | 15 | 17300 | 1580000 | 1553336 – 1746664 |

Table 2.2.2. NR-ARFCN for NTN Ka-band

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n512, n511,n510 | 60 | 1553336 – <4> – 1746664 |
|  | 120 | 1553336 – <8> – 1746664 |

Proposal 2: to use following GSCN in the Table 2.3-3/2.3-4 for NTN Ka-band MSS-band.

Table 2.2-3: GSCN parameters for the global frequency raster

| 3000 – 24250 MHz | 3000 MHz + N * 1.44 MHz N = 0:14756 | 7499 + N | 7499 – 22255 |
| --- | --- | --- | --- |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. |  |  |  |

| 17700 – 20200 MHz | 3000MHz MHz + N *1.44 MHz N = 10209:11944 | 7499 + N | 17430 – 19443 |
| --- | --- | --- | --- |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. |  |  |  |

Table 2.2.4. GSCN for NTN Ka-band

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n512] | 120 kHz | Case D | 17448- <12> - 19428 |
|  | 240 kHz | Case E | 17472 - <24> - 19416 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [10]. NOTE 2: Step size 12 equal to 17.28MHz by 1.44MHz*12 and step size 24 equals to 34.56MHz by 1.44MHz*24. |  |  |  |

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828

- R4-2217496, Email discussion summary for [104-bis-e][312] NR_NTN_enh_Part1

- R4-2217467, WF on NR NTN Enhancements Part1, Thales, approved.

- R4-1709181,WF on NR band Numbering, approved.

- R4-2220224 Ad-hoc minutes for NTN above 10GHz band introduction

- R4-2220239 WF for above 10GHz band definition and system parameters,Thales, Approved.

- R4-2302879, Ad-hoc minutes for system parameters of above 10GHz NTN band introduction, Qualcomm, Approved.