3GPP TSG-RAN WG4 Meeting#106bis                               R4-2305419

E-meeting,April 17 – April 26,2023

Agenda item:	5.27.4

Source: 	ZTE Corporation

Title: 	Further discussion on UE RF requirements for NTN in Ka-band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. During the last RAN4 meeting, there were some initial discussions for NTN UE in Ka-band with some high level agreement reached in [9]. In this contribution, we want to share some further understandings for NTN UE in Ka-band.

Discussion

## 2.1. RF requirement

2.1.1. UE types

In the previous meeting, we have extensive discussions on the naming for UE types and how many UE types are needed in 3GPP specification, however there are still no much progress for it. However for the progress, we need to move forward regardless of the naming or the number of UE types. Our proposal is to start with single naming NTN VSAT and further discuss the number of NTN VSAT types if there are any requirement differentiation.

Proposal 1: start with single name for NTN UE (e.g. NTN VSAT) and further discus the number of UE types if there are any requirement differentiation.

2.1.2. Beam tracking requirement

Agreement:

- Discuss whether and how to define the requirements for beam tracking for NTN UE.

In the last RAN4 meeting, there were some discussion on BC requirement and finally it was agreed to discuss further how to define the requirement for it. Indeed based on the inputs in the previous meeting, it seems that following two regulation could be used as reference for the requirement definition in 3GPP.

- the (Rx and Tx) beam pointing error are compliant with the relevant ETSI harmonized standard. (e.g.

  - EN 303 978    “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands”;

  - EN 303 979    “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in non-geostationary orbit, operating in the 27,5 GHz to 29,1 GHz and 29,5 GHz to 30,0 GHz frequency bands”).

In addition, regarding the following EN 301 428 regulation, it seems that it’s targeted for other bands (e.g. Ku bands) instead of Ka-band.

Proposal 2: for beam tracking requirements of NTN VSAT, propose to use EN 303 978 and 303 979 as reference.

EN 301 428;

2.1.3. Off-axis and On-axis requirements

In the last RAN4 meeting, there are some initial discussions on off-axis and on-axis related requirement which is coming from the existing regulation. From our understanding, this kind of regulatory requirement should be also considered for NTN VSAT in 3GPP.

Proposal 3: to consider the Off-axis and On-axis EIRP and spurious emission requirement in EN 303 978 and 303 979.

2.1.4. Tx power related requirements

First of all, there are the different antenna aperture from the same frequency range which are used to meet different coverage requirement. For instance, for 60cm parabolic antenna aperture which could be used for M-ESIM close to sea coast, however for 80cm parabolic antenna aperture which could be used for E-SIM far away from sea coast for international shipping.

In addition, based on the following required EIRP for User spot 11 L beam on Inmarsat 5-F3 at 179.6°E in Figure 4, to meet the uplink coverage requirement in different areas, then EIRP are required differently by satellite vendors. The required EIRP might be also depending on specific satellite and its specific beams and its geographical locations of VSAT and ESIM. From our understanding, it’s not necessary to define any EIRP limits and leave this to the market demand.

Figure 1. The required EIRP for the User Spot 11 L beam is available on Inmarsat 5-F3 at 179.6°E [12]

In addition, based on EN 303 978, the EIRP limit for NTN VSAT could based on declaration, therefore from 3GPP perspective, it should be also reasonable to use the declared approach.

Proposal 4: for EIRP of NTN VSAT, propose to use the declaration approach and FFS of any signalling to network.

locations of VSAT and ESIM. From our understanding, it’s not necessary to define any EIRP limits and leave this to the market demand.

Under operational conditions an ESOMP may dynamically change the occupied bandwidth and other transmission parameters (e.g. FEC, modulation, symbol rate) of the transmitted signal. For each declared occupied bandwidth an EIRPmax and a nominated bandwidth shall be declared by the applicant. For the purposes of verifying that the ESOMP complies with these specifications, the applicant may declare the worst case combination of transmission parameters. The following specifications apply to the ESOMP for each occupied bandwidth and other transmission parameters. In the present document, ESOMPs that use OFDM modulation shall be considered as transmitting a single carrier.

Another question is whether we need to define the spherical coverage requirement for NTN VSAT. From our understanding, it might be not needed anymore since EIRP is up to the vendor declaration. For each device, there might be different radiation pattern and different spherical coverage performance. In addition, if there are no regulation upper limit for NTN VSAT, it might be also not necessary to the NTN VSAT maximum output power limits.

Proposal 5: propose not to consider the spherical coverage requirement for NTN VSAT and no requirement for maximum output power if there are no existing regulation limits.

Table 1. Summary of related UE RF requirement

|  | Band-specific or not | Applicability |
| --- | --- | --- |
| Transmitter Characteristics |  |  |
| General | No | To follow the existing text from TN UE in TS 38.101-2 |
| Tx power | Yes | Please see the above analysis |
| MPR | No | Since EIRP value is up to the vendor’s declaration, it is not necessary to define MPR requirement similar as IAB and ATG CPE. |
| A-MPR | Yes | Not necessary to define A-MPR requirement for it similar as MPR. |
| Configured Tx power | No | Not necessary to define configured Tx power requirement since all values in equation would be up to declaration at the end. |
| Output Power Dynamics | No | The minimum output power for NTN VSAT, this could be further discussed. Transmitter OFF power and ON-OFF time mask and power control related parameter in TS 38.101-1/2 could be good starting point. |
| Transmit signal quality |  |  |
| - Frequency error | No | to follow the requirement defined in TS38.101-5 where UE UL pre-compensation is still needed. |
| - Transmit modulation quality | No | To follow the existing requirement defined for TS 38.101-2, however the maximum modulation order could be further discussed similar as Rel-17 NR over NTN Carrier leakage and in-band emission are also power class specific requirement and this could be further discussed. Carrier leakage and in-band emission requirement might be not necessary as Rel-16 FR2 IAB device. |
| Output RF spectrum emissions |  |  |
| - Occupied bandwidth | No | To follow the existing requirement defined for TS 38.101-1/2. |
| - Out of band emission |  |  |
| - SEM | No | This depends on the outcome of coexistence study. The following RF spectrum from FCC 47CFR25.138, 47CFR25.202 could further checked |
| - Additional SEM | Yes | additional requirement are expected for ITU resolution 169 in WRC-19 and [156] for WRC-15 |
| - ACLR | No | This depends on the outcome of coexistence study. |
| - Spurious emission |  |  |
| - General | No | To follow the existing requirement defined for TS 38.101-2. |
| - For UE coexistence | Yes | Coexistence requirement for the surrounding TN bands should be considered. |
| Transmit intermodulation | No | Not applicable similar as FR2 UE RF |
| Beam correspondence | No | Please see the above analysis |
| Receiver characteristics |  |  |
| General | No |  |
| Diversity characteristics | No |  |
| Reference sensitivity | Yes | For NTN VSAT, the following requirements should be defined for NTN VSAT UE. - EIS on-axis up to the declaration |
| Maximum input level | No | Further system level evaluation is needed and this requirement might be relaxed similar as Rel-17 NR NTN. |
| ACS | No | This depends on the outcome of coexistence study. |
| Blocking characteristics |  |  |
| - In-band | No | This depends on the outcome of coexistence study. |
| - Out-of-band | NA | NA |
| - Narrow band | NA | NA |
| Spurious response | NA | NA. |
| Intermodulation | NA | NA |
| Spurious emissions | No | To follow the existing requirement defined for TS 38.101-1/2. |

Conclusions

In this contribution, we want to share some further views on UE RF requirements for NTN in Ka-band.and proposals are made as following:

Proposal 1: start with single name for NTN UE (e.g. NTN VSAT) and further discus the number of UE types if there are any requirement differentiation.

Proposal 2: for beam tracking requirements of NTN VSAT, propose to use EN 303 978 and 303 979 as reference.

EN 301 428;

Proposal 3: to consider the Off-axis and On-axis EIRP and spurious emission requirement in EN 303 978 and 303 979.

Proposal 4: for EIRP of NTN VSAT, propose to use the declaration approach and FFS of any signalling to network.

Proposal 5: propose not to consider the spherical coverage requirement for NTN VSAT and no requirement for maximum output power if there are no existing regulation limits.

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828

- https://www.kingsat-tech.com/product/enhanced-vsat-ku-band-3-axis-stabilized-and-4-axis-tracking114.html

- https://www.mitsubishielectric.com/news/2020/0206.html

- R4-2217792, Email discussion summary for [104-bis-e][140] NR_NTN_enh_UERF, ZTE, Noted

- R4-2217743, WF on NTN UE in Ka-band, ZTE, Approved

- http://satstar.net/satellites/space2.html

- https://www.satbeams.com/satellites

- http://satstar.net/beams/inmar5f3_sb11.html

- R4-2216515,NTN enhancements – Regulatory aspects, Ericsson, Noted

- R4-2220573	WF on NTN UE RF requirements, ZTE, approved.

- R4-2220119, Topic summary for [105][139] NR_NTN_enh_UERF, ZTE

- https://www.etsi.org/deliver/etsi_en/301400_301499/301459/02.01.01_60/en_301459v020101p.pdf

- RF Spectrum FCC 47CFR25.138, 47CFR25.202,

- https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-25/subpart-C/section-25.202

- https://www.digisat.org/enterprise-75cm-professional-ka-band-terminal

- https://www.satsig.net/eut2beac.htm

- R4-2303533,WF on NTN UE RF requirements for Ka band, ZTE, Approved.

- Solution 169 pdf limit ESIM to France  E-PFD mask

- ITU-R recommendation S.524