3GPP TSG RAN WG4 Meeting #106-bis-e		 R4-2305420

# 17 April - 26 April 2023, e-Meeting

Agenda Item:	5.4.2.2

Source: 	MediaTek Inc.

Title: 	Discussion on how to implement CA_n28-n105

Document for:	Discussion

1. Introduction

In RAN-P#99, a new SI was approved to further study on more L-L-L band combination cases[1]. In this contribution we provide our study on CA_n28-n105

2. Discussion

2.1 RF Architecture

The operation frequency of CA_n28-n105 is listed below

| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| --- | --- | --- | --- |
| n105 | 663 MHz – 703 MHz | 612 MHz – 652 MHz | FDD |

There may be two feasible RF architecture to realize the carrier aggregation: 2-antenna and 3-antenna architecture:

Figure 1. Three antenna architecture

Figure 2. Two antenna architecture

Similar to the antenna configuration discussed for CA_n5-n28, 3-antenna architecture is easier for UE implementation but may be challenge for smart phone form factor antenna design due to limited space. 2-antenna architecture is easier for antenna design but would be quite challenge for UE implementation on both  passive component design and UE RF design from baseband to radio transceiver. The passive component feasibility of dual-band triplexer needed to be studied first. There are more implementation issues on sharing TX path due to the uplink frequency range of the two bands are consecutive. If UE is configured UL CA, single band MPR/AMPR may not be applicable and the applicable MPR/AMPR may need to be characterized. The two transmit bands may need to be synchronized, to apply intra-band NRCA requirement may be needed. Further, TX AGC control of the two transmit bands could be another issue if the two transmit band requires different uplink power.

Observations regarding 2-antenna architecture:

Observation 1: The passive component feasibility of dual-band triplexer needed to be studied first

Observation 2: If Tx path is shared by the two bands, single band MPR/AMPR may not be applicable and the applicable MPR/AMPR may need to be characterized

Observation 3: For shared TX path, TXAGC control as well as synchronization requirement of the two transmit bands could be another issue if the two transmit band requires different uplink power

Observation regarding 3-antenna architecture:

Observation 4: The antenna design for 3 low band antenna may be challenge for smart phone form factor implementation.

Proposal 1: Considering feasibility on UE implementation, 3-antenna architecture is baseline assumption for CA_n28-n105. Other architectures are not precluded

2.2 MSD due to dual-uplink IMDs

The IMD frequency analysis is listed in the table:

Table 1 IMD interference analysis for CA_n28-n105 with 2 ULs

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 703 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1406 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2109 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2812 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3515 | 3515 | 3740 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 3978 | 4218 | 4218 | 4488 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 4641 | 4921 | 4921 | 5236 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 85 | 0 | 1366 | 1451 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 578 | 703 | 703 | 833 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2029 | 2154 | 2069 | 2199 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 170 | 0 | 2732 | 2902 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1241 | 1406 | 1406 | 1581 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2692 | 2857 | 2772 | 2947 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2329 | 2109 | 2109 | 1904 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3475 | 3695 | 3355 | 3560 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 918 | 703 | 703 | 493 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3435 | 3650 | 3395 | 3605 |

From the above table, it can be found the CA combination is subject to MSD due to dual-uplink IMD3 and IMD5 on both bands.

Proposal 2: Proposed test points for MSD due to dual-uplink IMDs

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n105 | n28 | 745.5 | 5 | 25 | 800.5 | [TBD] | FDD | IMD3 |
|  | n105 | 690.5 | 5 | 25 | 639.5 | N/A | FDD | N/A |
|  | n28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | IMD3 |
|  | n105 | 694.5 | 5 | 25 | 643.5 | [TBD] | FDD | N/A |
|  | n28 | 715 | 5 | 25 | 770 | [TBD] | FDD | IMD5 |
|  | n105 | 687.5 | 5 | 25 | 636.5 | N/A | FDD | N/A |
|  | n28 | 715.5 | 5 | 25 | 770.5 | N/A | FDD | IMD5 |
|  | n105 | 690 | 5 | 25 | 639 | [TBD] | FDD | N/A |

3. Conclusion

Observations regarding 2-antenna architecture:

Observation 1: The passive component feasibility of dual-band triplexer needed to be studied first

Observation 2: If Tx path is shared by the two bands, single band MPR/AMPR may not be applicable and the applicable MPR/AMPR may need to be characterized

Observation 3: For shared TX path, TXAGC control as well as synchronization requirement of the two transmit bands could be another issue if the two transmit band requires different uplink power

Observation regarding 3-antenna architecture:

Observation 4: The antenna design for 3 low band antenna may be challenge for smart phone form factor implementation.

Proposal 1: Considering feasibility on UE implementation, 3-antenna architecture is baseline assumption for CA_n28-n105. Other architectures are not precluded

Proposal 2: Proposed test points for MSD due to dual-uplink IMDs

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n105 | n28 | 745.5 | 5 | 25 | 800.5 | [TBD] | FDD | IMD3 |
|  | n105 | 690.5 | 5 | 25 | 639.5 | N/A | FDD | N/A |
|  | n28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | IMD3 |
|  | n105 | 694.5 | 5 | 25 | 643.5 | [TBD] | FDD | N/A |
|  | n28 | 715 | 5 | 25 | 770 | [TBD] | FDD | IMD5 |
|  | n105 | 687.5 | 5 | 25 | 636.5 | N/A | FDD | N/A |
|  | n28 | 715.5 | 5 | 25 | 770.5 | N/A | FDD | IMD5 |
|  | n105 | 690 | 5 | 25 | 639 | [TBD] | FDD | N/A |

Reference:

- RP-230789 Study on enhancement for sub-1GHz NR band combinations