3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305426
Online, April 17 – April 26, 2023

Agenda item:	5.31.2.1.3

Source:	OPPO

Title:	R18 SL-U single CC Rx requirements

Document for:	Approval

- Introduction

Rel-18 NR SL evolution WI was approved with latest WID in [1] and the work in RAN4 starts from this meeting. In this paper, the RF impacts will be discussed.

- Discussion

2.1 Refsens

In current specification, the REFSENS of NR-U and NR V2X are different in the RB configurations and RMCs.

NR-U has reused the RMC of NR single CC, and with same RB configuration of NR single CC without considering the NR-U interlaced RB allocation feature or intraCellGuardBandsDL-List configurations. In other words, the REFSENS requirement is defined with no intra-cell guard bands in DL, and UL configuration is with all sub-bands and all RB’s fully allocated using DFT-S-OFDM [2]. In this case, the NR-U has no difference from the NR single CC.

Observation 1:   NR-U REFSENS has no difference from the NR single CC, considering there is no intra-cell guard bands in DL and the UL configuration is with all sub-bands and all RB’s fully allocated.

On the contrary, NR V2X has defined specific RMCs [6][7] for Rx requirements considering the sub-channel size restrictions. With that, NR V2X also used specific SNR assumptions, and NF in REFSENS calculation according to [3][8]:

The V2X UE REFSENS is defined by the following equation: REFSENSV2X = kTB + SNRV2X +10log10(LCRB*SCS*12/RX_BW) +( NFV2X+ IM) – Diversity gain Where - kTB: Thermal noise level is [-174dBm(kT) + 10*log10(RX BW)] dBm. - NF: Noise figure. 13 dB is used for LAA and can be reused for NR V2X requirements. Assumed NF is 9dB < 3GHz, NF is 10dB>= 3GHz (e.g B42, n77, n78, n79…) at licensed bands at FR1. - IM: 2.5 dB is assumed. When the number of RB size equal to or less than 24RBs, 0.5dB additional relaxation is allowed. - Target SNR: -0.5 dB - Diversity gain: 3dB

And it was clarified in [4] that the REFSENS is defined at the UE antenna connector without considering the external components.

Observation 2:   NR V2X has defined specific RMCs and SL Tx RB configurations for Rx requirements which considered the sub-channel size restrictions.

Therefore, SL-U should follow NR V2X principles in RB configurations, RMCs, and other REFSENS calculation assumptions like SNR and NF to derive the final results.

The RB allocations need to consider the number of subchannels and subchannel size, the NRB values to be used in the spec for RF requirements should be as large as possible as long as it does not exceed the NRB values of Uu for supported CBWs [5]. In 38.331, the subchannel size is as following figure. The proposed max RB configurations for CBW 60MHz, 80MHz, and 100MHz are as shown in table 1.

Proposal 1:         The SL Tx RB configurations in table 1 are adopted for SL-U REFSENS, especially the RB configurations for 60MHz, 80MHz and 100MHz CBW which are new for SL.

Table 1 SL Tx RB configurations for SL-U REFSENS

|  |  | NR band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Band | SCS kHz | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Duplex Mode |
| n46 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 135 |  |
| n96 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 135 |  |
| n102 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 135 |  |

The corresponding REFSENS are as shown in table 2.

Proposal 2:         The SL-U REFSENS are adopted in Table 2.

Table 2 REFSENS for SL-U

| NR SL-U Band | SCS kHz | Channel bandwidth / PREFSENS_V2X (dBm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Duplex Mode |
| n46 | 15 | -89.2 | -86.1 |  |  |  | HD |
|  | 30 | -89.4 | -86.2 | -84.4 | -83.1 | -82.1 |  |
|  | 60 | -89.6 | -86.4 | -84.6 | -83.4 | -82.1 |  |
| n96 | 15 | -89.2 | -86.1 |  |  |  | HD |
|  | 30 | -89.4 | -86.2 | -84.4 | -83.1 | -82.1 |  |
|  | 60 | -89.6 | -86.4 | -84.6 | -83.4 | -82.1 |  |
| n102 | 15 | -89.2 | -86.1 |  |  |  | HD |
|  | 30 | -89.4 | -86.2 | -84.4 | -83.1 | -82.1 |  |
|  | 60 | -89.6 | -86.4 | -84.6 | -83.4 | -82.1 |  |

2.2 Max input level

In current specification, there is no specific max input level requirement for NR-U which can be interpreted as same requirement as NR single CC. And the NR V2X use same requirement as NR single CC also [9], but with NR V2X RMCs.

For SL-U, the NR V2X max input level can be reused.

Observation 3:   NR V2X and NR-U both reuse the NR single CC max input level, and NR V2X apply specific RMCs.

Proposal 3:         SL-U max input level reuse NR V2X requirements.

2.3 ACS

In NR single CC, the ACS was defined with two test configurations, i.e. case1 and case2, and they were further distinguished with two frequency ranges, one is for bands below 2.7GHz, and the other is for bands above 3.3GHz. In NR V2X, it was intended to follow the same requirements as NR single CC which can be found with the description in TR38.886 [8], copied below:

“ACS reflects the digital domain filter attenuation capability, which is model together with ACLR in ACIR. Based on PRR metric. In co-existence study, the existing NR ACS requirement reuse for both NR UE to V2X UE and DSRC UE to V2X UE urban interference scenarios, PRR loss is less than 5%. Therefore, it is proposed that the same ACS values shall be kept unchanged for V2X UE at n47. In licensed band, RAN4 shall reuse the same ACS values since there was legacy NR or LTE system. As 10MHz is typical channel bandwidth used in the V2X band, the interferer bandwidth shall be used with 10MHz for NR V2X operating bands in Table 8.1-1.”

However, in the spec, the ACS for NR V2X is as below figure 1 which is same as the ACS requirement for NR single CC with frequencies below 2.7GHz. Considering the band n47 is above 3.3GHz, the ACS requirement seems missing in current spec.

Figure 1 ACS for NR V2X in 38.101-1

Observation 4:   NR V2X ACS requirement for bands above 3.3GHz, especially for n47, is missing in the spec which need to be introduced in the maintenance.

Proposal 4:         Introduce the NR V2X ACS requirement for bands above 3.3GHz, especially for n47, as maintenance.

The ACS for NR-U was defined for bands above 3.3GHz considering the unlicensed bands of n46, n96, n102 are all high bands currently. However, the requirement format is different from NR single CC which can be found below in figure 2, i.e. the ACS is adjusted according to the CBW while the NR single CC is fixed as 33dB for bands above 3.3GHz. The ACS requirement has been extensively discussed in NR-U WI with various view considering the difference of ACLR, ACIR impacts, and comparison with WIFI system, and coexistence with LAA, the final value is a compromised outcome [2][10-13].

Figure 2 ACS for NR-U in 38.101-1

And for the testing configurations, the case2 in NR single CC was not defined with considerations of deployment scenarios [14] or case2 in reality is similar to case1 in NR-U [11].

When SL-U works in unlicensed bands, the coexistence with other systems like WIFI, LAA and NR-U need to be considered, and the current requirements for NR-U can be reused to achieve similar protection levels.

Proposal 5:         Reuse NR-U ACS requirements and test configurations with NR V2X RMC for SL-U to get similar coexistence results.

2.4 IBB

NR V2X reused the IBB requirements of LTE V2X with test configurations of NR single CC. The NR-U IBB requirements were defined referring to the LAA system and the 20MHz channel allocation in unlicensed spectrum [15].

For SL-U, similar as ACS the IBB requirements are more relevant to the NR-U, the requirements can be reused.

Figure 3 IBB requirement for NR V2X

Figure 4 IBB requirement for NR-U

Proposal 6:         Reuse NR-U IBB requirements and test configurations with NR V2X RMC for SL-U.

2.5 OBB/Spurious response

Similar as IBB, the OOB of NR-U can be reused for SL-U.

Proposal 7:         Reuse NR-U OBB and Spurious response requirements and test configurations with NR V2X RMC for SL-U.

2.6 NBB

Narrow band blocking was introduced in NR single CC to mimic the potential interference coming from narrow band systems, but such system is not existing in the targeting unlicensed spectrum therefore the NBB is not introduced in NR-U, neither is needed in SL-U.

Proposal 8:         Narrow band blocking is not introduced for SL-U.

2.7 Wide band intermodulation

NR V2X reused the intermodulation requirement of LTE V2X. And NR-U is different from NR V2X in interferer BW and location. With the above other Rx requirements reusing NR-U requirements, the wide band intermodulation requirements and configurations of NR-U is proposed also to be reused for SL-U.

Figure 5 Wide band intermodulation requirement for NR V2X

Figure 6 Wide band intermodulation requirement for NR-U

Proposal 9:         Reuse NR-U Wide band intermodulation requirement and test configuration with NR V2X RMC for SL-U.

- Conclusions

In this contribution, the Rel-18 NR SL evolution Rx requirements for single CC are analysed and got the following observations and proposals.

Observation 1:   NR-U REFSENS has no difference from the NR single CC, considering there is no intra-cell guard bands in DL and the UL configuration is with all sub-bands and all RB’s fully allocated.

Observation 2:   NR V2X has defined specific RMCs and SL Tx RB configurations for Rx requirements which considered the sub-channel size restrictions.

Proposal 1:         The SL Tx RB configurations in table 1 are adopted for SL-U REFSENS, especially the RB configurations for 60MHz, 80MHz and 100MHz CBW which are new for SL.

Proposal 2:         The SL-U REFSENS are adopted in Table 2.

Observation 3:   NR V2X and NR-U both reuse the NR single CC max input level, and NR V2X apply specific RMCs.

Proposal 3:         SL-U max input level reuse NR V2X requirements.

Observation 4:   NR V2X ACS requirement for bands above 3.3GHz, especially for n47, is missing in the spec which need to be introduced in the maintenance.

Proposal 4:         Introduce the NR V2X ACS requirement for bands above 3.3GHz, especially for n47, as maintenance.

Proposal 5:         Reuse NR-U ACS requirements and test configurations with NR V2X RMC for SL-U to get similar coexistence results.

Proposal 6:         Reuse NR-U IBB requirements and test configurations with NR V2X RMC for SL-U.

Proposal 7:         Reuse NR-U OBB and Spurious response requirements and test configurations with NR V2X RMC for SL-U.

Proposal 8:         Narrow band blocking is not introduced for SL-U.

Proposal 9:         Reuse NR-U Wide band intermodulation requirement and test configuration with NR V2X RMC for SL-U.

- References

- RP-222806, WID revision: NR sidelink evolution, OPPO, RAN#98-e, Dec 2022.

- R4-2005222, WF on NR-U UE Rx requirements, QC, RAN4#94bis-e, Apr 2020

- R4-2011708, WF on REFSENS requirements for NR V2X UE at licensed/unlicensed bands, LGE, RAN4#96e, Aug 2020

- R4-2011710, LS on Rel-16 RAN4 Clarification for UE Antenna Connector Interpretation, RAN4->RAN5, RAN4#96e, Aug 2020

- R4-2010818, On NR V2X SL allocated RB size numbers for different CBWs, HW, RAN4#96e, Aug 2020

- R4-2010822, On NR V2X reference measurement channels, HW, RAN4#96e, Aug 2020

- R4-2011705, Correction on 5G V2X UE RF requirements in rel-16, LGE, RAN4#96e, Aug 2020

- TR38.886, V2X Services based on NR; User Equipment (UE) radio transmission and reception

- R4-2009170, WF on remaining Rx RF requirements for NR V2X, CATT, RAN4#95e, May 2020

- R4-2008435, WF on NR-U ACS and blocking requirements, Apple, RAN4#95e, May 2020

- R4-2010496, Discussion on NR-U UE ACS, Huawei, RAN4#96e, Aug 2020

- R4-2010346, Additional RX requirements for NR-U operation, Ericsson, RAN4#96e, Aug 2020

- R4-2011847, Email discussion summary for RAN4#96e_#107_NR_unlic_UE_RF, QC, RAN4#96e, Aug 2020

- R4-2004726, NR-U receiver ACS and blocking, QC, RAN4#94bis-e, Feb 2020

- R4-2008122, NR-U receiver ACS and blocking, QC, RAN4#95e, May 2020