3GPP TSG-RAN WG4 Meeting #106bis-e                                       R4-2305436

Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Cross band isolation MSD analysis for CA_n40-n41

Agenda Item:	4.27.2

Document for:	Approval

Introduction

A new WI on simultaneous Rx/Tx was approved in RAN#97e, in which CA_n40A-n41A is listed as one band combination to be analysed [1]. This contribution provides cross band isolation MSD analysis for CA_n40-n41.

Discussion

Assumptions for MSD analysis

Filter/Duplexer assumptions

Band n41 duplexer performance

Typical duplexer performance of n41 is shown in the figure below:

Figure 3: Example duplexer performance of band n41

According to the available data from vendors, band n41 Rx/Tx filter rejection at n40 is around 35~39dB. We take the value of 35dB for the MSD calculation

Band n40 filter performance

In the analysis, we adopt value of 35dB for the band n40 Rx/Tx filter rejection at n41 referring to the filter performance of band n40 shown below.

Figure 4: Example filter performance of band n40

List of assumptions for MSD analysis

Table 1 lists the assumptions for cross band isolation MSD analysis for CA_n40-n41.

Table 1: Assumptions on MSD analysis for CA_n40-n41

| PA far end noise floor (dBm/Hz) | -130 |
| --- | --- |
| Transceiver noise floor at PA output (dBm/Hz) | -132 |
| n41 PA Tx noise regrowth beyond ACLR2 (dBc) | 40 for 10MHz @2505M 45 for 100MHz@2550M |
| n40 PA Tx noise regrowth beyond ACLR2 (dBc) | 40 for 10MHz @2395M 45 for 100MHz@2350M |
| PA output power at antenna port (dBm) | 26 |
| n41 Rx filter rejection at n40 Tx (dB) | 35 |
| n40 Tx filter rejection at n41Rx (dB) | 35 |
| n40 Rx filter rejection at n41Tx (dB) | 35 |
| n41Tx filter rejection at n40 Rx (dB) | 35 |
| RFIC IIP2 (dBm) | 50 |
| Band n40 Tx LO phase noise (dBc/Hz) | -155 |
| Band n41 Rx LO phase noise (dBc/Hz) | -155 |
| Antenna ISO (dB) | 10 |
| Band n40/n41 Tx FE Loss (dB) | 4 |
| Band n40/n41 Rx FE Loss (dB) | 4 |
| Band n40 Rx noise figure (dB) | 9 |
| Band n41 Rx noise figure (dB) | 11 |

MSD calculation

Based on the assumptions in section 2.1.2, cross band isolation MSD values are calculated for cases with either n40 or n41 as victim bands.

The noise and interference values as well as MSD results for band n40 with n41 as UL band and for band n41 with n40 as UL band are listed in Table 2 and Table 3. As the MSD values in spec for n41 in SUL_n41-n97 are assumed for CBW allocated at ACLR2, different noise levels are considered for different CBWs at different frequencies. It is noticed that MSD for 10MHz is similar to that specified for SUL_n41-n97, which is 20.6dB, however, the value for 100MHz is a bit different, which is 15.6 dB.

Table 2: Noise and interference in Band n40/n41 Rx

| MSD for 5MHz | Main | Diversity |
| --- | --- | --- |
| Couple noise from Tx thermal @LNA input (dBm/Hz) | -139.0 | -149.0 |
| IM2 induced by IIP2 @LNA input (dBm) | -94.0 | -114.0 |
| Noise induced by Rx reciprocal mixing @LNA input (dBm/Hz) | -174.0 | -184.0 |
| Noise induced by Tx LO phase noise @LNA input (dBm/Hz) | -174.0 | -184.0 |

Table 3: n40 MSD when n41 is UL band

| DL CBW (MHz) | 10 | 100 |
| --- | --- | --- |
| n40 MSD (dB) | 22.6 | 17.7 |

Table 4: n41 MSD when n40 is UL band

| DL CBW (MHz) | 10 | 100 |
| --- | --- | --- |
| n41 MSD (dB) | 20.6 | 15.6 |

Proposal 1: It is proposed to specify the crossband isolation MSD values for in CA_n40-n41 as in Table 3 and Table4.

Conclusion

This contribution provides further cross band isolation MSD analysis for CA_n40-n41. Given the close frequency separation between n40 and n41, the large MSD due to Tx noise spectrum regrowth is expected since the filter/duplexer could not provide enough rejection for the constituent bands to each other.

Proposal 1: It is proposed to specify the crossband isolation MSD values for CA_n40-n41 as in Table below.

| DL CBW (MHz) | 10 | 100 |
| --- | --- | --- |
| n40 MSD (dB) | 22.6 | 17.7 |

| DL CBW (MHz) | 10 | 100 |
| --- | --- | --- |
| n41 MSD (dB) | 20.6 | 15.6 |

- References

[1] RP-222648, New WID Simultaneous RxTx inter-band combinations in Rel-18, Huawei, HiSilicon