3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2305453

Online, April 17 – April 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | draftCR | rev | - | Current version: | 16.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to 38.101-3: Clarification on the application of time mask for Tx switching (R16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2023-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the LS R1-2302198 from RAN1, the clarification and update on the application of time mask for Tx switching is required. RAN1 informed RAN4 that Agreement - If the gNB provides sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier to absorb the switching period, - The time of no UL transmission allocated absorbs the switching period - Neither of the uplink transmissions (the one ending on the switch-from carrier nor the one starting on the switch-to carrier) are interrupted by the switching period. - The setting of uplinkTxSwitchingPeriodLocation has no impact. - Defer the discussion on whether/how to define the exact location of the switching period indicated by the UE capability in time domain to RAN4 - From RAN1 point of view, for Rel-16, the implication is to the time domain location of potential interruption of downlink reception if reported by the UE for the band combination - Defer the potential RAN1 spec change until RAN4 has had the time to react to the RAN1 LS to RAN4. - Send an LS to RAN4 requesting RAN4 to, in this regard, clarify TS38.101-1 subclauses 6.3A.3.3.2 and 6.3C.3.1 for CA, and SUL based UL Tx Switching, and to TS38.101-3 subclause 6.3B.4.1 for EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify that the time masks defined for CA and SUL are only applicable when gNB doesn’t provide sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier to absorb the switching period. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are some ambiguities for the application of CA and SUL Tx switching time masks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3B.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change>>

6.3B.4	Output power dynamics for switching between two uplink carriers

6.3B.4.1	E-UTRA and NR switching time mask between two uplink carriers

In addition to the requirements in 6.3B.0 and the maximum output power requirement specified in Table 6.2B.1.3-1 with inter-band EN-DC (two bands), the switching time mask specified in this sub-clause is applicable for an uplink band pair of a inter-band EN-DC configuration without SUL band when the capability uplinkTxSwitchingPeriod is present, and is only applicable for uplink switching mechanisms specified in sub-clause 6.1.6 of TS 38.214 [14] when gNB doesn’t provide sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier to absorb the switching period, where E-UTRA UL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2.

The switching periods described in Figure 6.3B.4.1-1 are only located in NR carrier when gNB doesn’t provide sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier to absorb the switching period,, and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

Figure 6.3B.4.1-1: Time mask for switching between E-UTRA UL carrier and NR UL carrier, where the switching period is located in NR carrier

The requirements apply for the case of co-located and synchronized network deployment with the max receiving timing difference of 3us between the two carriers.

<<End of Change>>