3GPP TSG-RAN WG4 Meeting #106bis-e                                       R4-2305458

Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon

Title: 	On inter-band con-current operation on Uu and sidelink

Agenda Item:	5.31.2.2

Document for:	Approval

Introduction

The WID revision [1] was approved in RAN#99e with the scope for the objective of con-current operation on Uu and sidelink in NR sidelink evolution.

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

In this contribution, we provide our consideration and initial analysis on RAN4 work for con-current operation of sidelink on unlicensed band and Uu on licensed band(s).

Discussion

The example band combination

In RAN4#106, it was agreed that an example band combination was needed for concurrent operation for sidelink on unlicensed band and Uu on licensed band.

Issue 1-4-1: Concurrent operation on Uu@Licensed and SL@Un-licensed Agreement: - Inter-band combinations for concurrent operation for Uu @Licensed and SL @Un-licensed are to be studied based on company inputs. In order to meet the WI objectives and schedule it is requested that combinations are decided in the next RAN4 meeting. - One example band combination is needed. FFS on which band combination is chosen.

In [2], an example band combination was proposed SL_n46A-n48A, with Unlicensed SL+ licensed Uu. In the table, the channel bandwidths for n46 and n48 are also proposed. We can take it as the starting point.

Table 1 Example band combination for Uu @Licensed and SL @Un-licensed

| SL configuration | Uplink configuration | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| SL_n46A-n48A | SL_n46A-48A | n46 | 20, 40, 60, 80 | 0 |
|  |  | n48 | 20 |  |

Proposal 1: Take Table 1 as the starting point to discuss the example band combination for Uu @Licensed and SL @Un-licensed.

RF requirements for con-current operation

The intra-band con-current operation of Uu and SL was specified in Rel-17. And the inter-band concurrent operation of Uu and NR V2X Sidelink was specified in Rel-16. The RF requirements for them can be the reference framework for inter-band concurrent operation of Uu @Licensed and SL @Un-licensed.

Table 2 The requirements for intra-band concurrent operaion of Uu and SL

Transmitter characteristics

Transmitter power

Minimum output power

Transmit OFF power

Transmit ON/OFF time mask

Power control

Frequency error

Transmit modulation quality

Occupied bandwidth

Spectrum emission mask

ACLR

Spurious emissions for UE co-existence

Transmit intermodulation

Receiver characteristics

REFSENS

Maximum input level

Adjacent channel selectivity

In-band blocking

Out-of-band blocking

Spurious response

Wide band Intermodulation

Considering that the requirements for sidelink on a single unlicensed spectrum haven’t been specified, the requirements for concurrent operation on Uu@Licensed and SL@Un-licensed can be on hold until the accomplishment of those for single unlicensed band.

Proposal 2: The requirements for concurrent operation on Uu@Licensed and SL@Un-licensed can be on hold until the accomplishment of the requirement for sidelink on a single unlicensed band.

Conclusion

This contribution provides our consideration and initial analysis with the following observation and proposals:

Proposal 1: Take Table 1 as the starting point to discuss the example band combination for Uu @Licensed and SL @Un-licensed.

Table 1 Example band combination for Uu @Licensed and SL @Un-licensed

| SL configuration | Uplink configuration | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| SL_n46A-n48A | SL_n46A-48A | n46 | 20, 40, 60, 80 | 0 |
|  |  | n48 | 20 |  |

Proposal 2: Discuss how to handle the requirements for concurrent operation on Uu@Licensed and SL@Un-licensed before the accomplishment of the requirement for sidelink on a single unlicensed band.

- References

[1] RP-230077 WID revision: NR sidelink evolution, OPPO, TSG RAN Meeting #99-e, March. 20-23, 2023

[2] RP-230490 New WID: Rel­18 Inter­band con-current operation of NR/LTE Uu in licensed bands and NR SL in unlicensed band, Huawei, Hisilicon, TSG RAN Meeting #99-e, March. 20-23, 2023