3GPP TSG-RAN WG4 Meeting #106                                            R4-2305459

Athens, Greece, Feb 27 – Mar 03, 2023

Source: 	Huawei, HiSilicon

Title: 	On sidelink CA

Agenda Item:	5.31.2.3

Document for:	Approval

Introduction

The WID revision [1] was approved in RAN#99e with the updated scope for the objective of sidelink CA operation in NR sidelink evolution.

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] - Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, power control for simultaneous sidelink TX, packet duplication) - The work is limited to intra-band CA for the ITS band in FR1 (Band n47). - No specific enhancements of Rel-17 sidelink features with sidelink CA support. - This feature is backwards compatible in the following regards - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI) - Only Mode 2 operation - Same subcarrier spacing (SCS) among CA carriers to avoid resource selection enhancements and AGC issues - Time resources for PSFCH are aligned among the carriers for CA - No enhancement related to SCI transmissions on PSCCH/PSSCH, PSFCH transmission, RSRP feedback, CSI feedback and congestion control compared to Rel-16 (i.e., per-carrier operation) - SL resource indication remains to be per-resource pool and per-carrier basis (no cross-carrier scheduling in SCI) - UE transmits SL HARQ feedback on the same carrier on which it receives the associated PSSCH - No consideration for limited transmission and reception capability - No primary/secondary carrier differentiation - Reuse the LTE sidelink CA design for the following aspects: - Sidelink carrier (re-)selection, synchronization of aggregated carriers, Tx power split for simultaneous sidelink transmissions, packet duplication - The CA band combination work in RAN4 is limited to intra-band contiguous CA in Rel-18. - Note: The SL CA work in Rel-18 mainly targets some V2X use cases

In this contribution, we provide our consideration and initial analysis on RAN4 work for NR sidelink CA operation.

Discussion

Sidelink CA operation

In Rel-15 LTE V2X, the requirements of intra-band contiguous multi-carrier operation bandwidth class B and class C were defined in addition to the single carrier V2X case. It is noted that it is a full set of Tx and Rx requirements in order to support the multi-carrier operation. Similarly, the NR sidelink CA operation is to be specified in RAN4, another set of requirements needs to be defined in addition to that of Rel-16/17 V2X/Sidelink. In the updated WID for sidelink CA, only intra-band contiguous CA is to be considered.

The requirements of sidelink CA operation can be included into the suffix of E for V2X, following the convention for LTE V2X operation.

Proposal 1: For sidelink CA operation, a set of requirements needs to be defined in addition to that of Rel-16/17 V2X/Sidelink.

Requirements and methodology for sidelink CA operation

Channel BW

In the previous discussion, for the sidelink on ITS spectrum spectrum, the maximum channel bandwidth is 40MHz.

Table 1 LTE V2X Communication channel bandwidth

| E-UTRA V2X band / V2X channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA V2X Operating Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 47 |  |  |  | Yes |  | Yes |

Table 2 NR V2X operation channel bandwidths for n47

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz |
| n47 | 15 |  | Yes | Yes | Yes | Yes |
|  | 30 |  | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes |

In Rel-15 LTE V2X intra-band contiguous multi-carrier operation, the V2X bandwidth classes and corresponding nominal guard bands are as follows.

Table 3 V2X bandwidth classes and corresponding nominal guard bands [2]

| V2X Bandwidth Class | Aggregated Transmission Bandwidth Configuration | Number of contiguous CC | Nominal Guard Band BWGB |
| --- | --- | --- | --- |
| A | NRB,agg ≤ 100 | 1 | a1 BWChannel(1) - 0.5f1 (NOTE 2) |
| B | 25 < NRB,agg ≤ 100 | 2 | [0.05 max(BWChannel(1),BWChannel(2)) - 0.5f1] |
| C | 100 < NRB,agg ≤ 200 | 2 | [0.05 max(BWChannel(1),BWChannel(2)) - 0.5f1] |
| C1 | 100 < NRB,agg ≤ 200 | 3 | [0.05 max(BWChannel(1),BWChannel(2)) - 0.5f1] |
| D | 200 < NRB,agg ≤ 300 | 3 | NOTE 3 |
| E | 300 < NRB,agg ≤ 400 | 4 | NOTE 3 |
| F | 400 < NRB,agg ≤ 500 | 5 | NOTE 3 |
| I | 700 < NRB,agg ≤ 800 | 8 | NOTE 3 |
| NOTE 1: BWChannel(j), j = 1, 2, 3, 4 is the channel bandwidth of an E-UTRA component carrier according to Table 5.6-1 and f1 = f for the downlink with f the subcarrier spacing while f1 = 0 for the uplink. NOTE 2: a1 = 0.16/1.4 for BWChannel(1) = 1.4 MHz whereas a1 = 0.05 for all other channel bandwidths. NOTE 3: Applicable for later releases. |  |  |  |

For V2X contiguous multi-carrier and CA operation, the V2X Communication channel bandwidths for each operating band is specified in Table 4 except for C1.

Table 4 V2X intra-band multi-carrier and CA configurations [2]

|  |  | V2X multi-carrier and CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X multi-carrier and CA Configuration | V2X multi-carrier and CA Configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| V2X_47B | V2X_47B | 10 | 10 |  |  |  | 20 | 0 |
| V2X_47C | V2X_47C | 10 | 20 |  |  |  | 30 | 0 |
|  |  | 20 | 10 |  |  |  |  |  |
| V2X_47C1 | V2X_47B | 10 | 10 | 10 |  |  | 30 | 0 |

In Rel-18 sidelink CA operation, the bandwidth classes and corresponding nominal guard bands for V2X multicarrier operation can be reused. But the CA configurations for sidelink should be reconsidered due to the larger channel bandwidth in addition to 10MHz and 20MHz in NR.

Table 5 Rel-18 Sidelink CA configuration / Bandwidth combination set

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SL_n47C | SL_n47C | 10 | 40 |  |  |  | 60 | 0 |
|  |  | 20 | 30 |  |  |  |  |  |
|  |  | 20 | 40 |  |  |  |  |  |

The specific channel bandwidths for SL_n47C should be considered according to operators’ demand.

Proposal 2: CA configurations for sidelink should be reconsidered due to the larger channel bandwidth in addition to 10MHz and 20MHz in NR.

Requirements for sidelink intra-band CA

Given that Tx and Rx requirements have been introduced for LTE V2X multi-carrier in Rel-15, some requirements can be reused in Rel-18 sidelink CA, but some of them are different due to NR design. For SL CA operation, requirements for LTE V2X multi-carrier operation should be referred. The group can focus on MPR/A-MPR for Tx and REFSENS for Rx firstly, and further consider other requirements after thorough comparison of difference between LTE V2X and NR SL.

Table 6 UE Tx and Rx RF requirement for Rel-15 V2X multi-carrier operation

Transmitter characteristics

Maximum output power

Maximum Power Reduction (MPR)

Spectrum emission mask

ACLR requirements

Spurious emission

Transmit intermodulation

Receiver characteristics

REFSENS

Maximum input level

Adjacent Channel Selectivity (ACS)

In-band blocking

Out-of-band blocking

Spurious response

Intermodulation characteristics

RX spurious emissions

Receiver image

Proposal 3: The MPR for sidelink CA operation should be studied first.

Methodology for the MPR of sidelink intra-band CA

The MPR simulation assumption for sidelink CA operation is illustrated in Table 6

Table 6 MPR simulation assumption for sidelink CA operation

| parameter | Assumption |
| --- | --- |
| center frequency | 5.9GHz |
| Bandwidth | 60MHz |
| Maximum output power | 23dBm |
| numerology | 15 kHz/30kHz/60kHz |
| Modulation | QPSK/16QAM/64QAM |
| Waveform | CP-OFDM/DFT-s-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc |
| PA calibration | PA calibrated to deliver -30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

Conclusion

This contribution provides our consideration and initial analysis on RAN4 work for Rel-18 SL evolution with the following observation and proposals:

Proposal 1: For sidelink CA operation, a set of requirements needs to be defined in addition to that of Rel-16/17 V2X/Sidelink.

Proposal 2: CA configurations for sidelink should be reconsidered due to the larger channel bandwidth in addition to 10MHz and 20MHz in NR.

Proposal 3: The MPR for sidelink CA operation should be studied first.

- References

[1] RP-230077 WID revision: NR sidelink evolution, OPPO, TSG RAN Meeting #99-e, March. 20-23, 2023

[2] TR36.788 v15.0.0 Vehicle-to-Everything (V2X) Phase 2; User Equipment (UE) radio transmission and reception.