3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2305466

# Online, April 17-April 26, 2023

Title: 	Simulation results on 4TX BS demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.4.5.2

Document for:	Discussion

Background

In this paper, we will provide our simulation results for 4Tx BS demodulation requirements.

Simulation results

The simulation assumptions are captured in Table 2-1:

Table 2-1: Simulation assumptions for 4Tx PUSCH requirements

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
|  | DM-RS port | {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0, FFS on 4 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

The simulation results are captured in Table 2-2:

Table 2-2: Summary of simulation results for 4Tx PUSCH requirements

| SCS | Bandwidth | Propagation conditions | MCS | Antenna | Target SNR(dB) |
| --- | --- | --- | --- | --- | --- |
| 15kHz | 5MHz | TDLA30-10 | 16 | 4T4R | 16.8 |
|  |  |  |  | 4T8R | 8.2 |
|  |  |  | 17 | 4T4R | 17.0 |
|  |  |  |  | 4T8R | 9.0 |
|  |  |  | 20 | 4T4R | 20.6 |
|  |  |  |  | 4T8R | 11.5 |
|  |  | TDLC300-100 | 12 | 4T4R | 12.4 |
|  |  |  |  | 4T8R | 5.1 |
|  |  |  | 16 | 4T4R | 19.9 |
|  |  |  |  | 4T8R | 9.8 |
|  | 20MHz | TDLA30-10 | 16 | 4T4R | 17.6 |
|  |  |  |  | 4T8R | 8.6 |
|  |  |  | 17 | 4T4R | 17.7 |
|  |  |  |  | 4T8R | 9.0 |
|  |  |  | 20 | 4T4R | 23.3 |
|  |  |  |  | 4T8R | 12.2 |
|  |  | TDLC300-100 | 12 | 4T4R | 12.8 |
|  |  |  |  | 4T8R | 5.2 |
|  |  |  | 16 | 4T4R | 19.9 |
|  |  |  |  | 4T8R | 9.9 |
|  | 50MHz | TDLA30-10 | 16 | 4T4R | 18.6 |
|  |  |  |  | 4T8R | 8.9 |
|  |  |  | 17 | 4T4R | 18.6 |
|  |  |  |  | 4T8R | 9.3 |
|  |  |  | 20 | 4T4R | 25.1 |
|  |  |  |  | 4T8R | 12.5 |
|  |  | TDLC300-100 | 12 | 4T4R | 12.8 |
|  |  |  |  | 4T8R | 5.2 |
|  |  |  | 16 | 4T4R | 20.6 |
|  |  |  |  | 4T8R | 10.5 |

Conclusion

In this paper we provide our simulation results for 4TX BS demodulation requirements.

- 4   Reference

- R4-2302926 WF on FR1 4Tx demodulation requirement.  Huawei, HiSilicon