3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2305467

# Online, April 17-April 26, 2023

Title: 	Discussions on Adaptation Absolute Physical Layer requirements

Source: 	Huawei, HiSilicon

Agenda item:       5.16.2

Document for:	Discussion

Background

A WF [1] was agreed that interested companies can bring simulation results on Adaptation Absolute Physical Layer with and without HARQ retransmission enabled in this meeting. In this contribution, we will provide the corresponding simulation results and views on remaining issues.

Discussions

Precoding for HARQ enabled

In last meeting, a consensus was reached that the retransmission shall have same rank and CQI as initial transmission and follow the UE reported PMI.

Issue 1-2-4: Maximum number of HARQ transmission - Companies are encouraged to bring evaluation results with and without retransmission - Pending on the alignment outcome of further evaluation results with retransmission, if the feasibility concluded RAN4 can update the assumption with retransmission - For retransmission number 4 including initial transmission, RV {0,2,3,1} with same MCS and rank as initial transmission; for precoder following UE reported PMI

The common understanding of “Follow UE reported PMI” is that TE will schedule lasted reported PMI for retransmission which doesn't conform to the codebook design when retransmissions have different rank with lasted reported CSI information. The observation of our simulation results show that this circumstance is much likely to occur especially at middle SNR range. Based on the offline email discussion, most companies agree the following:

- for precoder, follow the latest UE reported PMI whose rank is same as the initial transmission.

We agree with this solution if HARQ enabled test is eventually selected for requirements definition.

Proposal 1: TE shall follow the latest UE reported PMI whose rank is same as the initial transmission if HARQ enabled is eventually agreed

Applicability and release independent

- Option 1: The requirement with link adaptation should be applicable for all NR UEs without any new applicability rules, and the requirement should be release independent from Rel-15 - Option 2: The requirement with link adaptation should be applicable from Rel-18 and not release independent from Rel-15 considering that companies are providing the latest results.

Some companies insist on defining ATP requirements release independent from Rel-15. Taking into account that ATP requirements is introduced in Rel-18, some algorithm and implementation may be improved compared to Rel-15 low capability UE. So it is better and safer to define this requirements release independent from Rel-18. Meanwhile no applicability rules is introduced.

Proposal 2: Define ATP requirements release independent from Rel-18

Feasibility of requirements definition

As pointed out in last meeting, it may be difficult to align the simulation results with HARQ enabled due to potential implementation difference of CQI reporting compared to HARQ disabled. The typical upper bound of span is 2.5dB which can be reused for HARQ enabled alignment. I.e. If the span of SNR with no rank transitions at %T of max TP is larger than 2.5dB for HARQ enabled, don’t define ATP requirements with HARQ enabled and only define requirements for HARQ disabled.

Proposal 3: Use 2.5dB as upper bound of span to check the feasibility of HARQ enabled requirements definition

Test point T(%) selection

The exact test point should be selected based on the updated simulation results in this meeting, the general procedure from our side is that it is enough to test only 1 SNR point corresponding to Rank 2 and far from rank transition.

Proposal 4: Only test 1 SNR point corresponding to Rank2 and far from rank transition.

Simulation results

Simulation assumptions are captured in Table 2-1:

Table 2-1: Simulation assumptions Table

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U | DDSU S:11D+3G+0U |
| SNR |  | dB | 0:2:20 | 0:2:20 | 0:2:16 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 | 2 |
|  | Length (L) |  | 12 | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 | 2 |
|  | PRB bundling type |  | Static | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 | As specified in Table A.4-1 of TS 38.101-4: Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) | Row 5, (4) | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 | 13 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) | Row 3 (6) | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 2 | Table 2 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 | 9 | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  | CodebookSubsetRestriction |  | Not configured | Not configured | Not configured |
|  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 | 5.5 | 1.375 |
| Maximum number of HARQ transmission |  |  | 1 and 4 | 1 and 4 | 1 and 4 |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

FR1 FDD

In this section we provide our simulation results for FR1 FDD.

Table 3-1: FR1 FDD 2Rx results (Throughput, CQI, RI, BLER statistics)

|  | HARQ enabled |  |  |  | HARQ Disabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | Throughput | CQI | RI | BLER | Throughput | CQI | RI | BLER |
| 0 | 3.80 | 3 | 1 | 0.06 | 3.67 | 3 | 1 | 0.07 |
| 2 | 5.57 | 3 | 1 | 0.06 | 5.17 | 3 | 1 | 0.09 |
| 4 | 7.46 | 4 | 1 | 0.07 | 7.16 | 4 | 1 | 0.07 |
| 6 | 9.77 | 5 | 1 | 0.05 | 9.55 | 5 | 1 | 0.06 |
| 8 | 12.25 | 6 | 1 | 0.05 | 11.80 | 6 | 1 | 0.07 |
| 10 | 14.88 | 7 | 1 | 0.06 | 14.61 | 7 | 1 | 0.06 |
| 12 | 17.83 | 7 | 1 | 0.06 | 17.71 | 7 | 1 | 0.05 |
| 14 | 20.77 | 8 | 2 | 0.07 | 20.68 | 8 | 2 | 0.06 |
| 16 | 24.07 | 8 | 2 | 0.07 | 23.13 | 8 | 2 | 0.09 |
| 18 | 27.21 | 9 | 2 | 0.08 | 25.83 | 9 | 2 | 0.11 |
| 20 | 29.87 | 9 | 2 | 0.12 | 28.13 | 9 | 2 | 0.15 |

Table 3-2: FR1 FDD 4Rx results (Throughput, CQI, RI, BLER statistics)

|  | HARQ enabled |  |  |  | HARQ Disabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | Throughput | CQI | RI | BLER | Throughput | CQI | RI | BLER |
| 0 | 6.09 | 4 | 1 | 0.17 | 5.27 | 4 | 1 | 0.22 |
| 2 | 8.42 | 4 | 2 | 0.08 | 7.73 | 3 | 2 | 0.13 |
| 4 | 10.76 | 4 | 2 | 0.09 | 10.21 | 4 | 2 | 0.10 |
| 6 | 15.01 | 5 | 2 | 0.07 | 14.72 | 5 | 2 | 0.07 |
| 8 | 19.32 | 6 | 2 | 0.05 | 18.98 | 6 | 2 | 0.06 |
| 10 | 23.21 | 6 | 2 | 0.02 | 23.07 | 6 | 2 | 0.03 |
| 12 | 27.15 | 8 | 2 | 0.06 | 26.78 | 8 | 2 | 0.06 |
| 14 | 31.31 | 9 | 2 | 0.08 | 30.18 | 9 | 2 | 0.09 |
| 16 | 36.30 | 9 | 2 | 0.06 | 35.20 | 9 | 2 | 0.08 |
| 18 | 40.93 | 10 | 2 | 0.04 | 40.24 | 10 | 2 | 0.06 |
| 20 | 43.90 | 11 | 2 | 0.07 | 42.81 | 11 | 2 | 0.08 |

FR1 TDD

In this section we provide our simulation results for FR1 FDD.

Table 3-3: FR1 TDD 2Rx results (Throughput, CQI, RI, BLER statistics)

|  | HARQ enabled |  |  |  | HARQ Disabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | Throughput | CQI | RI | BLER | Throughput | CQI | RI | BLER |
| 0 | 11.16 | 3 | 1 | 0.03 | 11.00 | 3 | 1 | 0.02 |
| 2 | 14.80 | 3 | 1 | 0.04 | 14.16 | 3 | 1 | 0.06 |
| 4 | 20.98 | 4 | 1 | 0.06 | 20.65 | 4 | 1 | 0.06 |
| 6 | 27.80 | 5 | 1 | 0.06 | 27.33 | 5 | 1 | 0.06 |
| 8 | 34.80 | 6 | 1 | 0.04 | 34.29 | 6 | 1 | 0.04 |
| 10 | 43.51 | 7 | 1 | 0.05 | 42.95 | 7 | 1 | 0.04 |
| 12 | 51.50 | 7 | 1 | 0.06 | 50.17 | 7 | 1 | 0.08 |
| 14 | 60.98 | 7 | 2 | 0.05 | 60.07 | 7 | 2 | 0.05 |
| 16 | 70.02 | 8 | 2 | 0.07 | 68.42 | 8 | 2 | 0.07 |
| 18 | 78.67 | 8 | 2 | 0.10 | 74.32 | 8 | 2 | 0.12 |
| 20 | 88.24 | 9 | 2 | 0.12 | 83.12 | 9 | 2 | 0.14 |

Table 3-4: FR1 TDD 4Rx results (Throughput, CQI, RI, BLER statistics)

|  | HARQ enabled |  |  |  | HARQ Disabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | Throughput | CQI | RI | BLER | Throughput | CQI | RI | BLER |
| 0 | 18.10 | 3 | 1 | 0.16 | 15.67 | 3 | 2 | 0.20 |
| 2 | 24.76 | 3 | 2 | 0.12 | 21.79 | 3 | 2 | 0.18 |
| 4 | 31.75 | 4 | 2 | 0.08 | 30.27 | 4 | 2 | 0.09 |
| 6 | 46.78 | 5 | 2 | 0.05 | 45.63 | 5 | 2 | 0.06 |
| 8 | 59.37 | 6 | 2 | 0.05 | 58.58 | 5 | 2 | 0.05 |
| 10 | 70.71 | 6 | 2 | 0.04 | 69.98 | 6 | 2 | 0.04 |
| 12 | 85.38 | 8 | 2 | 0.04 | 83.29 | 8 | 2 | 0.05 |
| 14 | 98.78 | 9 | 2 | 0.06 | 96.05 | 9 | 2 | 0.08 |
| 16 | 107.81 | 9 | 2 | 0.09 | 101.22 | 9 | 2 | 0.13 |
| 18 | 121.20 | 10 | 2 | 0.08 | 116.75 | 10 | 2 | 0.10 |
| 20 | 131.91 | 11 | 2 | 0.10 | 124.42 | 11 | 2 | 0.14 |

FR2 TDD

In this section we provide our simulation results for FR2 TDD.

Table 3-5: FR1 TDD 2Rx results (Throughput, CQI, RI, BLER statistics)

|  | HARQ enabled |  |  |  | HARQ Disabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | Throughput | CQI | RI | BLER | Throughput | CQI | RI | BLER |
| 0 | 19.40 | 5 | 1 | 0.10 | 18.35 | 5 | 1 | 0.11 |
| 2 | 25.49 | 6 | 1 | 0.09 | 23.60 | 6 | 1 | 0.13 |
| 4 | 33.11 | 7 | 1 | 0.12 | 30.50 | 7 | 1 | 0.14 |
| 6 | 42.49 | 8 | 1 | 0.12 | 39.76 | 8 | 1 | 0.14 |
| 8 | 51.81 | 9 | 1 | 0.10 | 48.73 | 9 | 1 | 0.13 |
| 10 | 62.63 | 10 | 1 | 0.13 | 57.81 | 10 | 1 | 0.16 |
| 12 | 75.74 | 10 | 1 | 0.12 | 72.00 | 10 | 1 | 0.14 |
| 14 | 89.49 | 9 | 2 | 0.09 | 85.71 | 9 | 2 | 0.10 |
| 16 | 99.70 | 9 | 2 | 0.14 | 93.22 | 9 | 2 | 0.15 |

Conclusion

In this paper we provide our updated link level simulation results for ATP requirements and discussions on remaining issues. The proposals are:

Proposal 1: TE shall follow the latest UE reported PMI whose rank is same as the initial transmission if HARQ enabled is eventually agreed

Proposal 2: Define ATP requirements release independent from Rel-18

Proposal 3: Use 2.5dB as upper bound of span to check the feasibility of HARQ enabled requirements definition

Proposal 4: Only test 1 SNR point corresponding to Rank2 and far from rank transition.

Reference

[1]   R4-2302930  Way forward on physical layer throughput requirements. Intel.