3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2305474

# Online, 17th - 26th Apr, 2023

Title: 	Discussion on general aspects for NR ATG demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.14.5.1

Document for:	Discussion

Background

During RAN4#106 meeting, WF [1] for ATG demodulation requirements was approved. In this contribution, we share our views about general aspects for NR ATG demodulation requirements.

Discussion

Scenario

- Option 1: Consider following deployment scenario for Rel-18 ATG demodulation discussion: - One ATG UE only served by one TRP from ATG BS at the same time. - ATG BS is 30m high and its beam direction is fixed and pointing to the horizon. The vertical HPBW is α and its lower boundary is parallel to the horizon. - ATG UE moving toward to ATG BS with the elevation angle from 0o to α at the speed 1200km/h. The corresponding maximum distance between UE and BS could be 200km ~300km. - Option 2: For the perspective of demodulation, it is necessary to consider the both scenarios. - Figure 2-1(a). Scenario of ATG of TN deployment - Figure 2-1(b). Scenario of ATG of ATG deployment - From ATG demodulation requirements perspective, at least scenario in option 1 shall be considered - FFS whether the deployment scenario Figure 2-1(a). from option 2 can be considered or not - The detailed parameters need to be further discussed

In last meeting, there is discussion about the deployment scenario, related to uni-directional/bi-directional and other deployment parameters such as elevation angle, ATG BS altitude, maximum UE speed and maximum ISD. From our understanding, it is not clear for the impact of these parameters on ATG demodulation requirement. There is no any limitation for the maximum “Doppler/timing jump” during the handover from the original TRP to the target TRP since PLL should firstly give up tracking the original TRP and then re-track the target TRP. Moreover, since UE DL frequency and timing compensation is baseline assumption, only residual Doppler and timing error should be considered that is fixed value for all time, it is no need to define complex channel model.

Parameters such as elevation angle, ATG BS altitude, maximum UE speed and maximum ISD have no impact on ATG demodulation requirement.

Do not consider parameters such as elevation angle, ATG BS altitude, maximum UE speed and maximum ISD for the channel model for ATG demodulation evaluation and requirements.

Channel model

- Candidate channel models for ATG deployment: - Option 1: Single path AWGN channel - Option 2: NTN-TDL-C (FFS for the delay spread and K factor)

For the channel model, LoS condition is agreed in last meeting. The remained issue is that whether to use AWGN channel or NTN TDL channel. From our understanding, both options are possible and it depends on the assumption of the environment, i.e. cloudless or cloudy. To make the UE performance more robust, we propose to consider NTN-TDL-C channel model for ATG performance requirements. For the delay spread and K factor, although it is different comparing to NTN, we prefer to use the same value as NTN, i.e. NTN-TDLC5-200 to reduce the test effort and the simulation effort, considering that same receiver algorithm is expected for NTN and ATG.

Consider NTN-TDL-C channel model for ATG performance requirements. Use the same value as NTN for the delay spread and K factor, i.e. NTN-TDLC5-200.

Proposals

In this contribution, we discuss general aspects for NR ATG demodulation requirements. Our observations and proposals are:

- Parameters such as elevation angle, ATG BS altitude, maximum UE speed and maximum ISD have no impact on ATG demodulation requirement.

- Do not consider parameters such as elevation angle, ATG BS altitude, maximum UE speed and maximum ISD for the channel model for ATG demodulation evaluation and requirements.

- Consider NTN-TDL-C channel model for ATG performance requirements. Use the same value as NTN for the delay spread and K factor, i.e. NTN-TDLC5-200.

Reference

R4-2302996, WF for ATG demodulation requirements, RAN4#106, CMCC