3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2305487

# Online, 17th - 26th Apr, 2023

Title: 	Discussion on demodulation performance requirements for NR network-controlled repeaters

Source: 	Huawei, HiSilicon

Agenda item:	5.29.6

Document for:	Discussion

Background

During RAN#98 meeting, Revised WID [1] on NR network-controlled repeaters was approved and there is NCR-MT demodulation requirements to be defined.

Study and define the RF conformance testing requirements and EMC requirements of NCR if necessary [RAN4] Study and define the RRM performance requirements of NCR-MT if necessary [RAN4] Define the demodulation performance requirements for C-link of NCR-MT [RAN4] Note-1: The existing requirements defined in RAN4 can be reused if applicable. Note-2: The demodulation requirements for NCR-MT are expected to be based on a subset of the UE demodulation requirements in 38.101-4 with some adaptations for e.g. conformance testing procedure and FRCs that are similar to IAB-MT.

In this contribution, we share our views about demodulation performance requirements for NR network-controlled repeaters.

Discussion

Test scope

Considering that Rel-15 is the first version of NR and provides baseline features, in our view, NCR-MT performance requirements should be defined based on Rel-15 UE performance requirements defined in TS 38.101-4.

Define NCR-MT performance requirements based on Rel-15 UE performance requirements.

It is agreed that NCR-MT demodulation requirements is a subset of the UE demodulation requirements in 38.101-4. Here we list all demodulation requirements defined in TS 38.101-4 and give our preference for further selection, as shown in Table 2.1-1. The mainly consideration is to reduce the scenario that is not typical for NCR-MT.

Table 2.1-1 Further selection from TS 38.101-4 for NCR-MT demodulation requirements

| Physical channel | Requirements | Description | View |
| --- | --- | --- | --- |
| PDSCH | PDSCH Mapping Type A | QPSK/16QAM/256QAM TDL-A/B/C, HST 2x2, 2x4, 4x4 rank 1/2/3/4 Enhanced Receiver Type 1 | Do not consider HST scenario. Do not consider 256QAM. Other cases can be reused. |
|  | PDSCH Mapping Type A and CSI-RS overlapped with PDSCH | 16QAM TDL-C 2x2, 2x4 rank 2 | Do not consider CSI-RS overlapped with PDSCH cases. |
|  | PDSCH Mapping Type B | QPSK TDL-A 2x2, 2x4 rank 1 | Do not consider PDSCH Mapping Type B cases. |
|  | PDSCH Mapping Type A and LTE-NR coexistence | QPSK TDL-A 4x2, 4x4 rank 1 | Do not consider LTE-NR coexistence scenario. |
| PDCCH | 1/2 Tx | AL 2/4/8/16 TDL-A/C 2/4 Rx | Requirements can be reused. |
| PBCH | SSB index known/unknown | TDLC 1x2 | Do not consider PBCH cases. |
| SDR | Single carrier/CA | Based on UE capability | Do not consider SDR cases. |
| CSI | CQI | AWGN/Fading 2x2, 2x4 Wideband/Subband P/AP CSI-RS configuration P/AP CSI reporting configuration | Requirements can be reused. |
|  | PMI | 4Tx/8Tx 2Rx/4Rx Wideband/Subband P/AP CSI-RS configuration P/AP CSI reporting configuration | Requirements can be reused. |
|  | RI | 2x2, 2x4 Wideband P/AP CSI-RS configuration P/AP CSI reporting configuration | Requirements can be reused. |

Further selection from TS 38.101-4 for NCR-MT demodulation requirements based on Table 2.1-1.

Proposals

In this contribution, we discuss on demodulation performance requirements for NR network-controlled repeaters. Our observations and proposals are:

- Define NCR-MT performance requirements based on Rel-15 UE performance requirements.

Further selection from TS 38.101-4 for NCR-MT demodulation requirements based on Table 2.1-1.

Reference

RP-223505, Revised WID on NR network-controlled repeaters, RAN#98, ZTE, Sanechips