3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305508

Online, April 17 – April 26, 2023

Agenda Item:	5.5.4.1.1

Source:	Ericsson

Title:	Remaining issues on PDSCH requirements for 8 Rx in FR1

Document for:	Discussion

# 1	Introduction

Last meeting #106, some progress has been achieved in this WI. In this meeting, we go through the remaining issues that still need to be addressed as stated in [1]. Below, we summarize the remaining open issues on the general aspects and PDSCH requirements’ definition.

General aspects Issue 1-4: Whether to define PDCCH requirements - FFS whether to consider PDCCH requirements Remaining issues on PDSCH Issue 2-6: Two MCS configuration for Rank>4 - Option 1: Configure two MCSs for Rank>4 and discuss the spatial channel model to be used to guarantee different precoding gain for each layer - Option 2: Only consider single MCS Issue 2-8: Mapping Type - PDSCH mapping Type A - FFS PDSCH mapping Type B Issue 2-11: Whether consider scenarios with configuration of NZP CSI-RS overlapping with PDSCH for CSI-RS ports larger than 4Tx - Option 1: Yes, also define ‘Minimum requirements for PDSCH Mapping Type A and CSI-RS overlapped with PDSCH’ for 8Rx UE similar as 5.2.2.1.2, 5.2.3.1.2 in 38.101-4. - Option 2: No

# 2	General aspects for UE demodulation and CSI reporting requirements

A good progress has been achieved on the general aspects to be considered. However, it remained an open question, whether to define requirements for PDCCH. After examining this issue thoroughly through simulation, we noted that the signal-to-noise (SNR) levels are low, even when considering TDLC300-100 channels, as compared to the PDSCH requirements. Consequently, we do believe that it is not needed to define PDCCH requirements.

Proposal 1: Do not define PDCCH requirements for 8 RX UE in FR1.

# 3	PDSCH remaining issues

The parameters specified in TS38.101-4 Table 5.2-1 [2] are valid for all PDSCH tests unless otherwise stated. However, to save efforts, it is preferrable to limit our requirements to PDSCH Type A.

Proposal 2: Define the 8 Rx UE demodulation requirements for PDSCH Type A only.

In this WI, we are supposed to provide the minimum requirements for UE equipped with 8 Rx and support up to 8 layers. However, by considering different MCS for each codeword, the overall performance will be limited by the lowest MCS. We do believe that both codewords should carry same MCS level.

Proposal 3: For Rank > 4, consider same MCS at each codeword.

We summarize the parameters that have been agreed in [1] to carry out the simulation and help in defining the PDSCH requirements.

Proposal 4: Define PDSCH demodulation requirements for UE at 70% of the peak throughput using the following simulation parameters.

| Assumptions | Value |
| --- | --- |
| Duplex mode | TDD |
| TDD UL-DL pattern | 7D1S2U with S=6D:4G:4U |
| Carrier Frequency [GHz] | 3.5 GHz |
| Subcarrier Spacing [KHz] | 30 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) and TDLC (300 ns delay spread) |
| Antenna configuration | 2x8 ULA Medium B, 4x8 and 8x8 ULA Low |
| Velocity | 3 km/h (Doppler spread = 10 Hz), 30 km/h (Doppler spread = 100 Hz) |
| PA Model | None |
| Pre-loaded Tx EVM | 6% (QPSK/16QAM/64QAM) and 3% (256QAM) |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 2, Rank 4 and Rank 8 |
| DMRS Configuration | 2+2 Type 1 DMRS symbols at (2,3,10,11) symbol indexes for Rank 8 1+1 Type 1 DMRS symbols at (2, 10) symbol indexes for Rank 2 and Rank 4 |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). From MCS Table 2 (TS38.214): Up to MCS 20 (256QAM) |
| NOH | 0 |
| Number of HARQ Processes | 8 |
| Maximum HARQ transmissions | 4 |

| Parameter |  | Value |
| --- | --- | --- |
| SCS (kHz)/Bandwidth (MHz) |  | 30/40 |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 |

According to our simulation results detailed in [3] and based on the parameters above, the propose to define requirements for PDSCH as stated Proposal 4.

Proposal 5: Referring to the Tables in Proposal 4 and based on our simulation results, consider PDSCH test cases at 70% of the peak throughput for 40 MHz/30 kHz SCS as follow

FR1 TDD, 40 MHz/30 kHz, Rank = 8, Metric: 70% Peak Throughput

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format an.d code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 4 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

FR1 TDD, 40 MHz/30 kHz, Rank = 4, Metric: 70% Peak Throughput

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 4 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| 2-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| 2-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.85 MCS 26 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

FR1 TDD, 40 MHz/30 kHz, Rank = 2, Metric: 70% Peak Throughput

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 2 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 7 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.56 MCS 13 | 2 | TDLA30-10 | 2x8, ULA Medium B | 70 | TBD |
| 3-4 | 3.5 | 40 / 30 (106) | 256QAM, 0.89 MCS 26 | 2 | TDLA30-10 | 2x8, ULA Medium B | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM Use Tx EVM 3% for 256QAM. |  |  |  |  |  |  |  |  |

# 3	Specification structure

Table 1	Specification structure for UE Demodulation performance requirements in FR1 (Conducted requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 5.1 | General | Updates on FR1 8Rx |
| 5.2 | PDSCH demodulation requirements |  |
| 5.2.1 | 1RX requirements |  |
| 5.2.2 | 2RX requirements |  |
| 5.2.3 | 4RX requirements |  |
| 5.2.4 | 8RX requirements | New section |
| 5.2.4.1 | FDD | Void |
| 5.2.4.2 | TDD | New section |
| 5.2.4.2.1 | Minimum requirements for PDSCH Mapping Type A | New section |
| 5.5 | Sustained downlink data rate provided by lower layers | Updates on FR1 8 Rx |
| 5.5.1 | FR1 single carrier requirements | Updates on FR1 8 Rx |

Table 2	Specification structure for UE CSI reporting requirements in FR1 (Conducted requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 6.1 | General | Updates on FR1 8 Rx |
| 6.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 6.2.4 | 8RX requirements | New section |
| 6.2.4.1 | FDD | Void |
| 6.2.4.2 | TDD | New section |

# 6	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Proposal 1: Do not define PDCCH requirements for 8 RX UE in FR1.

Proposal 2: Define the 8 Rx UE demodulation requirements for PDSCH Type A only.

Proposal 3: For Rank > 4, consider same MCS at each codeword.

Proposal 4: Define PDSCH demodulation requirements for UE at 70% of the peak throughput using the following simulation parameters.

| Assumptions | Value |
| --- | --- |
| Duplex mode | TDD |
| TDD UL-DL pattern | 7D1S2U with S=6D:4G:4U |
| Carrier Frequency [GHz] | 3.5 GHz |
| Subcarrier Spacing [kHz] | 30 kHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) and TDLC (300 ns delay spread) |
| Antenna configuration | 2x8 ULA Medium B, 4x8 and 8x8 ULA Low |
| Velocity | 3 km/h (Doppler spread = 10 Hz), 30 km/h (Doppler spread = 100 Hz) |
| PA Model | None |
| Pre-loaded Tx EVM | 6% (QPSK/16QAM/64QAM) and 3% (256QAM) |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 2, Rank 4 and Rank 8 |
| DMRS Configuration | 2+2 Type 1 DMRS symbols at (2,3,10,11) symbol indexes for Rank 8 1+1 Type 1 DMRS symbols at (2, 10) symbol indexes for Rank 2 and Rank 4 |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). From MCS Table 2 (TS38.214): Up to MCS 20 (256QAM) |
| NOH | 0 |
| Number of HARQ Processes | 8 |
| Maximum HARQ transmissions | 4 |

| Parameter |  | Value |
| --- | --- | --- |
| SCS (kHz)/Bandwidth (MHz) |  | 30/40 |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 |

Proposal 5: Referring to the Tables in Proposal 4 and based on our simulation results, consider PDSCH test cases at 70% of the peak throughput for 40 MHz/30 kHz SCS as follow

FR1 TDD, 40 MHz/30 kHz, Rank = 8, Metric: 70% Peak Throughput

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format an.d code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 4 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

FR1 TDD, 40 MHz/30 kHz, Rank = 4, Metric: 70% Peak Throughput

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 4 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| 2-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| 2-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.85 MCS 26 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

FR1 TDD, 40 MHz/30 kHz, Rank = 2, Metric: 70% Peak Throughput

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 2 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 7 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.56 MCS 13 | 2 | TDLA30-10 | 2x8, ULA Medium B | 70 | TBD |
| 3-4 | 3.5 | 40 / 30 (106) | 256QAM, 0.89 MCS 26 | 2 | TDLA30-10 | 2x8, ULA Medium B | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM Use Tx EVM 3% for 256QAM. |  |  |  |  |  |  |  |  |

# References

- R4-2302942, “WF for 8Rx UE performance requirements”, Huawei.

- 3GPP TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”, v.17.8.0.

- R4-2305509, “Simulation results for 8 Rx PDSCH requirements in FR1”, Ericsson.

# Appendix	Simulation assumption

## A.1	PDSCH

### FR1

Please refer to TS.38.101-4 Clause 5.2.3.2.1 that will be updated.

Table 5.2.3.2.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Tests 1-1, 1-8, 1-9 wideband for Test 3-1 2 for other tests |
|  | Resource allocation type |  | Test 1-2: Type 1 with start RB = 50, LRBs = 6 Other tests: Type 0 |
|  | RBG size |  | Test 1-2: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Tests 1-1, 1-7, 1-8, 1-9, 1-10, 1-11 1 for other tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | Tests 1-8, 1-9: l0 = 4 for CSI-RS resource 1 and 3 l0 = 8 for CSI-RS resource 2 and 4 Other tests; Table 5.2-1. |
|  | CSI-RS periodicity | Slots | Test 1-7, 1-10, 1-11: 20 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  | CSI-RS offset | Slots | Test 1-7, 1-10, 1-11: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
|  | Frequency Occupation |  | Test 1-7, 1-10, 1-11: Start PRB 0 Number of PRB = 52 Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  | 16 for Test 1-4 10 for Test 1-9 8 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |