3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305513

Online, April 17 – April 26, 2023

Agenda Item:	5.5.4.1.3

Source:	Ericsson

Title:	Simulation results for CQI reporting for 8Rx in FR1

Document for:	Discussion

# 1	Introduction

In this paper, we present the simulations results for CQI reporting for 8Rx UE in FR1. Based on last meeting discussions [1], we carried out simulations for CQI reporting considering the agreed parameters.

# 2	CQI Reporting Simulation Results

## 2.1 Simulation setup

As stated in WF [1], we will consider the following table throughout the simulations

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Andwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 with static channel specified in updated Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,0 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-i1/CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | CodebookSubsetRestriction |  | The codebook is restricted to i1,1=0, i1,2=0,i1.3=0 and i2=0 |  |  |  |
|  |  | RI Restriction |  | 00001000 |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |

It is worth reminding Table A.4-3 in TS 38.101-4 given by

| TBS Scheme |  |  |  | TBS.3-1 | TBS.3-2 | TBS.3-3 | TBS.3-4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 256QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 52 | 106 | 106 |  |  |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 |  |  |
| Number of PDSCH MIMO layers |  |  |  | 3 | 4 | 3 | 4 |  |  |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 | 24 | 24 |  |  |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 |  |  |
| Available RE-s for PDSCH |  |  |  | 6240 | 6240 | 12720 | 12720 |  |  |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A |  |  |
| 1 | 0.2344 | 0 | QPSK | 4360 | 5896 | 8976 | 11784 |  |  |
| 2 | 0.3770 | 1 |  | 7048 | 9480 | 14344 | 18976 |  |  |
| 3 | 0.8770 | 3 |  | 16392 | 22032 | 33816 | 45096 |  |  |
| 4 | 1.4766 | 5 | 16QAM | 27656 | 36896 | 56368 | 75792 |  |  |
| 5 | 1.9141 | 7 |  | 35856 | 48168 | 73776 | 98376 |  |  |
| 6 | 2.4063 | 9 |  | 45096 | 60456 | 92200 | 122976 |  |  |
| 7 | 2.7305 | 11 | 64QAM | 51216 | 67584 | 104496 | 139376 |  |  |
| 8 | 3.3223 | 13 |  | 62504 | 81976 | 127080 | 167976 |  |  |
| 9 | 3.9023 | 15 |  | 73776 | 98376 | 147576 | 196776 |  |  |
| 10 | 4.5234 | 17 |  | 83976 | 112648 | 172176 | 229576 |  |  |
| 11 | 5.1152 | 19 |  | 96264 | 127080 | 196776 | 262376 |  |  |
| 12 | 5.5547 | 21 | 256QAM | 104496 | 139376 | 213176 | 278776 |  |  |
| 13 | 6.2266 | 23 |  | 116792 | 155776 | 237776 | 319784 |  |  |
| 14 | 6.9141 | 25 |  | 129128 | 172176 | 262376 | 352440 |  |  |
| 15 | 7.4063 | 27 |  | 139376 | 184424 | 278776 | 376896 |  |  |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is not scheduled on slots containing CSI-RS for tracking, CSI-RS for CSI acquisition and CSI-RS for beam refinement or slots which are not full DL Note 3: PDSCH is not scheduled on slots containing PBCH, i.e. slot#0 per 20ms periodicity Note 4: Spectral efficiency is based on MCS Table defined in Table 5.1.3.1-2 of TS 38.214 |  |  |  |  |  |  |  |  |  |

## 2.2 Simulation results

| SNR | Median CQI | Pr(Median CQI – 1 < CQI < Median CQI+1) [%] | BLER of Median CQI-1 [%] | BLER of Median CQI [%] | BLER of Median CQI+1 [%] |
| --- | --- | --- | --- | --- | --- |
| -6 | 2 | 100 | 0 | 63 | 100 |
| -5 | 2 | 100 | 0 | 0.1 | 100 |
| -4 | 2 | 100 | 0 | 0 | 100 |
| -3 | 3 | 100 | 0 | 100 | 100 |
| -2 | 3 | 100 | 0 | 33.2 | 100 |
| -1 | 3 | 100 | 0 | 0 | 100 |
| 0 | 3 | 100 | 0 | 0 | 100 |
| 1 | 4 | 100 | 0 | 92.6 | 100 |
| 2 | 4 | 100 | 0 | 8.6 | 100 |
| 3 | 5 | 100 | 0 | 86.8 | 100 |
| 4 | 5 | 100 | 0 | 0 | 100 |
| 5 | 6 | 100 | 0 | 86.1 | 100 |
| 6 | 6 | 100 | 0 | 0 | 100 |
| 7 | 7 | 100 | 0 | 77.9 | 100 |
| 8 | 7 | 100 | 0 | 0 | 100 |
| 9 | 8 | 100 | 0 | 19.9 | 100 |
| 10 | 9 | 100 | 0.1 | 100 | 100 |
| 11 | 9 | 100 | 0 | 0.6 | 100 |
| 12 | 10 | 100 | 0 | 100 | 100 |
| 13 | 10 | 100 | 0 | 11.7 | 100 |
| 14 | 11 | 100 | 0 | 100 | 100 |
| 15 | 11 | 100 | 0 | 86.2 | 100 |
| 16 | 12 | 100 | 0.1 | 100 | 100 |
| 17 | 12 | 100 | 0 | 1 | 100 |
| 18 | 13 | 100 | 0 | 100 | 100 |
| 19 | 13 | 100 | 0 | 15.3 | 100 |
| 20 | 14 | 100 | 0.6 | 100 | 100 |
| 21 | 14 | 100 | 0.1 | 21.8 | 100 |
| 22 | 14 | 100 | 0 | 0 | 100 |
| 23 | 15 | 100 | 0 | 17.6 | NaN |
| 24 | 15 | 100 | 0 | 0.1 | NaN |
| 25 | 15 | 100 | 0 | 0 | NaN |

Fig.1 - Simulation results depicting the median CQI as function of SNR for 8 Rx UE using 30 kHz/40 MHz and Rank 4 in FR1.

Observation 1: The results show that the lowest SNR to report efficiently a CQI with Rank 1 related to

- 256QAM is at SNR = 16/17 dB (CQI 12).

- 64QAM is at SNR = 7/8 dB (CQI 7).

- 16QAM is at SNR = 1/2 dB (CQI 4).

# 6	Summary

In this paper, we provided some initial simulation results that should help in the definition of the PDSCH requirements. In the following, some useful observations that can be further discussed.

Observation 1: The results show that the lowest SNR to report efficiently a CQI with Rank 4 related to

- 256QAM is at SNR = 16/17 dB (CQI 12).

- 64QAM is at SNR = 7/8 dB (CQI 7).

- 16QAM is at SNR = 1/2 dB (CQI 4).

# References

- R4-2302942, “WF on 8RX UE performance requirements”, Huawei.