3GPP TSG-RAN WG4 Meeting #106bis-e              	R4-2305539

Online meeting, 17th April-26th April, 2023

Agenda item:	5.13.5.2

Source:	Samsung

Title:	View on UE demodulation requirements for Rel-18 FR2 HST

Document for:	Discussion

- Introduction

In RAN#97-e meeting, the updated WID on enhanced NR support for high-speed train scenario in frequency range 2 (FR2) was approved with the following objective as [1]

- Core requirement - Only train roof-mounted high-power devices with target applicable carrier frequency up to 30GHz and up to 350km/h velocity are considered in this WI - Specify the RF requirements for intra-band carrier aggregation (CA) scenario, and investigate and specify the RRM requirements for intra-band carrier aggregation (CA) scenario [RAN4] - Specify the requirement for simultaneous multi-panel operation for train roof-mounted FR2 high power devices [RAN4]: - Maximum 2 active panels supporting the multi-panel simultaneous reception. - NOTE: Focus on FR2 HST specific requirements, and avoid the overlap with the scope of FR2 multi-Rx DL reception - Study on reference tunnel deployment scenario for FR2 HST and specify the channel model and corresponding core requirements if any [RAN4] - Specify UL timing adjustment solution, including explicit NW signaling assistance, for FR2 HST scenario with large UL/DL propagation delay difference from different RRHs/TRPs to UE [RAN4, RAN2]. - Note: RAN1/RAN2 work can be triggered by RAN4 LS. - Performance requirement - Specify the necessary RRM test cases based on the outcome on corresponding core part. - Investigate and if needed specify the demodulation performance requirements for intra-band carrier aggregation (CA) HST scenario - Specify the necessary demodulation performance requirements for simultaneous multi-panel reception. - NOTE: Focus on FR2 HST specific requirements, and avoid the overlap with the scope of FR2 multi-Rx DL reception - Specify the other necessary RRM and demodulation performance requirements depending on the outcome of core part.

In the RAN4#104bis-e meeting, the work plan for performance part including demodulation requirement and RRM requirement has been approved in [2]. According to meeting schedule and work plan, the performance part of demodulation will be discussed from this meeting as

RAN4#106-bis-e: Apr. 17th – 26th, 2023 - RRM (performance) requirement: - Discussion on UE RRM performance requirement impact due to intra-band CA for FR2 PC6; - Discussion on RRM performance requirement for simultaneous multi-panel operation, specifically for FR2 HST, based upon conclusions from FR2 multi-RX reception WI. - Discussion on RRM performance requirement for UL timing adjustment solution. - Demodulation: - Discussion on test case and applicability rule for intra-band CA for FR2 PC6. - Discussion on test case for simultaneous multi-panel reception for FR2 PC6. - Discussion on test case in reference tunnel deployment channel, if identified.

In this contribution, the view on test scope and test setup for UE demodulation is provided.

# 2	Discussion

## 2.1 UE demodulation requirement for CA

In Rel-17 work item on NR support for high-speed train scenario in frequency range 2 (FR2), RAN4 has focused on train roof-mounted high-power devices for NR SA single carrier scenario in FR2.  The intra-band carrier aggregation (CA) operation is also of interests to operators, which is not yet covered in Rel-17 work item scope.

Channel Bandwidth

Based on WID, RAN4 will specify the demodulation requirements for intra-band carrier aggregation for HST scenario. Based on the discussion on FR2 NR CA in Rel-16 WI on NR performance requirement enhancement, PDSCH requirements on single carrier of BW {50, 100, 200, 400} need to be specified.

Proposal 1: to support FR2 HST intra-band CA, PDSCH requirements on single carrier of following bandwidth need to be specified:

- For 120KHz SCS, specify PDSCH requirements on single carrier of BW of {50, 100, 200, 400} MHz

Transmission schemes and Channel Model

In the Rel-17 FR2 HST WI, RAN4 introduced transmission scheme with DPS scheme 1a and DPS scheme 1b for both Uni-direction deployment scenario A and Bi-directional deployment scenario B. Since DPS scheme 1a and DPS scheme 1b are UE feature with different UE capability. Therefore, both two DPS transmission schemes should be considered to support CA scenario.

Proposal 2: Transmission schemes with both DPS scheme 1a and DPS scheme 1b should be considered to support CA, under Uni-directional deployment scenario A and Bi-directional deployment scenario B, separately

- Case 1: Uni-directional scenario A with DPS scheme 1b with {50, 100,200, 400} MHz

- Case 2: Bi-directional scenario B with DPS scheme 1a with {50, 100, 200, 400} MHz

Proposal 3: Apply the Rel-17 FR2 HST channel model parameters to PDSCH CA demodulation requirements with DPS transmission schemes

| Parameter | Value |
| --- | --- |
|  | HST-DPS-FR2-UNI-A |
| Ds | 700 m |
| Ds_offset | 10 m |
| Dmin | 10 m |
| v | 350 km/h |
| fd | 9722 Hz |

| Parameter | Value |
| --- | --- |
|  | HST-DPS-FR2-BI-B |
|  | 700 m |
|  | 150 m |
|  | 350 km/h |
|  | 9722 Hz |

Test parameters

As for test parameters, the adopted in Rel-17 FR2 HST WI can be reused

Proposal 4: Reused the exiting test parameters of FR2 HST with DPS 1a and DPS 1b for CA requirements

## 2.2	UE demodulation requirement for simultaneous multi-panel reception

In the Rel-17 WI, similar as FR2 handheld and other UE types, the single active panel operation was focused, i.e. only one active antenna panel at a time as baseline antenna assumption. Based on the Rel-17 conclusion, two panels shall be physically installed to flexibly support either forward or backward incoming signal direction; however, the restriction of only one active antenna panel limits utilizing two neighbouring RRHs to serve one HST UE in the bi-directional RRH deployment scenario. Accordingly, it is of importance to introduce the support of simultaneous reception with maximum 2 active panels at the train roof-mounted FR2 high power devices for the bi-directional RRH deployment scenario.

In Rel-16 eMIMO WI, the DL transmission scheme with transmission scheme with simultaneous multi-beam reception from multi-TRP/panel was introduced in RAN1. FR2 UE capability for simultaneous multi-beam reception was introduced (simultaneousReceptionDiffTypeD-r16).

Deployment scenario

In the previous meeting, the deployment scenario for supporting simultaneous multi-panel reception is discussed for both Bi-directional and Uni-directional deployment scenarios respectively, where bi-directional RRH deployment scenario as indicated in figure 1, the feasibility has been confirmed

figure 1:  Bi-directional scenario

For uni-directional RRH deployment scenario, the feasibility is still open issue. Uni-directional scenario A is agreed not feasible, and uni-directional scenario B need further feasibility study in in terms of new deployment assumption and new UE panel assumption. Therefore, we can prioritize the PDSCH requirement with simultaneous multi-panel reception under Bi-directional deployment scenario firstly.

For Bi-directional deployment scenario, both scenario A and scenario B are considered to identify RRM requirement. From baseband processing perspective, there is no difference foreseen, only different is the doppler experienced by UE. Therefore, we propose to introduce PDSCH requirement with one scenario if both two scenarios are feasible from core requirement perspective. Considering scenario, A has small value of Dmin, which will result in the large doppler value experienced compared with scenario B. Therefore, scenario A can be regarded as starting point.

Proposal 5: RAN4 introduces PDSCH requirement with simultaneous multi-panel reception under Bi-directional deployment scenario firstly.  Scenario A can be regarded as starting point. FFS on introduce PDSCH requirement with simultaneous multi-panel reception under Uni-directional scenario pending on the core requirement discussion.

Transmission schemes

Based on the RRM discussion, the following agreement is made for transmission scheme [2]

- Transmission Scheme for FR2 HST multi- - RAN4 to focus on NC JT as the main transmission scheme in Rel-18 HST FR2 enhanced deployments

According to agreed eMIMO UE feature in RAN1, there are single-DCI and multi-DCI transmission based on multi-TRP/Panel transmission as described in Figure 2

Figure 2:  Single -DCI and Multi-DCI based transmission scheme

For multi-DCI-based NCJT transmission, up to 4 transmission layers per PDSCH, a UE may expect to receive two PDCCHs scheduling two fully/partially/non-overlapped PDSCHs respectively in time and frequency domain with same/different PDSCH scrambling ID(s)

For single-DCI-based NCJT transmission, the following transmission schemes can be supported

Figure 3:  Single -DCI based transmission schemes

From UE receiver processing aspect, single-DCI and multi-DCI are two different transmission schemes including the difference among PDCCH scheduling, TCI state configuration and DMRS port mapping etc. Therefore, demodulation requirements should be defined for both single-DCI and multi-DCI schemes.

In the last meeting for multi-panel Rx WI [3], both Single-DCI with SDM and FDM scheme A, and multi-DCI with fully over lapping and non-overlapping are selected as initial simulation assumption, which can be used as starting point for FR2 HST with multi-panel reception.

Different with multi-panel Rx WI, for HST scenario, due to the movement of train, there is power imbalance between the signal received by two panel, especially for the train moving into the location below of severed RRH.  Therefore, the feasible of 2+2 reception with considering power imbalance should be further studied.

Proposal 6: Both Single-DCI and multi-DCI transmission schemes should be introduced in Rel-18 FR2 HST with simultaneous multi-panel reception demodulation requirement. The following transmission schemes can be used as starting point

- Single DCI with SDM: 1+1, FFS on 2+2

- Multi DCI with fully over lapping:  1+1 as layer combination, FFS on 2+2

- Multi DCI with non-over lapping: 2+2 as layer combination

- Receiver assumption: MMSE-IRC

  - Option 1: UE joint processing with 4x4

  - Option 2: UE processing with 2x2 per RRH

Simulation assumption

For Rel-17 FR2 HST with single-pannel reception with DPS transmision scheme, 120KHz with 200MHz are considered for requirement. And MCS 17 with rank 2 is conisdered.

Proposal 7: Apply the following MCS, and BW for simulation assumption for multi-panel Rx in FR2 HST scenario as starting point

- MCS 17

- BW 200MHz

## 2.3	UE demodulation requirement for simultaneous multi-panel reception with CA

Introducing intra-band CA demodulation requirement is included in the FR2 HST enhancement work item. If FR2 HST PC6 UE capable of multi-panel operation, we see no reason to restrict the Rel-18 work item into the case where UE is just configured with one single carrier. Therefore, we propose to introduce PDSCH requirement with CA for FR2 PC6 UEs capable of multi-panel operation.

Proposal 8: RAN4 introduce the PDSCH requirement with simultaneous multi-panel reception with intra-band CA for FR2 PC6 UE

# 3	Conclusion

In this contribution, the view on the test scope and test setup for UE demodulation requirement is provided

Proposal 1: to support FR2 HST intra-band CA, PDSCH requirements on single carrier of following bandwidth need to be specified:

- For 120KHz SCS, specify PDSCH requirements on single carrier of BW of {50, 100, 200, 400} MHz

Proposal 2: Transmission schemes with both DPS scheme 1a and DPS scheme 1b should be considered to support CA, under Uni-directional deployment scenario A and Bi-directional deployment scenario B, separately

- Case 1: Uni-directional scenario A with DPS scheme 1b with {50, 100,200, 400} MHz

- Case 2: Bi-directional scenario B with DPS scheme 1a with {50, 100, 200, 400} MHz

Proposal 3: Apply the Rel-17 FR2 HST channel model parameters to PDSCH CA demodulation requirements with DPS transmission schemes

| Parameter | Value |
| --- | --- |
|  | HST-DPS-FR2-UNI-A |
| Ds | 700 m |
| Ds_offset | 10 m |
| Dmin | 10 m |
| v | 350 km/h |
| fd | 9722 Hz |

| Parameter | Value |
| --- | --- |
|  | HST-DPS-FR2-BI-B |
|  | 700 m |
|  | 150 m |
|  | 350 km/h |
|  | 9722 Hz |

Proposal 4: Reused the exiting test parameters of FR2 HST with DPS 1a and DPS 1b for CA requirements

Proposal 5: RAN4 introduces PDSCH requirement with simultaneous multi-panel reception under Bi-directional deployment scenario firstly.  Scenario A can be regarded as starting point. FFS on introduce PDSCH requirement with simultaneous multi-panel reception under Uni-directional scenario pending on the core requirement discussion.

Proposal 6: Both Single-DCI and multi-DCI transmission schemes should be introduced in Rel-18 FR2 HST with simultaneous multi-panel reception demodulation requirement. The following transmission schemes can be used as starting point

- Single DCI with SDM: 1+1, FFS on 2+2

- Multi DCI with fully over lapping:  1+1 as layer combination, FFS on 2+2

- Multi DCI with non-over lapping: 2+2 as layer combination

- Receiver assumption: MMSE-IRC

  - Option 1: UE joint processing with 4x4

  - Option 2: UE processing with 2x2 per RRH

Proposal 7: Apply the following MCS, and BW for simulation assumption for multi-panel Rx in FR2 HST scenario as starting point

- MCS 17

- BW 200MHz

Proposal 8: RAN4 introduce the PDSCH requirement with simultaneous multi-panel reception with intra-band CA for FR2 PC6 UE

# References

RP- 222272, Revised WID on enhanced NR support for high-speed train scenario in frequency range 2 (FR2)

R4-2303173	WF on NR FR2 HST Enhancements RRM requirements		Samsung

R4-2302941          WF for NR_FR2_multiRX_DL_Demod