3GPP TSG-RAN WG4 Meeting #106bis-e 	R4-2305553

Electronic Meeting, April 17 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Network B requirements

Agenda item:	5.26.2.4

Document for:	Discussion

Introduction

In the last RAN4 meeting in Athens, RAN4 discussed the aspects concerning UE requirements related to the MUSIM gaps introduced in Rel-17.

One agreement was reached regarding Network B requirements. The RAN4 agreements are captured in [1] which also capture at least open aspects related to network B requirements to be discussed further.

We will address this further in this paper.

Discussion

On network B requirements

Whether to define network B requirements (4-1-1)

Following issues are still not fully concluded:

- Whether to define network B requirements

- Network B requirements if it will be defined

Related to the first issue RAN4 did agree following:

- Define NW B measurement/cell reselection requirements in IDLE mode only

  - Prioritize the scenario where

    - MUSIM gap is not colliding with other NW A gaps and not dropped

    - NW A configures MUSIM gaps requested by UE

- FFS whether and how to define test cases for these requirements

It was left whether other conditions would need to be defined. As a starting point we think above agreements looks reasonable. However, we also believe we need to return to the conditions once RAN4 has settled once the network B requirements have been defined and agreed.

For example, if we assume to more or less reuse existing idle mode requirements as network B requirements, it seems excessively relaxed to have above strict conditions in place, if the UE has requested a very dense MUSIM gap – for example MUSIM gap pattern #0.

Hence, we suggest keeping above agreement as it is for now and re-discuss the conditions once network B requirements are clearer.

Re-discuss the conditions for the RAN4#106 agreement once network B requirements are clearer.

Agree to continue discussion other conditions during or once NW B requirements are agreed.

Network B requirements if it will be defined (4-1-2)

It seems clear that RAN4 need to define UE measurement requirements for measurements performed on Network B while operating in Network A when UE is allocated MUSIM gaps. If RAN4 does not define such requirements there is no incentive for Network A to allocate any MUSIM gaps.

The requirements for Network B can be defined similar to the existing idle mode measurements requirements. Hence, RAN4 does not need to define any new Network B specific measurement requirements for when UE is measuring Network B using MUSIM gaps allocated by Network A.

Hence, existing idle mode measurement and accuracy requirement can be re-used directly.

The existing UE idle mode measurement and accuracy requirements can be re-used for Network B measurement requirements.

- When defining the requirements RAN4 need to account the scenarios agreed in RAN2.

Conclusion

In the last RAN4 meeting in Athens, RAN4 discussed the aspects concerning UE requirements related to the MUSIM gaps introduced in Rel-17. One agreement was reached regarding Network B requirements. At least two open aspects related to network B requirements needed to be discussed further. In this paper we have addressed these and propose following:

- Re-discuss the conditions for the RAN4#106 agreement once network B requirements are clearer (4-1-1).

- Agree to continue discussion other conditions during or once NW B requirements are agreed (4-1-1).

- The existing UE idle mode measurement and accuracy requirements can be re-used for Network B measurement requirements (4-1-2).

- References

- R4-2303248, WF on NR Dual TxRx Multi-SIM, vivo.

- R4-2217261, WF on RRM requirements for Rel-17 MUSIM gaps, vivo.

- R4-2220443, WF on NR Dual Tx/Rx Multi-SIM, vivo