3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2305560

# Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon

Title: 	UE capability for channel raster enhancement

Agenda Item:	5.6.3

Document for:	Approval

- Introduction

At RAN#99, a new WI on channel raster enhancement was approved in RP-230813, with the following objectives:

- Specify necessary changes to the UE channel raster such that configuring a narrower UE channel BW inside a wider gNB channel BW is always possible [RAN4].

- Changes to BS channel raster can be considered if required [RAN4].

- Specify the corresponding UE capability, if needed, to enable changes to the channel raster [RAN2, RAN4]:

  - RAN4 is to identify the release of the specifications 38.101-1 and 38.104 and the possibility of early implementation. If corresponding capability signalling is provided for early implementation and such early implementation is possible, the change is to be release independent from the identified release.

NOTE: Changes to channel raster need to be compliant with the definition of global channel raster in RAN4 specification.

In this contribution, we provide our consideration on the third objective, i.e. UE capability.

- Discussion

The purpose of the channel raster enhancement is to make it always possible that a narrower UE channel bandwidth can be configured inside wider gNB channel bandwidth. The odd/even constrains have been discussed during the study item on support of irregular channel bandwidth. As a result from the previous discussion, denser channel raster need to be introduced for UE to solve the issue. The issue exists for the bands with 100 kHz channel raster. The first question is whether all these bands are required to support non-100 kHz channel raster, or it will depend on the operator’s request.

Proposal 1: it is proposed to discuss whether all these bands with 100 kHz channel raster are required to support denser channel raster, or it will depend on the operator’s request.

For an earlier release, the additional channel rasters are optional support for UE. Hence a new UE capability is needed. It is proposed to further discuss which release can be optional supported after the new channel raster is decided.

Proposal 2: it is proposed to further discuss which release can be optional supported after the new channel raster is decided.

- Conclusion

In this contribution, we provide our view on the UE capability for channel raster enhancement.

Proposal 1: it is proposed to discuss whether all these bands with 100 kHz channel raster are required to support denser channel raster, or it will depend on the operator’s request.

Proposal 2: it is proposed to further discuss which release can be optional supported after the new channel raster is decided.