3GPP TSG-RAN WG4 Meeting #106-bis-e                                     R4-2305564

Online, April 17 - April 26, 2023

Title:	draft LS on lower MSD capability

Response to:

Release:	Rel-18

Work Item:	NR_ENDC_RF_FR1_enh2

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name: 	Liu Ye

E-mail Address: 	leo.liuye@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

WI on further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1) was approved in RAN#95e, in which there is an objective with study phase for feasibility of lower MSD capability for inter-band CA/EN-DC/DC combinations. In RAN4#106 meeting, it was concluded that:

- The feasibility for MSD improvement for all kinds of MSD has been confirmed based on the evaluation from companies on the selected example band combinations.

- Several promising options for allowing a UE to signal improved lower MSD performance have been discussed, with which companies think it is feasible to introduce the lower MSD capability.

- Details of the lower MSD capability will be further discussed in the UE RF FR1 WI.

Consequently, the WI objective for lower MSD was revised in RAN#99, and RAN2 was added as one responsible WG for the objective.

To facilitate the initial discussion of the signaling design for lower MSD capability, the following agreements reached so far are provided, which are essential information considered by RAN4 for the capability:

- Victim band

- MSD type (harmonic; harmonic mixing; cross band isolation; IMD) with orders

- MSD value/thresholds

- [Power class]

- [Aggressor UL and victim DL bandwidth]

Upon the essential information, some other agreements reached in RAN4 could also be referred for the signalling design:

- A UE should be allowed to report the low MSD capability for any MSD mechanism and order that have been defined in the 3GPP specifications for a given band combination.

- No dynamic signaling scheme will be introduced for lower MSD report in Rel-18

- It is suggested to define exact absolute multiple thresholds for lower MSD

  - Identical Lower MSD thresholds can be applicable to all the band combinations

  - Identical Lower MSD thresholds are applicable for each MSD mechanism

- Lower MSD capability for higher order combination is inherited from lower order fallback combinations

  - For 2-bands combination, the MSD values (or capability class) are supposed to be reported separately as per victim band per MSD type per band combination

  - For 3-bands combination, the MSD values (or capability class) are only reported for IMD of dual UL falling into the third band DL, other kinds of Lower MSD capability (harmonic/ harmonic mixing/cross band isolation/IMD due to dual UL falling into own DL) could inherit from 2-band combinations with the same power class.

  - For combination with more than 3 bands, no need to report the Lower MSD capability any more, the capability could inherit from the fallback combinations with the same power class.

Meanwhile, RAN4 is still working on the details for approaches for a UE to indicate the improved lower MSD performance based on the progress of the study phase. If any update, RAN4 will keep RAN2 informed.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take the above information into account for the following RAN2 work on the signalling design for lower MSD.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#107 	 					22nd – 26th May 2023								Incheon, Korea

TSG-RAN4 Meeting#108						21st – 25th August 2023							Toulouse, France