3GPP TSG-RAN WG4 Meeting #106-bis-e                                     R4-2305566

Online, April 17 - April 26, 2023

Source: 	Huawei, HiSilicon

Title: 	On 4Rx requirements for low operating bands

Agenda Item:	4.29.2

Document for:	Approval

# Introduction

The issues for 4Rx towards low operating bands have been discussed in last meeting. One particular issue on delta TRXSRS was FFS.

This contribution provides our consideration for the issue.

# Discussion

In the WF, the issue for TRxSRS is listed as below:

Issue 2-2-2:  delta TRxSRS

- Proposals

  - Option 1: Doesn’t need to specify with reasons below

    - All the requested lower bands currently are FDD bands in the WID, and SRS antenna switching is designed for TDD bands.

- WF

  - FFS whether specific delta TRxSRS for FDD low bands need to be defined in this WI.

In RAN4 specification, we have the following description for ∆TRxSRS:

The value of ∆TRxSRS is 4.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 3 or power class 5 or power class 1.5 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16.. The value of ∆TRxSRS is 7.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is capable of power class 2 in the band and ΔPPowerClass = 0 dB and not indicating txDiversity-r16.

From RAN4 spec, the boundary for the operating bands is selected as 4.9GHz, i.e. n79, but there is no limitation that the feature of SRS antenna switching is only applicable for TDD bands. Though originally, considering the channel reciprocal characteristic, the feature is more suitable for TDD bands, but even in RAN1, there is no such limitation as well.

Considering the full channel information facilitate the accurate assessment of partial reciprocity, which enables the effective deployment of Rel-17 partial reciprocity codebook, SRS with usage of antenna swathing should also be supported for FDD bands.

Therefore, for the FFS issue on SRS antenna switching, we prefer no restriction for the feature applicable for TDD bands only. And in our view, there is no specific specification work needed for the raised issue.

Proposal 1: No need to limit the feature of SRS antenna switching to TDD bands only.

# Conclusion

This contribution provides our consideration for an FFS issue on SRS antenna switching for 4Rx in low bands.

Proposal 1: No need to limit the feature of SRS antenna switching to TDD bands only.

# References

[1] R4-2303522, WF on 4Rx requirements for low operating bands, OPPO