3GPP TSG-RAN WG4 Meeting # 106bis-e													R4-2305608

Online, April 17 – April 26, 2023

Agenda item:	5.17.4

Source: 	Qualcomm Incorporated

Title: 	MU budget for FR2 MIMO OTA test system

Document for:	Approval

# Introduction

This paper provides the preliminary MU analysis for FR2 MIMO OTA test system.

# Discussion

The preliminary MU budget for FR2 3D-MPAC system has been discussed and specified in TS 38.151 as shown in below table (Table B.2.5-1 in TS 38.151). It could be observed that the only remaining issue is example value for channel emulator.

Table B.2.5-1: Preliminary measurement uncertainty budget for FR2 3D-MPAC system

| UID | Description of uncertainty contribution | Example value (26.5GHz≤f≤29.5GHz) | Example value (37GHz ≤f≤40GHz) | Distribution of the probability | Std Uncertainty (26.5GHz≤f≤29.5GHz) [dB] | Std Uncertainty (37GHz ≤f≤40GHz) [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 | Mismatch for measurement process | 1.30 | TBD | Actual | 1.30 | TBD |
| 2 | Measure distance uncertainty | 0.00 | TBD | Rectangular | 0.00 | TBD |
| 3 | Quality of quiet zone | 0.6 | TBD | Actual | 0.6 | TBD |
| 4 | Base Station simulator | 2.9 | TBD | Normal | 1.45 | TBD |
| 5 | Channel Emulator -absolute value -stability -linearity | TBD | TBD | Normal | TBD | TBD |
| 6 | Amplifier uncertainties | [2.10] | TBD | [Normal] | [1.05] | TBD |
| 7 | Random uncertainty | 0.50 | TBD | Normal | 0.25 | TBD |
| 8 | Throughput measurement: output level step resolution | 0.25 | TBD | Rectangular | 0.14 | TBD |
| 9 | DUT sensitivity drift | 0.2 | 0.2 | Rectangular | 0.12 | TBD |
| 10 | Signal flatness | 0.00 | 0.00 | Normal | 0.00 | TBD |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 11 | Mismatch for calibration process - loopback cable path - system input path - reference antenna | 0.00 | TBD | U-Shaped | 0.00 | TBD |
| 12 | Reference antenna positioning misalignment | 0.01 | TBD | Rectangular | 0.00 | TBD |
| 13 | Quality of quiet zone | 0.60 | TBD | Actual | 0.60 | TBD |
| 14 | Total uncertainty of the Network Analyzer | 0.73 | TBD | Normal | 0.37 | TBD |
| 15 | Uncertainty of an absolute gain of the calibration antenna | 0.60 | TBD | Normal | 0.30 | TBD |
| 16 | Offset of the Phase Center of the Reference Antenna | 0.00 | TBD | Rectangular | 0.00 | TBD |
| Total Expanded Uncertainty, U, with 95% Confidence Interval |  |  |  |  | TBD | TBD |

As described in Table B.2.5-1 of TS 38.151, the MU of channel emulator consists of absolute value, stability and linearity. In TR 38.810, uncertainty contributions for DL absolute power level are defined in Table B.2.1.2-2 for RRM test setup in which one of uncertainty sources is gNB emulator uncertainty on absolute level. As defined in Table B.2.1.2-2 of TR 38.810, the gNB emulator uncertainty is referring to Table B.1.3.3-2 as below.

Table B.1.3.3-2: IFF method 1 measurement uncertainty for EIS measurement

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.10 | Normal | 2.00 | 0.05 |
| 2 | Quality of Quiet Zone | 1.50 | Actual | 1.00 | [1.50] |
| 3 | Standing wave between DUT and measurement antenna | 0.00 | U-shaped | 1.41 | 0.00 |
| 4 | Mismatch (NOTE 1) | 1.30 | Actual | 1.00 | [1.30] |
| 5 | gNB uncertainty on absolute level | [3.34] | Normal | 2.00 | [1.67] |
| … |  |  |  |  |  |

Considering the uncertainty of absolute level in 3D-MAC would be similar as RRM test setup which includes uncertainty of the non-linearity, etc., it is proposed to reuse gNB uncertainty on absolute level defined in Table B.1.3.3-2 of TR 38.810 for FR2 3D-MPAC system as the baseline.

Proposal 1: RAN4 to consider following measurement uncertainty budget for FR2 3D-MPAC system as the baseline and further refine the value for measurement uncertainty elements based on the input from companies.

| UID | Description of uncertainty contribution | Example value (26.5GHz≤f≤29.5GHz) | Example value (37GHz ≤f≤40GHz) | Distribution of the probability | Std Uncertainty (26.5GHz≤f≤29.5GHz) [dB] | Std Uncertainty (37GHz ≤f≤40GHz) [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 | Mismatch for measurement process | 1.30 | TBD | Actual | 1.30 | TBD |
| 2 | Measure distance uncertainty | 0.00 | TBD | Rectangular | 0.00 | TBD |
| 3 | Quality of quiet zone | 0.6 | TBD | Actual | 0.6 | TBD |
| 4 | Base Station simulator | 2.9 | TBD | Normal | 1.45 | TBD |
| 5 | Channel Emulator -absolute value -stability -linearity | [3.34] | TBD | Normal | [1.67] | TBD |
| 6 | Amplifier uncertainties | [2.10] | TBD | [Normal] | [1.05] | TBD |
| 7 | Random uncertainty | 0.50 | TBD | Normal | 0.25 | TBD |
| 8 | Throughput measurement: output level step resolution | 0.25 | TBD | Rectangular | 0.14 | TBD |
| 9 | DUT sensitivity drift | 0.2 | 0.2 | Rectangular | 0.12 | TBD |
| 10 | Signal flatness | 0.00 | 0.00 | Normal | 0.00 | TBD |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 11 | Mismatch for calibration process - loopback cable path - system input path - reference antenna | 0.00 | TBD | U-Shaped | 0.00 | TBD |
| 12 | Reference antenna positioning misalignment | 0.01 | TBD | Rectangular | 0.00 | TBD |
| 13 | Quality of quiet zone | 0.60 | TBD | Actual | 0.60 | TBD |
| 14 | Total uncertainty of the Network Analyzer | 0.73 | TBD | Normal | 0.37 | TBD |
| 15 | Uncertainty of an absolute gain of the calibration antenna | 0.60 | TBD | Normal | 0.30 | TBD |
| 16 | Offset of the Phase Center of the Reference Antenna | 0.00 | TBD | Rectangular | 0.00 | TBD |
| Total Expanded Uncertainty, U, with 95% Confidence Interval |  |  |  |  | TBD | TBD |

# Conclusion

In this paper, we provided our views on MU budget for FR2 MIMO OTA test system with following proposal:

Proposal 1: RAN4 to consider following measurement uncertainty budget for FR2 3D-MPAC system as the baseline and further refine the value for measurement uncertainty elements based on the input from companies.

| UID | Description of uncertainty contribution | Example value (26.5GHz≤f≤29.5GHz) | Example value (37GHz ≤f≤40GHz) | Distribution of the probability | Std Uncertainty (26.5GHz≤f≤29.5GHz) [dB] | Std Uncertainty (37GHz ≤f≤40GHz) [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 | Mismatch for measurement process | 1.30 | TBD | Actual | 1.30 | TBD |
| 2 | Measure distance uncertainty | 0.00 | TBD | Rectangular | 0.00 | TBD |
| 3 | Quality of quiet zone | 0.6 | TBD | Actual | 0.6 | TBD |
| 4 | Base Station simulator | 2.9 | TBD | Normal | 1.45 | TBD |
| 5 | Channel Emulator -absolute value -stability -linearity | [3.34] | TBD | Normal | [1.67] | TBD |
| 6 | Amplifier uncertainties | [2.10] | TBD | [Normal] | [1.05] | TBD |
| 7 | Random uncertainty | 0.50 | TBD | Normal | 0.25 | TBD |
| 8 | Throughput measurement: output level step resolution | 0.25 | TBD | Rectangular | 0.14 | TBD |
| 9 | DUT sensitivity drift | 0.2 | 0.2 | Rectangular | 0.12 | TBD |
| 10 | Signal flatness | 0.00 | 0.00 | Normal | 0.00 | TBD |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 11 | Mismatch for calibration process - loopback cable path - system input path - reference antenna | 0.00 | TBD | U-Shaped | 0.00 | TBD |
| 12 | Reference antenna positioning misalignment | 0.01 | TBD | Rectangular | 0.00 | TBD |
| 13 | Quality of quiet zone | 0.60 | TBD | Actual | 0.60 | TBD |
| 14 | Total uncertainty of the Network Analyzer | 0.73 | TBD | Normal | 0.37 | TBD |
| 15 | Uncertainty of an absolute gain of the calibration antenna | 0.60 | TBD | Normal | 0.30 | TBD |
| 16 | Offset of the Phase Center of the Reference Antenna | 0.00 | TBD | Rectangular | 0.00 | TBD |
| Total Expanded Uncertainty, U, with 95% Confidence Interval |  |  |  |  | TBD | TBD |

# References

- R4-2302975, “Framework for FR2 MIMO OTA performance requirements”, CAICT

- R4-2302927, “WF on NR MIMO OTA enhancement”, CAICT