3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305614

e-meeting, April 17 – April 26, 2023

Title:	On hybrid approach for FR2 MIMO OTA

Source:	OPPO

Agenda item:	5.17.5

Document for:	Approval

- Introduction

For FR2 MIMO OTA performance requirement derivation, both pure measurement approach and hybrid approach were considered and the down-selection would be made no later than RAN4#109 [1].

This contribution further discusses the details on data collection for the hybrid approach.

- Discussion

In the approved framework for FR2 MIMO OTA [2], the Hybrid approach is described in clause 2.2.6.2, and three approaches are proposed and the applicability of the approaches depends on the conditions of simulation alignment results and the amount of measurement results.

Not matter which Hybrid approach is applied for performance requirement derivation, simulation results will be used together with measurement results. The simulation results submitted to the data pool are required to follow practical UE implementations.

As per general understanding of practical implementation, both the FR2 antenna patterns and the allocation of the antenna modules should come from real FR2 devices, favorably FR2 commercial devices. Following this understanding, there will be a circumstance that the simulation result with the simulation assumption based on a commercial device, naming Device A, is submitted to the data pool, while the measurement result of Device A is also submitted to the data pool. The two results are two collected data in the data pool for data post-processing. However, they are actually characterizing the same UE model. And this circumstance will bias the data distribution of the data pool, especially for the case that the collected data in the pool are few. To avoid the same model being double counted, similar approaches in [3] should be consider to adopt for not only the measurement data pool but also the hybrid data pool.

Proposal: similar approaches in R4-2220265 (WF for test device information collection for the measurement data pool from Rel-17 TRP TRS WI and Rel-17 FR1 MIMO OTA) should be consider to adopt for not only the measurement data pool but also the hybrid data pool.

- Conclusions

This paper presents our considerations on the collected data in the hybrid data pool when both simulated results and measurement results are submitted. And provide the following proposal.

Proposal: similar approaches in R4-2220265 (WF for test device information collection for the measurement data pool from Rel-17 TRP TRS WI and Rel-17 FR1 MIMO OTA) should be consider to adopt for not only the measurement data pool but also the hybrid data pool.

- References

[1] R4-2302975, WF for Rel-18 MIMO OTA, CAICT

[2] R4-2301046, Updated Framework for FR2 MIMO OTA performance requirements development, CAICT

[3] R4-2220265, WF for test device information collection for the measurement data pool from Rel-17 TRP TRS WI and Rel-17 FR1 MIMO OTA, Apple