# 3GPP TSG-RAN WG4 Meeting # 106-bis-e					R4-2305646

# Electronic Meeting, 17th  –26th  April, 2023

# Source:		Ericsson

Title:			Remaining Redcap RF issue

Agenda item:		5.32.2

Document for:		Approval

# Introduction

In this contribution, we present our view on the remaining RF issues for Rel-18 RedCap WI.

# Discussion

In WF [1], the remaining Redcap issue is listed below:

Issue 2-3: Other Rx requirement than REFSENS

Discussions

Agreement:

- FFS how to specify other Rx RF requirements other than REFSENS for eRedCap UE.

According to RAN1 agreement below, the PDSCH can be allocated anywhere within the 20MHz RF bandwidth. To test the worst case, the wanted signal resource allocation should be as close as the interferer signal. For REFSENS test, the note 1 in UL configuration table could assure this. For the ACS and blocking tests, RAN4 assumes UE should pass all the valid PDSCH allocation positions and if RAN5 want to reduce the test time and could define a worst case for it. For example, in TS 38.52-1, Table 7.3.2.4.1-2 is listed for downlink configuration of each RB allocation,  this table could be used to define the “worst case” test, such table is used in ACS, in-band blocking, out-of-band blocking , narrow band blocking, wideband intermodulation and spurious.  RAN4 does not need update the requirement to define the worst case for test.

Unicast PDSCH bandwidth

Agreement:

- For UE BB complexity reduction, a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

- The number of PRB scheduled in DCI is not larger than the maximum number of PRB agreed in previous agreement from 110b-e

For Rx requirements other than REFENS, RAN4 can rely on RAN5 to define a worst case and therefore , there is no RAN4 impact.

From the latest RAN1 agreement, the # of RB is agreed and RAN4 can start CR on the REFSENS for eRedcap UE.

Number of PRBs

Agreement:

For UE BB bandwidth reduction, for PUSCH, select the following option for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable:

- Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS

For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot:

- Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS

Note: No intention to change the RAN4 RF specifications about maximum transmission PRB number

RAN4 can start discuss REFSENS CR based on latest RAN1 agreement on the RB number.

For the REFSENS requirement, as the RAN1 does not intend to change the RAN4 specification on the maximum transmission PRB number, it means that the REFSENS for SCS 15kHz and channel bandwidth 10MHz, 15Mhz and 20MHz due to the limited PDSCH number can be the same with 5MHz. For the bands supporting the 30kHz SCS, the REFSENS for channel bandwidth 10MHz, 15Mhz and 20MHz, according to Table 5.3.2-1 in TS 38.101-1, reusing 5MHz is not possible so scaling the REFSENS from the 10MHz will be needed in this case. From current channel bandwidth support for all bands, the min channel bandwidth for SCS 30kHz is 10MHz, so no need to consider the REFSENS for 30MHz. In above RAN1 agreement, there is no agreement on 60kHz and also there is no mentioned 60kHz in WI objective, so RAN4 needs to wait RAN1 progress on 60kHz conclusion.

Wait RAN1 progress on 60kHz RB limitation.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

The eRedcap UE REFSENS for SCS 15kHz and channel bandwidth 10MHz, 15Mhz and 20MHz can be the same with 5MHz.

For 30kHz SCS, the agreed limited 12 PRB is bigger than the maximum transmission bandwidth configuration which is 11 RB.

The eRedcap UE REFSENS for SCS 30kHz and channel bandwidth 10MHz, 15Mhz and 20MHz can be scaled using 10MHz REFSENS.

From the discussion about, the REFSENS for 2 RX for eRedcap UE can be specified with additional table below.

Table 7.3I.2-1a: Two antenna port reference sensitivity for eRedcap UE for FDD band

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |
| --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) |
| n1 | 15 | 5,10,15, 20 | -100 |
|  | 30 | 10,15, 20 | -97.1+10log(12/24)= -100.1 |
|  | 60 | 10,15, 20 |  |
| n2 | 15 | 5,10,15,20 | -98 |
|  | 30 | 10,15,20 | -98.1 |
|  | 60 | 10,15,20 |  |
| n3 | 15 | 5,10,15,20 | -97.0 |
|  | 30 | 10,15,20 | -97.1 |
|  | 60 | 10,15,20 |  |
| n5 | 15 | 5,10,15,20 | -98.0 |
|  | 30 | 10,15,20 | -98.1 |
| n7 | 15 | 5,10,15,20 | -98.0 |
|  | 30 | 10,15,20 | -98.1 |
|  | 60 | 10,15,20 |  |
| n8 | 15 | 5,10,15,20 | -97.0 |
|  | 30 | 10,15,20 | -97.1 |
| n12 | 15 | 5,10,15 | -97.0 |
|  | 30 | 10,15 | -97.1 |
| n13 | 15 | 5,10 | -97.0 |
|  | 30 | 10 | -97.1 |
| n14 | 15 | 5,10 | -97.0 |
|  | 30 | 10 | -97.1 |
| n18 | 15 | 5,10,15 | -100.0 |
|  | 30 | 10,15 | -100.1 |
| n20 | 15 | 5,10,15,20 | -97.0 |
|  | 30 | 10,15,20 | -97.1 |
| n24 | 15 | 5,10 | -100.0 |
|  | 30 | 10 | -100.5 |
|  | 60 | 10 |  |
| n25 | 15 | 5,10,15,20 | -96.5 |
|  | 30 | 10,15,20 | -96.6 |
|  | 60 | 10,15,20 |  |
| n26 | 15 | 5,10,15,20 | -97.56 |
|  | 30 | 10,15,20 | -97.86 |
| n28 | 15 | 5,10,15,20 | -98.5 |
|  | 30 | 10,15,20 | -98.6 |
| n30 | 15 | 5,10 | -99.0 |
|  | 30 | 10 | -99.1 |
| n65 | 15 | 5,10,15,20 | -99.5 |
|  | 30 | 10,15,20 | -99.6 |
|  | 60 | 10,15,20 |  |
| n66 | 15 | 5,10,15,20 | -99.5 |
|  | 30 | 10,15,20 | -99.6 |
|  | 60 | 10,15,20 |  |
| n70 | 15 | 5,10,15,20 | -100.0 |
|  | 30 | 10,15,20 | -100.1 |
|  | 60 | 10,15,20 |  |
| n71 | 15 | 5,10,15,20 | -97.2 |
|  | 30 | 10,15,20 | -97.3 |
| n74 | 15 | 5,10,15,20 | -99.53 |
|  | 30 | 10,15,20 | -99.63 |
|  | 60 | 10,15,20 |  |
| n85 | 15 | 5,10,15 | -97.0 |
|  | 30 | 10,15 | -97.1 |
| n100 | 15 | 5 | -100 |
| n105 | 15 | 5,10,15,20 | -97.28 |
|  | 30 | 10,15,20 | -97.3 |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. NOTE 8: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |

Table 7.3I.2-4a: Two antenna port reference sensitivity for eRedcap UE for TDD band and FDD band with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 | TDD |
|  | 30 | 10, 15 | -97.1 +10log10(12/24)=-100.1 |  |
|  | 60 | 10, 15 |  |  |
| n381 | 15 | 5, 10, 15, 20, | -100 | TDD |
|  | 30 | 10, 15, 20 | -100.1 |  |
|  | 60 | 10, 15, 20 |  |  |
| n39 | 15 | 5, 10, 15, 20 | -100 | TDD |
|  | 30 | 10, 15, 20 | -100.1 |  |
|  | 60 | 10, 15, 20 |  |  |
| n40 | 15 | 5, 10, 15, 20 | -100 | TDD |
|  | 30 | 10, 15, 20 | -1001 |  |
|  | 60 | 10, 15, 20 |  |  |
| n411, n901 | 15 | 5, 10, 15, 20 | -94.8 | TDD |
|  | 30 | 10, 15, 20 | -98.1 |  |
|  | 60 | 10, 15, 20 |  |  |
| n481 | 15 | 5, 10, 15, 20 | -99 | TDD |
|  | 30 | 10, 15, 20 | -99.1 |  |
|  | 60 | 10, 15, 20 |  |  |
| n50 | 15 | 5, 10, 15, 20 | -100 | TDD |
|  | 30 | 10, 15, 20 | -100.1 |  |
|  | 60 | 10, 15, 20 |  |  |
| n51 | 15 | 5 | -100 | TDD |
| n53 | 15 | 5, 10 | -100 | TDD |
|  | 30 | 10 | -100.1 |  |
|  | 60 | 10 |  |  |
| n771,4 | 15 | 10, 15, 20 | -95.3 + 10log10(25/52)= -98.5 | TDD |
|  | 30 | 10, 15, 20 | -98.6 |  |
|  | 60 | 10, 15, 20 |  |  |
| n781 | 15 | 10, 15, 20 | -95.8 + 10log10(25/52)=-99 | TDD |
|  | 30 | 10, 15, 20 | -99.1 |  |
|  | 60 | 10, 15, 20 |  |  |
| n791 | 15 | 10, 20 | -95.8 + 10log10(25/52)=-99 | TDD |
|  | 30 | 10, 20 | -99.1 |  |
|  | 60 | 10, 20 |  |  |
| n91 | 15 | 5 | -100 | FDD |
| n92 | 15 | 5,10,15,20 | -100 | FDD |
|  | 30 | 10,15,20 | -100.1 |  |
| n93 | 15 | 5 | -100 | FDD |
| n94 | 15 | 5,10,15,20 | -100 | FDD |
|  | 30 | 10,15,20 | -100.1 |  |
| n101 | 15 | 5, 10 | -100 | TDD |
|  | 30 | 10 | -100.1 |  |
| n1041,10 | 15 | 20 | -90.7 + 10log10(25/106)=-97 | TDD |
|  | 30 | 20 | -90.8 +10log10(12/51)=-97.1 |  |
|  | 60 | 20 |  |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: Void. NOTE 10: A UE may implement two RX antenna ports for band n104 when conditions are met. The exact conditions are FFS. |  |  |  |  |

For the Uplink configuration, the table can be updated as below:

Table 7.3I.2-4a: Uplink configuration for reference sensitivity for eRedcap UE

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | Duplex Mode |
| n1 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 101 | 18 | 24 |  |
| n2 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 | 101 | 121 | 121 | 121 |  |
|  | 60 |  | 101 | 101 | 101 |  |
| n3 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 101 | 101 | 101 |  |
| n5 | 15 | 25 | 251 | 201 | 201 | FDD |
|  | 30 |  | 121 | 101 | 101 |  |
| n7 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 101 | 18 | 181 |  |
| n8 | 15 | 25 | 251 | 201 | 201 | FDD |
|  | 30 |  | 121 | 101 | 101 |  |
| n12 | 15 | 201 | 201 | 201 |  | FDD |
|  | 30 |  | 101 | 101 |  |  |
| n13 | 15 | 201 | 201 |  |  | FDD |
|  | 30 |  | 101 |  |  |  |
| n14 | 15 | 201 | 201 |  |  | FDD |
|  | 30 |  | 101 |  |  |  |
| n18 | 15 | 25 | 251 | 251 |  | FDD |
|  | 30 |  | 101 | 101 |  |  |
| n20 | 15 | 25 | 201 | 202 | 202 | FDD |
|  | 30 |  | 101 | 102 | 102 |  |
| n24 | 15 | 25 | 251 |  |  | FDD |
|  | 30 |  | 121 |  |  |  |
|  | 60 |  | 10 |  |  |  |
| n25 | 15 | 25 | 501 | 501 | 501 | FDD |
|  | 30 |  | 24 | 241 | 241 |  |
|  | 60 |  | 101 | 101 | 101 |  |
| n26 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
| n28 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 101 | 101 | 101 |  |
| n30 | 15 | 201 | 201 |  |  | FDD |
|  | 30 |  | 101 |  |  |  |
| n34 | 15 | 25 | 251 | 251 |  | TDD |
|  | 30 |  | 121 | 121 |  |  |
|  | 60 |  | 10 | 18 |  |  |
| n38 | 15 | 25 | 50 | 75 | 100 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n39 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n40 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n41, n90 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n48 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n50 | 15 | 25 | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n51 | 15 | 25 |  |  |  | TDD |
| n53 | 15 | 25 | 251 |  |  | TDD |
|  | 30 |  | 121 |  |  |  |
|  | 60 |  | 10 |  |  |  |
| n65 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 101 | 18 | 24 |  |
| n66 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 101 | 18 | 24 |  |
| n70 | 15 | 25 | 251 | 251 | Note 3 | FDD |
|  | 30 |  | 121 | 121 | Note 3 |  |
|  | 60 |  | 101 | 18 | Note 3 |  |
| n71 | 15 | 25 | 251 | 201 | 201 | FDD |
|  | 30 |  | 121 | 101 | 101 |  |
| n74 | 15 | 25 | 251 | 251 | 251 | FDD |
|  | 30 |  | 101 | 101 | 101 |  |
|  | 60 |  | 51 | 51 | 51 |  |
| n77 | 15 |  | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n78 | 15 |  | 251 | 251 | 251 | TDD |
|  | 30 |  | 121 | 121 | 121 |  |
|  | 60 |  | 10 | 18 | 24 |  |
| n79 | 15 |  | 251 |  | 251 | TDD |
|  | 30 |  | 121 |  | 121 |  |
|  | 60 |  | 10 |  | 24 |  |
| n85 | 15 | 201 | 201 | 201 |  | FDD |
|  | 30 |  | 101 | 101 |  |  |
| n91 | 15 | 254 | 201,4 |  |  | FDD |
| n92 | 15 | 25 | 201 | 201 | 201 | FDD |
|  | 30 |  | 101 | 101 | 101 |  |
| n93 | 15 | 254 | 251,4 |  |  | FDD |
| n94 | 15 | 25 | 251 | 201 | 201 | FDD |
|  | 30 |  | 121 | 101 | 101 |  |
| n100 | 15 | 25 |  |  |  | FDD |
| n101 | 15 | 25 | 251 |  |  | TDD |
|  | 30 |  | 121 |  |  |  |
| n104 | 15 |  |  |  | 251 | TDD |
|  | 30 |  |  |  | 121 |  |
|  | 60 |  |  |  | 24 |  |
| n105 | 15 | 25 | 251 | 201 | 201 | FDD |
|  | 30 |  | 121 | 101 | 101 |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). Note 2: For Band 20; for 15 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16; for 30 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 6 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 8; for 60 kHz SCS, in the case of 15 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 3 and in the case of 20 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 4; Note 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. Note 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. Note 5: Void |  |  |  |  |  |  |

WF[1] agrees to define the new requirement and also reusing the scaling scaling factor for 1RX/HD-FDD. With the agreed limited RB, the HD-FDD REFSENS can be added as below:

Table 7.3I.2-2a: HD-FDD eRedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -97.2+ 10log(12/24)= -100.2 | -100.2 | -100.2 |
|  | 60 |  |  |  |  |
| n2 | 15 | -98.8 | -98.8 | -98.8 | -98.8 |
|  | 30 |  | -99.0 | -99.0 | -99.0 |
|  | 60 |  |  |  |  |
| n3 | 15 | -97.8 | -97.8 | -97.8 | -97.8 |
|  | 30 |  | -98.0 | -98.0 | -98.0 |
|  | 60 |  |  |  |  |
| n5 | 15 | -98.8 | -98.8 | -98.8 | -98.8 |
|  | 30 |  | -99.0 | -99.0 | -99.0 |
| n7 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
|  | 60 |  |  |  |  |
| n8 | 15 | -97.8 | -97.8 | -97.8 | -97.8 |
|  | 30 |  | -98.0 | -98.0 | -98.0 |
| n12 | 15 | -97.8 | -97.8 | -97.8 |  |
|  | 30 |  | -98.0 | -98.0 |  |
| n13 | 15 | -97.8 | -97.8 |  |  |
|  | 30 |  | -98.0 |  |  |
| n14 | 15 | -97.8 | -97.8 |  |  |
|  | 30 |  | -98.0 |  |  |
| n18 | 15 | -100.0 | -100.0 | -100.0 |  |
|  | 30 |  | -100.2 | -100.2 |  |
| n20 | 15 | -97.8 | -97.8 | -97.8 | -97.8 |
|  | 30 |  | -98.0 | -98.0 | -98.0 |
| n24 | 15 | -100.0 | -100.0 |  |  |
|  | 30 |  | -100.2 |  |  |
|  | 60 |  |  |  |  |
| n25 | 15 | -97.3 | -97.3 | -97.3 | -97.3 |
|  | 30 |  | -97.5 | -97.5 | -97.5 |
|  | 60 |  |  |  |  |
| n26 | 15 | -98.3 | -98.3 | -98.3 | -98.3 |
|  | 30 |  | -98.5 | -98.5 | -98.5 |
| n28 | 15 | -99.3 | -99.3 | -99.3 | -99.3 |
|  | 30 |  | -99.5 | -99.5 | -99.5 |
| n30 | 15 | -99.5 | -99.5 |  |  |
|  | 30 |  | -99.7 |  |  |
| n65 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
|  | 60 |  |  |  |  |
| n66 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
|  | 60 |  |  |  |  |
| n70 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
|  | 60 |  |  |  |  |
| n71 | 15 | -98.0 | -98.0 | -98.0 | -98.0 |
|  | 30 |  | -98.2 | -98.2 | -98.2 |
| n74 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
|  | 60 |  |  |  |  |
| n85 | 15 | -97.8 | -97.8 | -97.8 |  |
|  | 30 |  | -98.0 | -98.0 |  |
| n91 | 15 | -100.0 |  |  |  |
| n92 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
| n93 | 15 | -100.0 |  |  |  |
| n94 | 15 | -100.0 | -100.0 | -100.0 | -100.0 |
|  | 30 |  | -100.2 | -100.2 | -100.2 |
| n100 | 15 | -100 |  |  |  |
| n105 | 15 |  |  |  |  |
|  | 30 |  |  |  |  |

For the 1 Rx scaling for HD-FDD eRedcap, the new table can be added as below:

Table 7.3I.2-3a: HD-FDD eRedCap UE with 1 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n2 | 15 | -96.3 | -96.3 | -96.3 | -96.3 |
|  | 30 |  | -96.5 | -96.5 | -96.5 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n3 | 15 | -95.3 | -95.3 | -95.3 | -95.3 |
|  | 30 |  | -95.5 | -95.5 | -95.5 |
|  | 60 |  | -92.8 | -90.7 | -89.5 |
| n5 | 15 | -96.3 | -96.3 | -96.3 | -96.3 |
|  | 30 |  | -96.5 | -96.5 | -96.5 |
| n7 | 15 | -96.3 | -96.3 | -96.3 | -96.3 |
|  | 30 |  | -96.5 | -96.5 | -96.5 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n8 | 15 | -95.3 | -95.3 | -95.3 | -95.3 |
|  | 30 |  | -95.5 | -95.5 | -95.5 |
| n12 | 15 | -95.3 | -95.3 | -95.3 |  |
|  | 30 |  | -95.5 | -95.5 |  |
| n13 | 15 | -95.3 | -95.3 |  |  |
|  | 30 |  | -95.5 |  |  |
| n14 | 15 | -95.3 | -95.3 |  |  |
|  | 30 |  | -95.5 |  |  |
| n18 | 15 | -97.5 | -97.5 | -97.5 |  |
|  | 30 |  | -97.7 | -97.7 |  |
| n20 | 15 | -95.3 | -95.3 | -95.3 | -95.3 |
|  | 30 |  | -95.5 | -95.5 | -95.5 |
| n24 | 15 | -97.5 | -97.5 |  |  |
|  | 30 |  | -97.7 |  |  |
|  | 60 |  | -95.0 |  |  |
| n25 | 15 | -94.8 | -94.8 | -94.8 | -94.8 |
|  | 30 |  | -95.0 | -95.0 | -95.0 |
|  | 60 |  | -92.3 | -90.2 | -89.0 |
| n26 | 15 | -95.8 | -95.8 | -95.8 | -95.8 |
|  | 30 |  | -96.0 | -96.0 | -96.0 |
| n28 | 15 | -96.8 | -96.8 | -96.8 | -96.8 |
|  | 30 |  | -97.0 | -97.0 | -97.0 |
| n30 | 15 | -97.0 | -97.0 |  |  |
|  | 30 |  | -97.2 |  |  |
| n65 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n66 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n70 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n71 | 15 | -95.5 | -95.5 | -95.5 | -95.5 |
|  | 30 |  | -95.7 | -95.7 | -95.7 |
| n74 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n85 | 15 | -95.3 | -95.3 | -95.3 |  |
|  | 30 |  | -95.5 | -95.5 |  |
| n91 | 15 | -97.5 |  |  |  |
| n92 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |
| n93 | 15 | -97.5 |  |  |  |
| n94 | 15 | -97.5 | -97.5 | -97.5 | -97.5 |
|  | 30 |  | -97.7 | -97.7 | -97.7 |

RAN4 discuss the above changed Tables for eRedCap UE.

# Conclusions

In this contribution, we present our view on the remaining specification impact with below observations and proposals:

Proposal-1:For Rx requirements other than REFENS, RAN4 can rely on RAN5 to define a worst case and therefore , there is no RAN4 impact.

Observation 1 RAN4 can start discuss REFSENS CR based on latest RAN1 agreement on the RB number.

Observation 2 The eRedcap UE REFSENS for SCS 15kHz and channel bandwidth 10MHz, 15Mhz and 20MHz can be the same with 5MHz.

Observation 3 For 30kHz SCS, the agreed limited 12 PRB is bigger than the maximum transmission bandwidth configuration which is 11 RB.

Observation 4 The eRedcap UE REFSENS for SCS 30kHz and channel bandwidth 10MHz, 15Mhz and 20MHz can be scaled using 10MHz REFSENS.

Proposal-2:Wait RAN1 progress on 60kHz RB limitation.

Proposal-3: RAN4 discuss the above changed Tables for eRedCap UE.

# References

R4-2303565, WF on eRedCap UE RF requirements, Ericsson, Nokia