3GPP TSG-RAN WG4 Meeting # 106bis-e           	R4-2305657

Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon

Title: 	UE Architectures Suitable for Non-Collocated Intra-Band CA/EN-DC Rel18

Agenda Item:	5.12.2

Document for:	Discussion

# Introduction

In RAN4 106 meeting, the following was captured in the WF[1]:

< Issue 3-2-1: Whether to keep TAE/network synchronization requirement for Type 3a/3b > - Option.1 Keep TAE 3us - Option.2 MRTD [3]us - Option.3 MRTD < CP Way Forward: Continue further discussions in the next meeting.

In the following sections we will present our view on synchronization requirements in type-3 UEs and type 4 UEs. Moreover RF requirements for type 4 UEs will be specified.

# Discussion

## Type-3 UE

The options proposed in the WF are diverging from network and UE perspective. These timing parameters are intertwined and changing one can result in changing the others.

The MRTD is a parameter of concern for UEs. It is governed by (1):

MRTD=TAE+DelayRF propagation+ Delay Spreadchannel        (1)

TAE is a parameter of concern for network. It consists all the timing synchronization imperfections between the two CCs. The TAE includes the timing errors due to GPS synchronization, LO jitter, backhaul propagation delay and etc. The RF propagation delay depends on the UE distance from the gNbs. In [2] we showed the propagation delay could vary up to 2.11us in LOS scenario with a maximum power imbalance of 25dB. Moreover, the delay spread is a channel intrinsic property and it shows the arrival time difference between copies of the signal at the UE. In TS 38.101-4 different delay profiles for NR channel models are considered where the delay spread can vary from 30 ns to 300 ns

Table 1: Delay profiles for NR channel models

| Model | Number of channel taps | Delay spread (r.m.s.) | Maximum excess tap delay (span) | Delay resolution |
| --- | --- | --- | --- | --- |
| TDLA30 | 12 | 30 ns | 290 ns | 5 ns |
| TDLB100 | 12 | 100 ns | 480 ns | 5 ns |
| TDLC300 | 12 | 300 ns | 2595 ns | 5 ns |

Currently the maximum TAE (TAEmax) is set to 3us. Although in an optimized network the TAE should be as low as possible, but due to implementation constraints it is possible that in some sites the TAE reaches its maximum values. By following option 2 the MRTD is suggested to be a fixed value (3 us as an example), this fixed value should be large enough to include the time budget for propagation delay and channel delay spread. The option 3 imposes more constraints to the MRTD as it sets the upper limit as the CP duration. In case of a SCS of 30 KHz the CP duration equals to 2.34us. As MRTD is directly related to TAE, with option 3, the type-3 UEs that operate in the cells, where TAE > CP, will not be able to use their non-collocated CA/EN-DC feature. On the other hand, the other UEs that operate in cells where TAE  ≤ CP, they will have a limited time budget for propagation delay and channel delay spread. Assuming option 2 and (1):

DelayRF propagation+ Delay Spreadchannel≤CP-TAE     (2)

Based on (2) and the fact that operators cannot control channel delay spread, the RF propagation delay will be limited. This reduces significantly the coverage range of type-3 UEs within these sites and takes all the benefit of the non-collocated CA/EN-DC feature from it.

As only two RAN4 meetings are left in the Core part of the WI and the fact that companies support different options which are not converging we propose to postpone type-3 UE to future releases.

Observation 1: Following option 2 (MRTD [3]us) the MRTD is suggested to be a fixed value (3 us as an example), this fixed value should be large enough to include the TAE, the time budget for propagation delay and the channel delay spread

Observation 2: Following option 3 (MRTD < CP), the type-3 UEs in the cells where TAE> CP will not be able to use their non-collocated CA/EN-DC features. The other UEs that operate in cells where TAE ≤ CP, they will have a limited time budget for propagation delay and channel delay spread. Therefore, their coverage range will be severely limited.

Proposal 1: Postpone type 3 UE to future releases as the companies views are diverging and only 2 meetings are left on the Core part of the WI

Proposal 2: Focus on delivery of Type 2 and type 4 within this WI.

## Type-4 UE

Type-4 UEs have separate transmission paths similar to type 2 UE as shown in Table 2[3]. Due to the transmission path separation (No shared LNA between the CCs), there is no need to be concerned about the MRTD value and  MRTD= 33us can be specified, as it is the case for type2 UE[4]. Moreover a maximum of 25dB power imbalance between the received CCs similar to type 2 UEs can be set.

Table 2 UE architectures to be considered in Rel18

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

The UE shall indicate in a signaling its capability of non-collocated intra-band CA for type 4. It can use the RF requirements as specified in section 7.10A of TS 38.101-1 and MRTD requirements as Table 7.6.4-2 of TS 38.133.

Proposal 3: for type-4 UEs: Maximum power imbalance 25dB. MRTD= 33us.

Proposal 4: for type-4 UEs: introducing a UE capability with the RF requirements as specified in section 7.10A of TS 38.101-1 and MRTD requirements as Table 7.6.4-2 of TS 38.133.

.

# Conclusion

Observation 1: Following option 2 (MRTD [3]us) the MRTD is suggested to be a fixed value (3 us as an example), this fixed value should be large enough to include the TAE, the time budget for propagation delay and the channel delay spread

Observation 2: Following option 3 (MRTD < CP), the type-3 UEs in the cells where TAE> CP will not be able to use their non-collocated CA/EN-DC features. The other UEs that operate in cells where TAE ≤ CP, they will have a limited time budget for propagation delay and channel delay spread. Therefore, their coverage range will be severely limited.

Proposal 1: Postpone type-3 UE to future releases as the companies views are diverging and only 2 meetings are left on the Core part of the WI

Proposal 2: Focus on delivery of Type 2 and type 4 within this WI.

Proposal 3: for type-4 UEs: Maximum power imbalance 25dB. MRTD= 33us.

Proposal 4: for type-4 UEs: introducing a UE capability with the RF requirements as specified in section 7.10A of TS 38.101-1 and MRTD requirements as Table 7.6.4-2 of TS 38.133.

# References

[1]	“R4-2303696, WF on the requirement for intra-band non-collocated CA,EN-DC, KDDI.”

[2]	“R4-2302494, Power imbalance and MRTD in case of non-collocated intra-band NC CA, Huawei, HiSilicon.”

[3]	“R4-2215736, Views on UE RF aspect for non-collocated EN-DC, NR-CA deployment, Samsung.”

[4]	“R4-2220398, WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA, Huawei, HiSilicon, Samsung.”