3GPP TSG-RAN WG4 Meeting #106bis-e                                                    R4-2305660

Online, 17 Apr. – 26 Apr. 2023

Agenda Item:	5.19.1.2

Source:	Ericsson

Title:	Simulation results for phase I study of intra-cell inter-user interference cancellation

Document for:	Information

# 1	Introduction

After last RAN4 #106 meeting’s discussion in Athens, there were several parameter configurations and assumptions agreed that should be used for doing simulations for the phase I study. A way forward was approved to carry all the agreements and open issues [1].

In this contribution, we shared our simulation results based on the agreed parameter configurations and assumptions for information. Detailed result figures are also attached.

# 2	Discussion

## 2.1 Assumptions

In this section, we summarized the simulation assumptions that we used for our evaluations.

Following assumptions are assumed:

- Maximum transmission bandwidth configuration for all UEs

- Assume all scheduled DMRS ports have same QCL assumptions

- Assume the scrambling ID for DMRS sequence is the same for the target UE the co-scheduled UE(s)

- Random precoder for rank 1+1, orthogonal precoder for rank 2+2 and 1+3

- Same CDM group for rank 1+1, different CDM groups for rank 2+2, 1+3, 1+1+1

The following table is showing the general parameter configurations, which are basically reuse what was agreed in Rel-17 topic: intra-cell inter-user interference.

Table 2.1-1 General parameter configurations

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Number |  |  | 1 | 1 |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 for FDD |  |
| Duplex mode |  |  | FDD |  |
| MCS |  |  | 13/4/19 | QPSK/16QAM/64QAM |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 1 | 3 |
|  |  |  | 2 | 2 |
|  | DMRS port allocation |  | For rank 1+1: Port 1000 For rank 2+2: Port 1000 and 1001 For rank 1+3: | For rank 1+1: Port 1001 For rank 2+2: Port 1002 and 1003 For rank 1+3: |
| MIMO configuration |  |  | 2T2R, 4T4R with different correlations for verification |  |
| CDM groups without data |  |  | Same for rank 1+1 / different for rank 1+3, 2+2, 1+1+1 |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - For case with rank 1+1: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure that any column of precoding matrix of Co-scheduled UE is not equal to any column of precoding matrix of Target UE - For case with rank 2+2: Select the precoding matrix to ensure orthogonality with Target UE |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | TDLC300-100 / TDLA30-10 |  |

## 2.2 Test cases and results summary

We’ve evaluated the following test cases and summarized the results.

Table 2.2-1 Simulation cases and results

| Test num | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Rank (target UE + co-scheduled UE) | Correlation matrix and antenna configuration | Reference value SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | Baseline | E-IRC | RML |
| Rank 1+1 |  |  |  |  |  |  |  |  |  |
| 1 | 10 / 15 | 16QAM, 0.48 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA Low | 13.3 | 12.4 | 14.3 |
| 2 | 10 / 15 | 16QAM, 0.48 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA medium | 20.7 | 19.6(+1.1) | / |
| 2a | 10 / 15 | 16QAM, 0.48 | 16QAM | TDLC300-100 | 1+1 | 2x2, ULA low | 13.4 | 12.4 | 12.1 |
| 2b | 10 / 15 | 16QAM, 0.48 | 16QAM | TDLC300-100 | 1+1 | 2x2, ULA medium | 15.7 | 14.5 | 13.6 |
| 3 | 10 / 15 | QPSK (MCS4) | QPSK | TDLC300-100 | 1+1 | 2x2, ULA Low | 1.8 | 2.1 | 2.2 |
| 4 | 10 / 15 | QPSK (MCS4) | QPSK | TDLC300-100 | 1+1 | 2x2, ULA Medium | 3.1 | 3.7 | 3.2 |
| TDLA30-10 for rank 2+2 |  |  |  |  |  |  |  |  |  |
| 5 | 10 / 15 | 16QAM, 0.48 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Low | 12.9 | 11.7(+1.2) | 13.5(-0.6) |
| 6 | 10 / 15 | 16QAM, 0.48 | 16QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | 12.9 | 11.8(+1.1) | 12.3(+0.6) |
| 6a | 10 / 15 | 16QAM, 0.48 | 64QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | 12.9 | 11.7(+1.2) | 13.1(-0.2) |
| 7 | 10 / 15 | 16QAM, 0.48 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Medium | TBA | TBA | TBA |
| 8 | 10 / 15 | 16QAM, 0.48 | 16QAM | TDLA30-10 | 2+2 | 4x4, ULA Medium | / | / | / |
| 9 | 10 / 15 | MCS19 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Low | TBA | TBA | TBA |
| 9a | 10 / 15 | MCS19 | 16QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | TBA | TBA | TBA |
| 10 | 10 / 15 | MCS19 | 64QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | / | / | / |
| 11 | 10 / 15 | MCS19 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Medium | TBA | TBA | TBA |
| 11a | 10 / 15 | MCS19 | 16QAM | TDLA30-10 | 2+2 | 4x4, ULA Medium | TBA | TBA | TBA |
| 12 | 10 / 15 | MCS19 | 64QAM | TDLA30-10 | 2+2 | 4x4, ULA Medium | / | / | / |
| 13 | 10 / 15 | MCS4 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Low | 1.4 | 0.5(+0.9) | 1.5(-0.1) |
| 13a | 10 / 15 | MCS4 | 16QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | 1.3 | 0.5(+0.8) | 1.8(-0.5) |
| 14 | 10 / 15 | MCS4 | 64QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | TBA | TBA | TBA |
| 15 | 10 / 15 | MCS4 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Medium | 11.6 | 9.6 | 8.5 |
| 15a | 10 / 15 | MCS4 | 16QAM | TDLA30-10 | 2+2 | 4x4, ULA Medium | TBA | TBA | TBA |
| 16 | 10 / 15 | MCS4 | 64QAM | TDLA30-10 | 2+2 | 4x4, ULA Medium | TBA | TBA | TBA |
| TDLC300-100 for 2+2 |  |  |  |  |  |  |  |  |  |
| 17 | 10 / 15 | 16QAM, 0.48 | QPSK | TDLC300-100 | 2+2 | 4x4, ULA Low | TBA | TBA | TBA |
| 17a | 10 / 15 | 16QAM, 0.48 | 16QAM | TDLC300-100 | 2+2 | 4x4, ULA Low | 21.5 | 14.2 | 13.8 |
| 18 | 10 / 15 | 16QAM, 0.48 | 64QAM | TDLC300-100 | 2+2 | 4x4, ULA Low | TBA | TBA | TBA |
| 19 | 10 / 15 | 16QAM, 0.48 | QPSK | TDLC300-100 | 2+2 | 4x4, ULA Medium | TBA | TBA | TBA |
| 19a | 10 / 15 | 16QAM, 0.48 | 16QAM | TDLC300-100 | 2+2 | 4x4, ULA Medium | / | / | / |
| 20 | 10 / 15 | 16QAM, 0.48 | 64QAM | TDLC300-100 | 2+2 | 4x4, ULA Medium | TBA | TBA | TBA |

Note: “/” means max Tput can not be reached.

# 3	Summary

In this contribution, we shared our simulation results based on the parameter configurations and assumptions agreed in the last RAN4 #106 meeting.

# References

[1]	R4-2302929: WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #106