3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305666

Electronic meeting, Apr 17 – 26, 2023

Agenda item:	5.10.2.2

Source: 	Qualcomm Incorporated

Title: 	On joint requirements for Rel-17 measurement gap enhancements – Case1 (pre-configured MG + Type-2 MG)

Document for:	Discussion

# Introduction

RAN4 continued discussing joint RRM requirements for Rel-17 concurrent measurement gaps (MG), pre-configured measurement gaps and NCSG in RAN4#106. Further agreements and open issues were captured in a WF [1].

In this paper, we provide our views and proposals on joint requirements for the concurrent MG, pre-configured MG and NCSG within the context of Case 1 in the WID [2].

# Discussion

## Collisions

Let us start by discussing the topic of collisions between gaps in Case 1. It was agreed earlier by RAN4 that a pre-configured MG can cause collisions only when it is active [4]. Considering that a pre-configured MG does not cause any interruptions while it is deactivated, the existing agreement by RAN4 may seem reasonable. However, the agreement does create new challenges and complexity in the handling of gap collisions when the status of a pre-configured MG changes dynamically.

If the status of a pre-configured MG changes at some point in time, it may cause future gap instances of other concurrent gaps (Type-2 or another pre-configured MG) to be dropped or not to be dropped. Worse yet, a pre-configured MG could be activated during another on-going gap instance. If the newly activated pre-configured MG has higher priority and collides with the on-going gap, what should the UE (and network) do? Should the UE abort the current gap and drop the measurements? The UE may not have enough time to get ready for the new measurements in the colliding pre-configured MG occasion; that one occasion may be wasted if it’s not dropped.

Issue 3-3-4: [Case 1] dynamic collisions definition

< Agreement from online session>:

- Dynamic collisions are gap collisions involving at least one [activated] pre-configured MG, where gap instances of other MGs (which has lower priority) are dropped.

  - [activated] is based on the assumption that only activated Pre-MG can cause collisions.

Dynamic collisions are significantly different from collisions between concurrent gaps defined in Rel-17. In Rel-17, collisions between concurrent gaps are statically resolved when the gaps are configured. Any gap instances that need to be dropped can be identified at that time and the resulting gap patterns do not change until there is a change (via RRC) in the gap configuration. On the other hand, dynamic collisions would cause the gap patterns to change whenever the state of a pre-configured MG changes.

If we keep the current definition of collisions with pre-configured MG is kept, RAN4 would need to define additional rules to resolve all corner cases that may arise due to dynamic collisions. Given the additional complexity and effort, in our view RAN4 should consider modifying the existing agreement/definition.

To simplify the handling of gap collisions, it was proposed to define collisions involving a pre-configured MG independent of its status. i.e. pre-configured MG occasions may collide with other MGs even if the pre-configured MG is deactivated. This alternate definition avoids many issues since collisions would be resolved statically at RRC (re-)configuration.

Issue 3-3-1: [Case 1] Required changes for Pre-MG on collision

< Wayforward >:

- No consensus on whether to consider deactivated Pre-MG in a collision.

- FFS whether an additional capability is needed if collisions on Pre-MG is only considered when Pre-MG is activated.

Proposal 1: For Case 1, gap collisions are determined/resolved independent of the state of the pre-configured MG(s). i.e. both activated/deactivated pre-configured MGs can cause collisions.

If the above proposal is adopted there would be no dynamic collisions. All issues related to dynamic collisions could be closed right away.

Observation 1: If Proposal 1 is adopted, there would be no dynamic collisions and all related issues can be closed.

## Activation/deactivation

In RAN4#106, RAN4 reached agreements on the definitions of (non-)simultaneous activation/deactivation of pre-configured MGs and on supporting simultaneous activation/deactivation of pre-configured MGs subject to UE capability [1]. The definitions are reproduced below for convenience.

Issue 3-2-1: [Case 1] Define definitions for simultaneous and non-simultaneous Pre-MGs activation/deactivation for Pre-MG + Pre-MG

< Agreement >:

  - Definitions for simultaneous and non-simultaneous multiple Pre-MGs activation/deactivation

    - In simultaneous case, the multiple Pre-MGs activation/deactivation duration are fully or partially overlapping (before any potential delay extension) in time.

    - In non-simultaneous case, the multiple Pre-MGs activation/deactivation duration are not overlapping (before any potential delay extension) in time.

    - FFS the requirements, e.g., triggered by the same or different commands.

Based on the above definitions, we understand that if a UE does not support simultaneous activation/deactivation of pre-configured MGs in Case 1, then behavior of the UE would be undefined if that condition does occur. The burden would be on the network to avoid all situations that would lead to simultaneous activation/deactivation. In our view, UEs that do not support simultaneous activation/deactivation of pre-configured MGs may not be able to support pre-MG + pre-MG in Case 1.

Observation 2: For UEs that do not support simultaneous activation/deactivation of two pre-configured MGs in Case 1, it may not be feasible to support pre-MG + pre-MG combinations.

In our view, many, if not all, of the issues related to simultaneous activation/deactivation of pre-configured MGs in Case 1 can also be addressed (avoided) by updating the definition of collisions as in Proposal 1. Once collisions are resolved statically at RRC configuration time, essentially the two gap patterns become decoupled. The gap pattern for each gap would not change regardless of activation/deactivation status of the other gap. Each gap can be activated/deactivated without affecting the other. Also, it would be guaranteed that gap occasions from the two gap patterns are separated in time by at least 4 ms. Thus, the time separation mandated by the proximity rule would act as a buffer that effectively extends (delays) the activation time of one of the gaps if simultaneous activation does occur.

Issue 3-2-3: [Case 1] When the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion

< Wayforward >:

- FFS the options…

Issue 3-2-4: [Case 1] Whether to extend the delay for simultaneous activation/deactivation for Pre-MG+Pre-MG

< Wayforward >:

- FFS.

Proposal 2: Adopt the modified definition of collision in Proposal 1 and close the following two issues from RAN4#106

- Issue 3-2-3: [Case 1] When the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion

- Issue 3-2-4: [Case 1] Whether to extend the delay for simultaneous activation/deactivation for Pre-MG+Pre-MG

## Requirements

Measurement delays in Case 1 could change for some MOs if there are changes in the state of one or more pre-configured MGs. E.g. a MO may be measured without gaps when a pre-configured MG is deactivated but it may have to be measured within gap when the pre-configured MG is activated. This could happen regardless of whether Proposal 1 is adopted.

Issue 3-5-1: [Case 1] Measurement delay requirements

< Way forward >

- Option 1:

  - When gap combinations including pre-configured MGs (Case 1) are provided to the UE, measurement requirements do not apply if the following parameters change during the measurement period due to changes in the status of any pre-configured MGs:

    - Kp for intra-frequency and inter-frequency measurements without gaps

    - Kgap for intra-frequency and inter-frequency measurements with gaps

    - Kgap_EUTRA for inter-RAT measurements

    - Kp_CSI-RS for CSI-RS L3 measurements

    - Kp,PRS,i for NR positioning measurements

    - CSSFintra for intra-frequency measurements

    - CSSFinter for intra-frequency measurements

    - CSSFinterRAT for intra-RAT measurements

    - P scaling factor for L1-RSRP and L1-SINR measurements

- Option 2:

  - Measurement delay requirements should also consider the case of delayed Pre-MG activation due to overlap between Pre-MG and concurrent MG

The most straightforward way to address the issue mentioned above would be to specify that, when Case 1 MG combinations are configured, measurement requirements do not apply if a change in the state of any pre-configured MGs affects any of the requirement parameters during the measurement period. i.e. applicability of requirements is not precluded if the state of a pre-configured MG changes but does not affect the measurement delay of any MOs.

Proposal 3: When gap combinations including pre-configured MGs (Case 1) are provided to the UE, measurement requirements do not apply if the following parameters change during the measurement period due to changes in the status of any pre-configured MGs:

- Kp for intra-frequency and inter-frequency measurements without gaps

- Kgap for intra-frequency and inter-frequency measurements with gaps

- Kgap_EUTRA for inter-RAT measurements

- Kp_CSI-RS for CSI-RS L3 measurements

- Kp,PRS,i for NR positioning measurements

- CSSFintra for intra-frequency measurements

- CSSFinter for intra-frequency measurements

- CSSFinterRAT for intra-RAT measurements

- P scaling factor for L1-RSRP and L1-SINR measurements

# Conclusions

Proposal 1: For Case 1, gap collisions are determined/resolved independent of the state of the pre-configured MG(s). i.e. both activated/deactivated pre-configured MGs can cause collisions.

Observation 1: If Proposal 1 is adopted, there would be no dynamic collisions and all related issues can be closed.

Observation 2: For UEs that do not support simultaneous activation/deactivation of two pre-configured MGs in Case 1, it may not be feasible to support pre-MG + pre-MG combinations.

Proposal 2: Adopt the modified definition of collision in Proposal 1 and close the following two issues from RAN4#106

- Issue 3-2-3: [Case 1] When the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion

- Issue 3-2-4: [Case 1] Whether to extend the delay for simultaneous activation/deactivation for Pre-MG+Pre-MG

Proposal 3: When gap combinations including pre-configured MGs (Case 1) are provided to the UE, measurement requirements do not apply if the following parameters change during the measurement period due to changes in the status of any pre-configured MGs:

- Kp for intra-frequency and inter-frequency measurements without gaps

- Kgap for intra-frequency and inter-frequency measurements with gaps

- Kgap_EUTRA for inter-RAT measurements

- Kp_CSI-RS for CSI-RS L3 measurements

- Kp,PRS,i for NR positioning measurements

- CSSFintra for intra-frequency measurements

- CSSFinter for intra-frequency measurements

- CSSFinterRAT for intra-RAT measurements

- P scaling factor for L1-RSRP and L1-SINR measurements

# References

[1] R4-2303197, WF on Rel-18 NR MG enhancements, RAN4#106,

[2] RP-222337, Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, RAN#97-e.

[3] R4-2220359, WF on NR_MG_enh2 Part 1, RAN4#105.

[4] R4-2214346, WF on further enhancements on measurement gaps and measurements without gaps, RAN4#104-e.

# Appendix

Case 1 agreements from RAN4#106 [1].

Issue 3-1-1: [Case 1] Whether to consider a new capability for Pre-MG + Pre-MG in an FR

< Agreement from online session >:

- It is up to UE capability to support the simultaneous activation/deactivation of two Pre-MGs in the same FR.

Issue 3-1-2: [Case 1] Detail measurement gaps combinations for UE supporting per-FR gap

< Agreement >:

  - Gap combination configuration of case 1 can use gap combination of concurrent gaps defined in TS38.133 table 9.1.8-1 as formatting baseline with the clarification that each configured gap can be Pre-MG or Type-2 MG.

  - FFS details and notes.

Issue 3-2-1: [Case 1] Define definitions for simultaneous and non-simultaneous Pre-MGs activation/deactivation for Pre-MG + Pre-MG

< Agreement >:

  - Definitions for simultaneous and non-simultaneous multiple Pre-MGs activation/deactivation

    - In simultaneous case, the multiple Pre-MGs activation/deactivation duration are fully or partially overlapping (before any potential delay extension) in time.

    - In non-simultaneous case, the multiple Pre-MGs activation/deactivation duration are not overlapping (before any potential delay extension) in time.

    - FFS the requirements, e.g., triggered by the same or different commands.

Issue 3-2-2: [Case 1] Whether to support simultaneous and non-simultaneous Pre-MGs activation/deactivation for Pre-MG + Pre-MG

< Agreement >:

  - Follow agreement from issue 3-1-1.

Issue 3-2-5: [Case 1] Whether to extend the delay for non-simultaneous activation/deactivation for Pre-MG+Pre-MG

< Agreement >:

- For the non-simultaneous two Pre-MGs activation/deactivation case, the existing Rel-17 Pre-MG (de)activation delay requirements can be reused.

Issue 3-3-2: [Case 1] Whether to consider equal priority?

< Agreement >:

- No requirements are defined for equal priority in Pre-MG with concurrent gaps.

Issue 3-3-3: [Case 1] If equal priority is supported whether to consider gap sharing rule when the two gaps are with equal priority?

< Agreement >:

- Based on the outcome of issue 3-3-2, no further discussion is needed.

Issue 3-3-4: [Case 1] dynamic collisions definition

< Agreement from online session>:

- Dynamic collisions are gap collisions involving at least one [activated] pre-configured MG, where gap instances of other MGs (which has lower priority) are dropped.

  - [activated] is based on the assumption that only activated Pre-MG can cause collisions.

Issue 3-4-1: [Case 1] Explicit and implicit association

< Agreement >:

- RAN4 shall not define implicit association of intra-frequency layers with Pre-MG (i.e. the priority of a Pre-MG which concurrent with other gaps should be up to network assignment).

Issue 3-4-2: [Case 1] Pre-MG association clarification

Moderator’s note: This issue is currently being discussed in Rel-17 measurement gap enhancements during the maintenance phase, hence RAN4 should postpone the discussion on this issue until it is resolved in Rel-17 measurement gap enhancements.

< Agreement >:

- Postpone until this issue is resolved in Rel-17 measurement gap enhancements maintenance.