3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305669

Electronic meeting, Apr 17 – 26, 2023

Agenda item:	5.26.2.3

Source: 	Qualcomm Incorporated

Title: 	On requirements for Rel-17 MUSIM gaps – Network A

Document for:	Discussion

# Introduction

RAN4 continued discussing requirements for Rel-17 MUSIM gaps in RAN4#106. The latest agreements and open issues were captured in a WF [1].

In this paper, we will discuss our proposals for defining requirements in Network A.

# Discussion

Measurement requirements in network A need to be updated to account for any collisions between MUSIM gaps and measurement gaps or RS resources. Fortunately, RAN4 went through a similar exercise in Rel-17 when requirements were updated for scenarios where concurrent MG are configured.

It should be straightforward to extend the requirements to include collisions with MUSIM gaps. For example, the requirements for NR intra-frequency measurements without gaps (38.133, clause 9.2.5) define a scaling factor Kp to account for collisions with measurement gaps, as shown below.

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps,

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.2B.3.

Kp = 1 when Navailable = 0.

In the above definition, two quantities would need to be modified:

- The duration of the window W should be updated to include the maximum period of configured MUSIM gaps, if any.

- Navailable should be updated to also account for collisions between SMTC and MUSIM gaps within the window W, after accounting for collisions between MUSIM gaps and measurement gaps.

Issue 3-1-1: Principle on layer 1 and layer 3 measurement requirements after gap collision handling

- Proposals

  - P1: Frameworks of LBT failure in NR-U design can be used as starting point when discussing NW A L3/L1 requirement impact due to MUSIM gaps (Apple)

  - P2: Reuse the principle used in Rel-17 concurrent gaps WI as the baseline to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured. (CMCC xiaomi vivo ZTE oppo Huawei Ericsson Qualcomm MTK Nokia)

  - P3: Since principle used in Rel-17 concurrent gaps WI on layer 1 and layer 3 measurement requirements is derived after gap collision handling, it can be reused for even the gap handling solution within MUSIM gap is not fully determined, no need to postpone the discussion. (vivo)

  - P4: RAN4 to postpone the detail NW-A’s requirement discussion until RAN4 achieves the consensus on MUSIM gaps’ priority. (Ericsson)

Proposal 1: Reuse the principle used in Rel-17 concurrent gaps WI as the baseline to define L1/L3 measurement requirements in network A when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured.

Proposal 2: For intra-frequency and inter-frequency measurements without gaps in network A, modify the scaling factor Kp as follows:

- The duration of the window W is max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 3: For inter-frequency and inter-frequency measurements with gaps in network A, modify the scaling factor Kgap as follows:

- The duration of the window W is max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of SMTC occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 4: For inter-RAT measurements with gaps in network A, modify the scaling factor Kgap_EUTRA as follows:

- The duration of the window W is MGRP_max, where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within FR1

- Ntotal is the total number of associated measurement gap occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions

- Navailable is the number of non-dropped associated measurement gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 5: For CSI-RS L3 intra-frequency measurements without gaps modify the scaling factor Kp_CSI-RS as follows:

- The duration of the window W is max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer

- Ntotal is the total number of CSI-RS occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of CSI-RS occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 6: For CSI-RS L3 inter-frequency measurements with gaps modify the scaling factor Kp_CSI-RS as follows:

- The duration of the window W is max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer

- Ntotal is the total number of CSI-RS occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of CSI-RS occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 7: For NR positioning measurements with gaps modify the scaling factor Kp,PRS,i as follows:

- The duration of the window W is max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the positioning frequency layer

- Ntotal is the total number of associated measurement gap occasions covering PRS occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions

- Navailable is the number of non-dropped associated measurement gap occasions covering PRS occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 8: Clarify the definition of CSSFintra for intra-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 9: Clarify the definition of CSSFinter for inter-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 10: Clarify the definition of CSSFinterRAT for intra-RAT measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 11: For L1-RSRP and L1-SINR measurements modify the scaling factor P as follows:

- The duration of the window W is max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the serving cell

- Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Noutside_MG is the total number of SSB resource occasions that do not overlap with measurement gap occasions, MUSIM gap occasions nor SMTC occasions within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

The last issue is whether to take into account potential collisions and/or overlap with aperiodic MUSIM gaps in all the above requirements in network A. Since aperiodic MUSIM gaps should occur rather infrequently, it may not be necessary to define precisely how much extra delay would be incurred in case such conflicts occur. We do not expect a test case to be defined for such scenarios. It should be sufficient to say that the measurement delay/period can be longer.

Issue 3-1-3: On the time window W for aperiodic gap

- Proposals

  - P1: Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer. (vivo ZTE oppo Huawei)

  - P2: max(SMTC period, MGRP_max)+[M], where MGRP_max is the largest periodicity among all the periodic gaps and [M] is a time margin for the one-shot aperiodic MUSIM gap. (MTK)

    - P2-1: If P3 is used, the new window length proposed by P3 will only apply during the window where aperiodic gap locates. (vivo)

Proposal 12: Do not take into account aperiodic MUSM gaps in the detailed measurement period/delay requirements in network A. Clarify that the measurement period/delay can be longer in case of collisions or overlap between an aperiodic MUSIM gap and a measurement gap or RS measured without gap.

# Conclusions

Proposal 1: Reuse the principle used in Rel-17 concurrent gaps WI as the baseline to define L1/L3 measurement requirements in network A when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured.

Proposal 2: For intra-frequency and inter-frequency measurements without gaps in network A, modify the scaling factor Kp as follows:

- The duration of the window W is max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 3: For inter-frequency and inter-frequency measurements with gaps in network A, modify the scaling factor Kgap as follows:

- The duration of the window W is max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of SMTC occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 4: For inter-RAT measurements with gaps in network A, modify the scaling factor Kgap_EUTRA as follows:

- The duration of the window W is MGRP_max, where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within FR1

- Ntotal is the total number of associated measurement gap occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions

- Navailable is the number of non-dropped associated measurement gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 5: For CSI-RS L3 intra-frequency measurements without gaps modify the scaling factor Kp_CSI-RS as follows:

- The duration of the window W is max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer

- Ntotal is the total number of CSI-RS occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of CSI-RS occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 6: For CSI-RS L3 inter-frequency measurements with gaps modify the scaling factor Kp_CSI-RS as follows:

- The duration of the window W is max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer

- Ntotal is the total number of CSI-RS occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of CSI-RS occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 7: For NR positioning measurements with gaps modify the scaling factor Kp,PRS,i as follows:

- The duration of the window W is max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the positioning frequency layer

- Ntotal is the total number of associated measurement gap occasions covering PRS occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions

- Navailable is the number of non-dropped associated measurement gap occasions covering PRS occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 8: Clarify the definition of CSSFintra for intra-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 9: Clarify the definition of CSSFinter for inter-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 10: Clarify the definition of CSSFinterRAT for intra-RAT measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Proposal 11: For L1-RSRP and L1-SINR measurements modify the scaling factor P as follows:

- The duration of the window W is max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the serving cell

- Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Noutside_MG is the total number of SSB resource occasions that do not overlap with measurement gap occasions, MUSIM gap occasions nor SMTC occasions within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Proposal 12: Do not take into account aperiodic MUSM gaps in the detailed measurement period/delay requirements in network A. Clarify that the measurement period/delay can be longer in case of collisions or overlap between an aperiodic MUSIM gap and a measurement gap or RS measured without gap.

# References

[1] R4-2303310, WF on NR Dual TxRx Multi-SIM, RAN4#106

[2] 3GPP TS 38.331, v17.2.0