3GPP TGS-RAN WG4 Meeting #106Bis-e	 R4-2305680
Electronic Meeting, April 17th – 26th, 2023

Agenda item:	5.23.2

Source:	Intel Corporation

Title:	NR positioning for RedCap UEs – Switching time

Document for:	Discussion

# 1	Introduction

Discussions for the Rel-18 Expanded and improved NR positioning work item started during RAN4 #106 [1]. Based on the objectives in the WID [2], a work plan to facilitate contribution preparation was approved [3]. For this RAN4 meeting, the scope of RF discussions in the work plan includes the following:

| RAN4 Meeting | Tentative Work Plan |
| --- | --- |
| RAN4#106Bis-e April 2023 RF TUs: 0.5 | RF - PRS/SRS bandwidth aggregation - Consider conclusions in TR 38.859 and further discuss potential RF requirement impact and impairment modelling, if needed - Handle any incoming LS from RAN1 - Carrier phase positioning - Discuss findings of the RAN1 study on carrier phase positioning and identify potential impact to RF requirements, if any - If needed, send LS to RAN1 - RedCap UE positioning - Discuss RF considerations for frequency hopping based DL PRS reception and SRS for positioning transmissions for RedCap positioning, including handling any incoming LS from RAN1 |

For RedCap UE positioning, the approved WF captures discussing the RF impact of frequency hopping based on RAN1 input [4]. On this issue, RAN4 received an LS from RAN1 requesting feedback on feasible switching time values [5].

To RAN4

ACTION:

RAN1 kindly asks RAN4 for feedback on the feasible values for the switching time between hops, at least when numerology and bandwidth for each hop can be the same, and the Tx/Rx antennas used in all hops can be the same.

In this paper we discuss switching times and share our views on potential values.

# 2	Discussion

## 2.1	RedCap UE

The characteristics of RedCap devices place them at a mid-point between high-performance and low complexity applications in NR. To achieve this, RedCap UEs support several reduced complexity features, including narrower maximum UE bandwidths (20 MHz for FR1 and 100 MHz for FR2). As noted by companies in [6,7], the reduced bandwidth has a negative impact on the positioning accuracy of RedCap UEs. To offset this degradation, supporting frequency hopping beyond the maximum RedCap UE bandwidths was recommended as a conclusion of the Rel-18 positioning study item [8] and included as an objective in the WID [2].

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs)

  - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2].

    - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs.

  - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

## 2.2	Switching time

For the frequency hopping in the preceding section, the impact of the switching time between hops on performance was discussed and evaluated in RAN1 based on current requirements for SRS switching and BWP switching [6,7]. The conclusion reached in RAN1 #110Bis-e is captured below:

Agreement

For the evaluation of TX/RX frequency hopping for positioning of redcap UEs, the value of the gap between two consecutive hops includes at least from 100us to 5ms.

- Companies should indicate if other smaller values are used in their evaluations, and justify the feasibility of smaller values

Observation 1: Switching time impact on frequency hopping performance has been discussed in RAN1 and led to the following conclusion:

- Agreement

For the evaluation of TX/RX frequency hopping for positioning of redcap UEs, the value of the gap between two consecutive hops includes at least from 100us to 5ms.

Considering faster switching time and shorter RF retuning times are important to reduce performance degradation, RAN1 previously asked RAN4 for input on feasible values for the switching time between hops [5]. We will address this in the upcoming section.

LS on switching time for DL PRS or UL SRS frequency hopping

RedCap UEs may perform frequency hopping-based transmission of SRS and reception of PRS beyond their maximum UE bandwidth to compensate for positioning accuracy loss. RAN1 has asked for guidance on the feasibility of a RedCap UE to perform fast retuning to support this frequency hopping, assuming the numerology and bandwidth for each hop is the same [5]. Additionally, the following RAN1 agreements are included in the LS:

Conclusion

For positioning enhancements for RedCap UEs, only Rx frequency hopping of the DL PRS is supported.

Agreement

For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap

- FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG

- FFS: the use of a single or multiple instances of a MGs

- FFS: the use of PPW

Conclusion

The scope for RedCap positioning includes FR1 and FR2.

Agreement

For Positioning enhancements for redcap UEs for UL SRS Tx and DL PRS Rx frequency hopping, from the RAN1 perspective, short switching time to allow RF retuning between adjacent hops may be beneficial in terms of accuracy and latency performance.

- Send an LS to RAN4 requesting feedback on the feasible values for the switching time between hops, at least when numerology and bandwidth for each hops can be the same, and the Tx/Rx antennas used in all hops can be the same.

Agreement

For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource

- FFS: whether there is specification update needed for RAN1

- FFS: remaining details

Agreement

For RedCap UEs, support SRS for positioning frequency hopping by

- Using a configuration separate from the existing BWP configuration

  - FFS: hopping is configured within a SRS resource or across SRS resources

Observation 2: RAN4 has been asked to assess feasible values for the switching time between hops, considering the following:

- Numerology and bandwidth for each hop can be the same

- Tx/Rx antennas used in all hops can be the same

- Existing BWP configuration will not be used to support SRS for positioning frequency hopping

From Observation 2, we can deduce that the main aspect to consider in RF retuning is a change in center frequency. To aid RAN4 assessment on feasible values for the switching time between hops, we provide a summary of previous switching time discussions in the next section.

Switching time discussion summary

Through the years, RAN4 has discussed time gaps, switching times and RF retuning times as part of several items, including eMTC, SRS switching time and NR UL/SUL switching. Since RedCap characteristics are bounded by high-performance and low complexity, it makes sense to consider MTC as the minimum performance to be met. From there, we summarize relevant discussions leading up to recent candidate values for SRS switching time when transmitting outside initial UL BWP or with different SCS [8], which were considered by RAN1 in their agreement for the value of the gap between two consecutive hops. Observation 3 provides summarized highlights for these discussions.

Observation 3: The outcome of relevant RAN4 discussions for switching and RF retuning times is summarized below:

- MTC RF retuning time (Rel-13 eMTC)  can serve as minimum baseline

- RAN4 conclusion on retuning time: maximum retuning time between narrowband regions is 2 symbols including CP length assuming normal CP

- UE RF bandwidth adaptation [10]

- FR1 intra-band with center frequency change: transition time is 50 – 200µs

- FR1 inter-band: transition time up to 900µs

- NR SRS carrier switching time [11] [12]

- Intra-band CA: 0µs, 30µs, 100µs, 140µs and 200µs

- Inter-band CA: 0µs, 30µs, 100µs, 200µs, 300µs, 500µs and 900µs

- NR UL/SUL switching (UL sharing in EN-DC)

- When SUL is not the same band as LTE or there is more than one LTE, NR or SUL band, 140µs switching time applies between NR UL and NR SUL

- Switching time candidate values when transmitting SRS outside initial UL BWP or with different SCS than initial BWP [8]

- {100µs, 140µs, 200µs, 300µs, 500µs}

Assessment of feasible values

Unlike BWP switching, for frequency hopping the RedCap UE would not be expected to transmit/receive other channels/signals. Therefore, the operations would be much lighter compared to Rel-15 BWP switching. Additionally, mainly the center frequency will change and as captured in Observation 3, the transition time for UE RF bandwidth adaptation of FR1 intra-band ranges from 50 to 200µs. Note that what value is Therefore, we think it is reasonable to have a max RF retuning time of 2 symbols as a starting point baseline.

Observation 4: Given the lighter load compared to BWP switching, the SRS carrier switching time values for intra-band CA and the UE RF bandwidth adaptation retuning time range of 50-200µs, it is reasonable to have the switching time for FR1 not exceed the MTC retuning time of 2 symbols of 15kHz SCS (140µs).

Proposal 1: Further discuss defining the switching time for SRS frequency hopping based on a UE capability with values up to 140µs for both FR1 and FR2.

# 3	Conclusions

In this paper, we provided our views on potential values for the switching time between hops. The following proposal and observations were made:

Observation 1: Switching time impact on frequency hopping performance has been discussed in RAN1 and led to the following conclusion:

- Agreement

For the evaluation of TX/RX frequency hopping for positioning of redcap UEs, the value of the gap between two consecutive hops includes at least from 100us to 5ms.

Observation 2: RAN4 has been asked to assess feasible values for the switching time between hops, considering the following:

- Numerology and bandwidth for each hop can be the same

- Tx/Rx antennas used in all hops can be the same

- Existing BWP configuration will not be used to support SRS for positioning frequency hopping

Observation 3: The outcome of relevant RAN4 discussions for switching and RF retuning times is summarized below:

- MTC RF retuning time (Rel-13 eMTC)  can serve as minimum baseline

- RAN4 conclusion on retuning time: maximum retuning time between narrowband regions is 2 symbols including CP length assuming normal CP

- UE RF bandwidth adaptation [10]

- FR1 intra-band with center frequency change: transition time is 50 – 200µs

- FR1 inter-band: transition time up to 900µs

- NR SRS carrier switching time [11] [12]

- Intra-band CA: 0µs, 30µs, 100µs, 140µs and 200µs

- Inter-band CA: 0µs, 30µs, 100µs, 200µs, 300µs, 500µs and 900µs

- NR UL/SUL switching (UL sharing in EN-DC)

- When SUL is not the same band as LTE or there is more than one LTE, NR or SUL band, 140µs switching time applies between NR UL and NR SUL

- Switching time candidate values when transmitting SRS outside initial UL BWP or with different SCS than initial BWP [8]

- {100µs, 140µs, 200µs, 300µs, 500µs}

Observation 4: Given the lighter load compared to BWP switching, the SRS carrier switching time values for intra-band CA and the UE RF bandwidth adaptation retuning time range of 50-200µs, it is reasonable to have the switching time for FR1 not exceed the MTC retuning time of 2 symbols of 15kHz SCS (140µs).

Proposal 1: Further discuss defining the switching time for SRS frequency hopping based on a UE capability with values up to 140µs for both FR1 and FR2.

# 4	References

- R4-2302832, “Topic summary for [106][139] FS_NR_pos_UERF,” Moderator (Intel), RAN4 #106, February 2023

- RP-223549, “New WID on Expanded and Improved NR Positioning,” Intel Corporation, CATT, Ericsson, RAN #98e, December 2022

- R4-2303569, “RAN4 Work plan for core requirements of WI on Expanded and Improved NR Positioning,” Intel Corporation, Ericsson, CATT, RAN4 #106, February 2023

- R4-2303570, “WF on UE RF for expanded and improved NR positioning,” Intel Corporation, RAN4 #106, February 2023

- R1-2302127, “LS on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs,” RAN1 #112, February 2023

- R1-2210476, “Feature Lead Summary #3 for Positioning for RedCap UEs,” Moderator (Ericsson, RAN1 #110Bis-e, October 2022

- R1-2212603, “Feature lead summary #3 for Positioning for RedCap UEs,” Ericsson, RAN1 #111, November 2022

- R4-2210604, “LS on switching time for SRS transmission outside initial UL BWP in RRC_INACTIVE,” RAN4 #103e, May 2022

- 3GPP TR 38.859, v18.0.0, December 2022

- R4-1702029, “[Draft] Reply LS on UE RF Bandwidth Adaptation in NR,” MediaTek, RAN4 #82, February 2017

- 3GPP TS 38.331, v17.4.0, March 2023

- 3GPP TS 38.133, v18.1.0, April 2023