3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2305692

Online, April 17 – April 26, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	Missing general and specific requirements for PC1.5 intra-band ULCA ul configuration request for inter-band HPUE

Agenda Item:	4.18.1	Rapporteur input	[HPUE_NR_FR1_TDD_intra_CA_R18]

Document for:	Approval

# Introduction

In RAN#99, new inter-band HPUE band combinations were requested with PC1.5 contiguous intra-band ULCA as UL configuration. In this contribution we detail the missing generic and specific requirements needed to enable this type of combination.

# Discussion

## Band combinations with PC1.5 contiguous ULCA

In RAN#99 the revised WI HPUE_NR_FR1_TDD_intra_CA_R18 [1] included the following band combination requests.

Table 1: HPUE band combinations with PC1.5 ULCA configuration

| CA_n77C | CA_n77C | PC1.5 | New |
| --- | --- | --- | --- |
| CA_n41(A-C) | CA_n41C | PC1.5 | New |
| CA_n41C | CA_n41C | PC1.5 | New |

Currently, intra-band contiguous ULCA requirements like MPR or A-MPR are only specified for PC3 (from release 16) and PC2 (from release 17).

There is no MPR or A-MPR specified for PC1.5. It should be feasible to develop these requirements based on the PC2 2Tx cases with 1LO and reuse some of the contiguous and non-contiguous allocations definitions. However, this work was performed under the FR1 enhanced non-spectrum WI in R16 and R17 and we are now unsure if these generic requirements can be developed within a spectrum HPUE basket. It should be noted that PC2 n41C NS04 A-MPR is not yet specified for all types of allocations.

This MPR and A-MPR work is intensive and required contributions from multiple companies based on example bands and it is unclear how this work is conducted in a spectrum HPUE WI, especially if more cases are added that need a band-specific A-MPR.

Proposal:

- MPR for PC1.5 contiguous ULCA contiguous and non-contiguous allocations needs to be evaluated and specified

- RAN4 should clarify if this general requirement work can be conducted under the spectrum WI HPUE_NR_FR1_TDD_intra_CA_R18

  - If so n41C and n77C are used as example bands

- After MPR is developed NS_04 A-MPR can be specified for PC1.5 together with the missing PC2 specification.

  - If a similar UL configuration is needed for Japan, NS_47 may be further needed

- The only applicable architecture is 2Tx with 1 LO transmitter

- The inter-band combinations with UL configuration PC1.5 n77C cannot be completed until the related MPR is specified

- The inter-band combinations with UL configuration PC1.5 n41C cannot be completed until the related MPR and NS_04 A-MPR is specified

  - If a similar UL configuration is needed for Japan, NS_47 may be further needed

- The WI objectives are updated to precisely describe the amount of work needed as today’s description is very succinct and does not represent properly the work involved.

# Conclusions

In this contribution, we point at the missing requirements needed for the support of n41C and n77C PC1.5 UL configurations and make the following proposal.

Proposal:

- MPR for PC1.5 contiguous ULCA contiguous and non-contiguous allocations needs to be evaluated and specified

- RAN4 should clarify if this general requirement work can be conducted under the spectrum WI HPUE_NR_FR1_TDD_intra_CA_R18

  - If so n41C and n77C are used as example bands

- After MPR is developed NS_04 A-MPR can be specified for PC1.5 together with the missing PC2 specification.

  - If a similar UL configuration is needed for Japan, NS_47 may be further needed

- The only applicable architecture is 2Tx with 1 LO transmitter

- The inter-band combinations with UL configuration PC1.5 n77C cannot be completed until the related MPR is specified

- The inter-band combinations with UL configuration PC1.5 n41C cannot be completed until the related MPR and NS_04 A-MPR is specified

  - If a similar UL configuration is needed for Japan, NS_47 may be further needed

- The WI objectives are updated to precisely describe the amount of work needed as today’s description is very succinct and does not represent properly the work involved.

# References

- RP-230479 WID on High power UE (power class 1.5 or 2) for intra-band Carrier Aggregation combinations of a single NR FR1 TDD band, Huawei, HiSilicon, RAN#99