3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305709

Online, April 17th – 26th, 2023

Agenda Item:	5.20.2.2.4

Source:	MediaTek Inc.

Title: 	On feasibility of FR2 UE aspect

Document for:	Discussion

# Introduction

RAN4 continues the study on duplex evolution, and from UE aspects, a way forward was agreed in RAN4#106[1]. One open issue related to feasibility of FR2 UE is shown as following:

- For sub-band/in-channel selectivity: - For FR2-1 companies are encouraged to further discuss values in the range of [20 to 34 dB] for sub-band/in-channel selectivity with accompanying clarification as how they calculate DL subband interference based on one value from this range and what guard band is assumed.

# Discussion

## 2.1 Sub-band selectivity

To check sub-band selectivity for a victim DL Rx UE interfered by an aggressor UL Tx UE nearby, the victim DL Rx UE receives a certain input level, and the aggressor transmits at its maximum output power. We can start with the general in-band emission requirements as specified in TS38.101-2 by dropping both IQ image and carrier leakage [4] for the aggressor UE:

| Parameter description | Unit | Limit (NOTE 1) | Applicable Frequencies |
| --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB | Any non-allocated (NOTE 2) |

Let I1=-25 -10.log10NRBLCRB, I2=20.log10EVM-5.|∆RB-1LCRB, and I3=-55.1dBm-PRB ,and IBE = max (I1,I2, I3).

And take one example where:

- SCS = 60kHz,

- CBW = 200MHz, and allocated UL BWP=40MHz at the outermost edge (e.g., NRB = 264, LCRB = 53, and |∆RB|=1,2…, 264-53=211),

- EVM=12.5% for 16QAM,

- maximum Tx power P_tx = 23dBm

Then we can calculate:

- PRB=23-10*log10LCRB=23-17.24=5.76 dBm

- I1=-25-10lg264LCRB=-25-10lg26453=-31.97

- I3=-55.1dBm-5.76=-60.86

- for I2, when |∆RB|=1 it becomes max which is -18.06. As an approximation, we could treat I2 as the dominant component of IBE, i.e., IBE= -18.06 - 5.|∆RB-1LCRB .

Note that this is in dB domain, to compute the total received interference generated by the UL BWP, it should be converted to linear domain and make integration as its approximation:

Itotal-DL-BWP=1NRB-LCRB10-11018.06+510*LCRBx-1dx

Itotal-DL-BWP-dB=10*log10Itotal-DL-BWP=-1.48

It is equivalent to a selectivity of Ptx-Itotal-DL-BWP-dB=23-(-1.48)=24.48 dB, and this can be rounded to half dB as 24.5dB for the sub-band selectivity.

Proposal 1: For FR2-1, RAN4 to select 24.5 dB for sub-band/in-channel selectivity.

# Conclusion

In this contribution we have the following proposal for feasibility of FR2 UE aspect for SBFD:

Proposal 1: For FR2-1, RAN4 to select 24.5 dB for sub-band/in-channel selectivity.

# References

- R4-2302977 WF for SBFD feasibility study and requirements impact from UE aspect, Qualcomm.

- R4-2303755, Topic summary for [106][310] FS_NR_duplex_evo_Part1

- TS 38.101-1 v17.8.0

- TS 38.101-2 v17.8.0