3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305711

Online, April 17th – 26th, 2023

Agenda Item:	8.1

Source:	MediaTek Inc.

Title: 	Discussion on standardization for RedCap PC2

Document for:	Discussion

# Introduction

In RANP#99, a new WI proposal was submitted for RedCap PC2 [1], and during the offline discussion, it was agreed that RAN4 study the following aspects and come back in the next plenary:

- RAN task RAN4 the following work in the next quarter and come back to RAN#100 - To study how to enable HPUE including PC2 for RedCap in the band agnostic way - To study the applicable form factor, e.g., sensor, camera

In this contribution, we discuss these two aspects respectively for the support of higher power class for RedCap.

# Discussion

## 2.1 RedCap PC3 in specs

In Rel-17, RedCap is supported only for PC3, and an suffix “I” is created for some clauses. The table below summarizes how RedCap is supported for PC3:

| Sub-clauses | RedCap PC3 |
| --- | --- |
| System parameters |  |
| 5.2I Operating bands for RedCap | Missing due to un-resolved controversies |
| 5.3I UE channel bandwidth for RedCap | Specified: ## 5.3I Channel bandwidth for RedCap The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 with maximum channel bandwidth of 20MHz. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths up to 20 MHz. The channel bandwidths are specified for both the TX and RX path. |
| 5.4I Channel arrangement for RedCap | - |
| 5.5I Configurations for RedCap | - |
| UE RF requirements |  |
| 6.1I General | Missing |
| 6.2I.1 UE maximum output power | Specified, indicating PC3 specified in 6.2.1 apply ## 6.2I Transmitter power for RedCap ### 6.2I.1 Maximum output power for RedCap For Redcap UE, the requirements for power class 3 specified in clause 6.2.1 apply. Note: ” 6.2.1I Void” should be removed? |
| 6.2I.2 MPR | - Note: power class related |
| 6.2I.3 A-MPR | - Note: power class related |
| 6.2I.4 Configured transmitted power | - Note: power class is one of the input parameters |
| 6.3I.1 Minimum output power | - |
| 6.3I.2 Transmit OFF power | - |
| 6.3I.3 Transmit ON/OFF time mask | - |
| 6.3I.4 Power control | - |
| 6.4I.1 Frequency error | - |
| 6.4I.2.1 EVM | - |
| 6.4I.2.2 Carrier leakage | - |
| 6.4I.2.3 In-band emissions | - |
| 6.4I.2.4 EVM equalizer spectrum flatness | - |
| 6.5I.1 Occupied bandwidth | - |
| 6.5I.2.2 Spectrum emission mask | - |
| 6.5I.2.3 Additional Spectrum emission mask | - |
| 6.5I.2.4.1 NR ACLR | - Note: Power class related |
| 6.5I.2.4.2 UTRA ACLR | - Note: only PC3 |
| 6.5I.3.1 General spurious emissions | - |
| 6.5I.3.2 Spurious UEtoUE co-ex | - |
| 6.5I.3.3 Additional spurious emissions | - |
| 6.5I.4 Transmit intermodulation | - |
| 7.1I General | Specified: ## 7.1I General For a Redcap UE the requirements in Section 7 shall be verified with the channel bandwidth up to 20MHz and REFSENS specified in clause 7.3I. |
| 7.2I Diversity characteristics | Specified in 7.2 instead |
| 7.3I.2 Reference sensitivity | Specified: relaxation & HD-FDD |
| 7.4I Max input level | Covered by 7.1I |
| 7.5I Adjacent channel selectivity | Covered by 7.1I |
| 7.6I.2 In-band blocking | Covered by 7.1I |
| 7.6I.3 Out of band blocking | Covered by 7.1I |
| 7.6I.4 Narrow band blocking | Covered by 7.1I |
| 7.7I Spurious response | Covered by 7.1I |
| 7.8I Intermodulation characteristics | Covered by 7.1I |
| 7.9I Spurious emissions | Covered by 7.1I |

From the table, we can observe that some improvements on RedCap PC3 specification may be needed.

Proposal 1: RAN4 to make necessary improvements on RedCap PC3 specification in the current RAN4 specs.

## 2.2 Enabling HPUE including PC2 for RedCap

As stated in [1], there is a strong commercial demand to extend cell coverage for NR deployment as well as improve the data rate of cell edge RedCap UE, in particular in a typical deployment where RedCap UE and non-RedCap normal UE are operated in the same network and on the same carrier, thus increasing the maximum transmit power from PC3 23dBm to PC2 26dBm or higher for RedCap UE is of strong interests.

From RAN4 specs perspective, enabling PC2 support for RedCap may require to revisit the following subclauses on both system parameters and ideally:

- 5.2I

- 6.1I

- 6.2I.1

- 7.3I.2

Proposal 2: To enable PC2 support for RedCap, RAN4 to at least revisit 5.2I, 6.1I, 6.2I.1 and 7.3I.2 in the RAN4 specs.

## 2.3 Applicable/target form factors

Since 1Tx is assumed in a RedCap UE, it may bring more challenges on the implementation with the increase of its power class. This may impose some restrictions on the applicable form factor. In particular, wearables have a very limited size, which may not be suitable for high power RedCap, therefore, we propose to focus on non-wearables for HPUE for RedCap.

Proposal 3: RAN4 to focus on RedCap PC2 for form-factors of non-wearables.

# Conclusion

In this contribution we have the following proposals for standardization of RedCap PC2:

Proposal 1: RAN4 to make necessary improvements on RedCap PC3 specification in the current RAN4 specs.

Proposal 2: To enable PC2 support for RedCap, RAN4 to at least revisit 5.2I, 6.1I, 6.2I.1 and 7.3I.2 in the RAN4 specs.

Proposal 3: RAN4 to focus on RedCap PC2 for form-factors of non-wearables.

# References

- RP-230312	New WID: NR power class 2 RedCap (Reduced Capability) UE in FR1,	China Telecom, MediaTek

- RP-230763 Summary of offline drafting discussions for RAN4 spectrum-related WI proposals, RAN WG4 Chair.