3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2305735

Athens, Greece, February 27 – March 3, 2023

Agenda Item:	6.8.6

Source:	Ericsson

Title:	Discussions on NTN IoT RRM performance requirements

Document for:	Discussion

# 1	Introduction

Good progress was reached at previous RAN4 meeting on the performance part of the IoT NTN WI. For example, the test cases and work-split were agreed. However, there are still some open issues remain to be resolved which are discussed in this contribution.

# 2	Discussion

Measurement accuracy

RAN4 has agreed to reuse the legacy measurement accuracy delay requirements for NB-IoT and eMTC. It is noted that the supported bands for NB-IoT and eMTC over NTN are different compared to the NB-IoT and eMTC over TN.

As an example, the legacy measurement accuracy requirements for eMTC are defined for set of frequency bands as shown in Table 1, see below [2]:

Table 1: RSRP Intra frequency absolute accuracy for UE category M1 with CE mode A for FDD and TDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 7 | 10 | -6 dB | FDD-M1_A, TDD-M1_A | -121 | N/A | -70 |
|  |  |  | FDD-M1_B | -120.5 | N/A | -70 |
|  |  |  | FDD-M1_D | -119.5 | N/A | -70 |
|  |  |  | FDD-M1_E, TDD-M1_E | -119 | N/A | -70 |
|  |  |  | FDD-M1_F | -118.5 | N/A | -70 |
|  |  |  | FDD-M1_G | -118 | N/A | -70 |
|  |  |  | FDD-M1_N | -114.5 | N/A | -70 |
| 9 | 12 | -6 dB | FDD-M1_A, TDD-M1_A, FDD-M1_D, FDD-M1_E, TDD-M1_E, FDD-M1_F, FDD-M1_G, FDD-M1_N | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

But eMTC over NTN is defined for different set of frequency bands as shown in Table 2 below [2]:

Table 2: Band groups for category M1 for satellite access

| Group | E-UTRA FDD |  |
| --- | --- | --- |
|  | Band group notation | Operating bands |
| A | FDD-M1_SAB_A | 255 |
| B | FDD-M1_SAB_B | 256 |
| C | FDD-M1_SAB_C |  |
| D | FDD-M1_SAB_D |  |
| E | FDD-M1_SAB_E |  |
| F | FDD-M1_SAB_F |  |
| G | FDD-M1_SAB_G |  |
| H | FDD-M1_SAB_H |  |
| I | FDD-M1_SAB_I |  |
| J | FDD-M1_SAB_J |  |
| K | FDD-M1_SAB_K |  |
| L | FDD-M1_SAB_L |  |
| M | FDD-M1_SAB_M |  |
| N | FDD-M1_SAB_N |  |

Therefore, even though the measurement accuracy requirements are same RAN4 still needs to define the accuracy requirements in separate clause with separate tables containing the IoT NTN specific frequency bands.

Observation 1: The supported frequency bands for eMTC and NB-IoT over NTN are different from frequency bands of eMTC and NB-IoT over TN.

- RAN4 to define the measurement accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands.

Similarly, the channel quality reporting accuracy requirements defined in clause 9.1.21 and 9.1.22 of TS 36.133 cannot be directly reused for IoT NTN since they listed frequency band groups are different. Therefore, RAN4 needs to define new channel quality reporting accuracy requirements IoT NTN.

- RAN4 to define the channel quality reporting accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands.

GNSS positioning acquisition method

Following open issue related to GNSS position acquisition method remains to be resolved [1]:

#### Issue 6: GNSS Position Acquisition Method - FFS whether to follow the same approach as NR NTN for GNSS Position Acquisition Method for UE transmit timing test cases.

We support the option of following the same approach as in NR NTN for GNSS positioning acquisition since the scenario and background are similar. This would allow reusing of the method from corresponding NR NTN test cases and simplifies the work.

- RAN4 to follow the same approach as NR NTN for GNSS Position Acquisition Method for UE transmit timing test cases.

PHR reporting

New power headroom reporting requirements were introduced in section 9.1.23 in TS 36.133 for NB-IoT in LTE. The motivations for introducing the new requirements were the multiple coverage levels and new power class that the NB-IoT UE could operate in, and mainly considering the limited number of reportable values compared to number of reportable values of a legacy LTE UE. More specifically, the NB-IoT UE has only 2 bits that can be used for reporting the power headroom, and this is considerably fewer values compared to the 6 bits used for PHR reporting by the legacy LTE UEs or cat-M UEs. This means that the NB-IoT UE can report only 4 different values while the cat-M or LTE UE can report up to 64 values for power headroom which gives more flexibility to adjust the reporting values to better reflect the actual power usage in the UE.

Table 1 shows the PHR reporting values in legacy requirements for normal coverage[2]:

Table 3: Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR [31] when the enhanced coverage level 0 is selected during random access procedure [17]

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH 5 |
| POWER_HEADROOM_1 | 5 PH 8 |
| POWER_HEADROOM_2 | 8 PH 11 |
| POWER_HEADROOM_3 | PH ≥ 11 |

One important difference between GEO and LEO is in the altitude they are operating at. The geostationary satellites operate around 36 000 km above the earth while the LEO satellites operate at altitude less than 1 200 km above the earth.

Observation 2: GEO satellites operate around 36 000 km above the earth while LEO satellites operates around 1 200 km above the earth.

The NTN network is operated by different types of satellites which have different characteristics in terms altitudes, speed, orbit etc. For example, the LEO and GEO have different characteristics in terms of altitude and mobility. The pathloss (PL) depends on the type of satellite serving the cell and leads to different power usage for the UL transmissions. The UE uplink transmit power is related to the operating altitude of the satellites, e.g. the UE may need to use more uplink transmit power or repetitions for reaching a satellite at 36 00 km above the earth compared to reaching a satellite above 16 000 km above the earth. This means, for a UE with only 4 reportable values, it is reasonable to assume that more negative values are needed to report for GEO compared to LEO since the UE is expected to operate at higher power for GEO compared to LEO. Therefore, we propose to modify the values in exiting tables for normal and enhanced coverage respectively to include more negative PHR reporting values. Specific examples are shown below.

- For UE in normal coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 4.

Table 4: Power headroom report mapping for UE category NB1 UEs for a cell (e.g. cell1) served by GEO satellite during random access procedure

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

- For UE in normal coverage managed by LEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 5.

Table 5: Power headroom report mapping for UE category NB1 UEs for a cell (e.g. cell1) served by LEO satellite during random access procedure

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH 5 |
| POWER_HEADROOM_1 | 5 PH 8 |
| POWER_HEADROOM_2 | 8 PH 11 |
| POWER_HEADROOM_3 | PH ≥ 11 |

- For UE in enhanced coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 6.

Table 6 Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure and UE is served by GEO satellite

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -30 |
| POWER_HEADROOM_1 | -30 PH -20 |
| POWER_HEADROOM_2 | -20 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

- For UE in enhanced coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 7.

Table 7: Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure and UE is served by LEO satellite

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

- For UE category M1 operation with GSO and NGSO, the legacy UE category M1 PHR reporting tables are reused.

# Conclusion

In this contribution we have provided discussed the performance requirements for NB-IoT and eMTC NTN. Based on the discussions, we have made following agreements and observations:

Observation 1: The supported frequency bands for eMTC and NB-IoT over NTN are different from frequency bands of eMTC and NB-IoT over TN.

- RAN4 to define the measurement accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands.

- RAN4 to define the channel quality reporting accuracy requirements for NB-IoT and eMTC in separate clause containing the IoT NTN specific frequency bands.

- RAN4 to follow the same approach as NR NTN for GNSS Position Acquisition Method for UE transmit timing test cases.

Observation 2: GEO satellites operate around 36 000 km above the earth while LEO satellites operates around 1 200 km above the earth.

- For UE in normal coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 4.

- For UE in normal coverage managed by LEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 5.

- For UE in enhanced coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 6.

- For UE in enhanced coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 7.

- For UE category M1 operation with GSO and NGSO, the legacy UE category M1 PHR reporting tables are reused.

# References

R4-2303311, "WF on NB-IoT/eMTC RRM requirements for NTN ", Moderator (MediaTek inc.)

TS 36.133 V18.1.0