3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2305782

Online, April 17 – April 26, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	Issues with invalid 2UL-3CC UL configurations requests

Agenda Item:	4.1.1.1 - band combinations with UL configurations including intra-band ULCA with IMD or triple beat issues [WI code]

Document for:	Approval

# Introduction

In RAN#99, new inter-band combinations were 2UL configuration using non-contiguous intra-band ULCA in one band were requested in [1]. In this contribution we highlight implementation issues and missing generic and band-specific requirements to enable these types of combination.

# Discussion

## Three UL cluster UL configurations

In RAN#99, two band three UL clusters UL configurations were requested in [1] reproduced in Table 1:

- These combinations are invalid as current agreements only covers up to 2 separate clusters

- Only nXXA-nYY C or B are valid, nXXA-nYY(2A) are invalid.

Table 1: Invalid 2UL/3CC UL configurations

| DL configuration | Invalid UL configuration and valid alternative |
| --- | --- |
| CA_n2(2A)-n7A | CA_n2(2A)-n7A => CA_n2A-n7A or CA_n2(2A) |
| CA_n2(2A)-n41A | CA_n2(2A)-n41A => CA_n2A-n41A or CA_n2(2A) |
| CA_n2(2A)-n71A | CA_n2(2A)-n71A => CA_n2A-n71A or CA_n2(2A) |
| CA_n2A-n77(2A) | CA_n2A-n77(2A) => CA_n2A-n77A or CA_n77(2A) |
| CA_n2A-n66(2A) | CA_n2A-n66(2A) => CA_n2A-n66A or CA_n66(2A) |
| CA_n7(2A)-n78A | CA_n7(2A)-n78A => CA_n7A-n78A or CA_n7(2A) |

Proposal:

- UL configurations CA_n46(2A) and CA_n102(2A) are not valid in Release 18 (R17 agreement)

- Band combinations using these UL configurations should be changed to alternative 2UL/2CC or 1UL/2CC

  - CA_n2(2A)-n7A modified to CA_n2A-n7A or CA_n2(2A)

  - CA_n2(2A)-n41A modified to CA_n2A-n41A or CA_n2(2A)

  - CA_n2(2A)-n71A modified to CA_n2A-n71A or CA_n2(2A)

  - CA_n2A-n77(2A) modified to CA_n2A-n77A or CA_n77(2A)

  - CA_n2A-n66(2A) modified to CA_n2A-n66A or CA_n66(2A)

  - CA_n7(2A)-n78A modified to CA_n7A-n78A or CA_n7(2A)

- The WI and band combination spreadsheet is revised accordingly.

# Conclusions

In this contribution, we examined the requested UL configuration for 2 band NRCA and found invalid configurations and make the following proposal.

Proposal:

- UL configurations CA_n46(2A) and CA_n102(2A) are not valid in Release 18 (R17 agreement)

- Band combinations using these UL configurations should be changed to alternative 2UL/2CC or 1UL/2CC

  - CA_n2(2A)-n7A modified to CA_n2A-n7A or CA_n2(2A)

  - CA_n2(2A)-n41A modified to CA_n2A-n41A or CA_n2(2A)

  - CA_n2(2A)-n71A modified to CA_n2A-n71A or CA_n2(2A)

  - CA_n2A-n77(2A) modified to CA_n2A-n77A or CA_n77(2A)

  - CA_n2A-n66(2A) modified to CA_n2A-n66A or CA_n66(2A)

  - CA_n7(2A)-n78A modified to CA_n7A-n78A or CA_n7(2A)

- The WI and band combination spreadsheet is revised accordingly.

# References

- RP-230133 Revised WID:Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), ZTE Corporation, RAN#99