3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305788

Online, , April 17 – April 26, 2023

Agenda item:	5.8.2.2

Source:	Keysight Technologies

Title:		On UE RF Requirements for Multi-AoA Rx Testing

Document for:	Approval

# Introduction

This contribution outlines our views on system parameter assumptions for multi-AoA reception testing. Most aspects are very similar to what is presented in the SI contribution [1].

# Discussion

In [2], a new Study Item (SI) was endorsed to define test methodologies for UE RF/demodulation/RRM testing to support reception of DL signals simultaneously from multiple Angles of Arrival (AoAs). This SI directly supports the Work Item (WI) in [3] with testability aspects to eventually enable testing of 4-layer MIMO reception which states [3]:

“The existing Rel-15 NR FR2 minimum UE requirements are defined with an assumption that UE is equipped with a single antenna panel and capable to perform DL reception using a single RX beam/chain reception. Furthermore, the UE performance requirements are limited for DL MIMO rank 1 and 2. In FR2, 4-layer MIMO reception requires beam reception from at least two directions. Although this is supported by the MIMO features since Rel-15, no performance requirements have yet been specified.” and “This work item aims to introduce the requirements for UEs capable of multi-beam/chain simultaneous DL reception on a single component carrier to achieve improved RF, RRM and UE demodulation performance.”

In the last two meetings, various important decisions were made to progress the multi-RX chain DL reception testing and system aspects [5][6][7][8].

This contribution addresses various open topics that were discussed last meeting without any conclusion [5][6].

# UE Orientation

For FR2 UE RF testing, different UE orientations are permissible for testing, i.e., Alignment Options 1, 2, and 3 as outlined in [10] in Clause N.2.

Table N.2-1: Test conditions and angle definitions for smartphones and tablets for Alignment Option 1 Test condition; DUT orientation; Link angle; Measurement angle; Diagram. Free space DUT Orientation 1 (default); α = 0º; β = 0º; γ = 0º; θLink; ϕLink with polarization reference PolLink = θ or ϕ; θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ. Table N.2-2: Test conditions and angle definitions for smartphones and tablets for Alignment Option 2 Test condition; DUT orientation; Link angle; Measurement angle; Diagram. Free space DUT Orientation 1 (default); α = 0º; β = -90º; γ = 0º; θLink; ϕLink with polarization reference PolLink = θ or ϕ; θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ. Table N.2-3: Test conditions and angle definitions for smartphones and tablets for Alignment Option 3 Test condition; DUT orientation; Link angle; Measurement angle; Diagram. Free space DUT Orientation 1 (default); α = 90º; β = 0º; γ = 0º; θLink; ϕLink with polarization reference PolLink = θ or ϕ; θMeas; ϕMeas with polarization reference PolMeas = θ or ϕ.

For UE RF testing with a single feed probe (generally in the z direction), the selection of any of these orientations has no impact on the test results (peak EIRP/EIS, spherical coverage CDF, TRP, etc) ignoring any fixturing impact on the performance.

Observation 1: FR2 UE RF test cases allow multiple alignment options; the selection of any of the orientations has no impact on the test results

For FR2 multi-AoA UE RRM testing, absolute probe locations are not defined and necessary (“The absolute position of the probes is left up to implementation” per [11]) and given that most RRM testing are based on pass/fail verdicts using multiple iterations (arbitrary selection of angular offsets), any of the above UE orientation is permitted.

For FR2 NR MIMO testing, absolute probe locations are defined [12] and the test verdict is based on parametric testing, i.e., MASC defined in [13], which is closely aligned to FR2 multi-Rx testing. For these test cases, only a single device orientation is permitted [12].

Table A.3-1: Summary of possible DUT positioning options Testing condition; DUT orientation angles; Diagram. XY plane or P0 Orientation 1 (default); α=0; β=0; γ=0.

Observation 2: FR2 NR MIMO testing where absolute probe locations are defined and where the test verdict is based on parametric testing, only a single device orientation is permitted

It is expected that different device orientations/alignment options yield different multi-AoA spherical coverage test results and whether an augmentation of the test system/AoA directions allows those differences to be de-embedded would need detailed investigations from OEMs.

Proposal 1: OEMs to determine whether different device orientations/alignment options yield different multi-AoA spherical coverage test results and what augmentations of the test system/AoA directions are necessary to de-embed those differences

Regardless of the outcome of Proposal 1, to avoid any additional system complexities and potentially test-time increases required for orientation de-embedding, it is proposed to define just a single permitted DUT orientation/alignment option for FR2 multi-Rx testing.

Proposal 2: Define just a single permitted DUT orientation/alignment option for FR2 multi-Rx testing.

# Weighting of AoA Pairs

In the last meeting, it was decided to use the constant-step size grids, e.g., [7]

Issue 1-1-6: Grids for 3D scan - Proposals - Option 1 (QC, Keysight): Use constant step size grid - Option 2: Specify other option if any - Agreements: - Option 1 is agreed

and [8]

- For optimized AoA1 and AoA2 test point/perceived DL direction coverage, utilize constant-step size grids only.

For legacy spherical coverage, the PDF needs to be scaled by either sin(q) or Clenshaw-Curtis weights W(q), e.g., Clause K.1.5.0 [10]

When using constant step size measurement grids, a theta-dependent correction shall be applied, i.e., the PDF probability contribution for each measurement point is scaled by sin(q) or the normalized Clenshaw-Curtis weights W()/W(90°), introduced in Section M.4.2.1, to account for the denser grid point distribution near the poles.

For FR2 multi-Rx testing that is not based on counting pass/fail verdicts, it is therefore proposed to apply similar corrections, i.e., scale the PDF probability contribution for each combination of AoA1&AoA2 by sin(qAoA1)*sin(qAoA2) or the normalized Clenshaw-Curtis weights W(qAoA1)/W(90°)* W(qAoA2)/W(90°)

Proposal 3: For FR2 multi-Rx testing not based on counting pass/fail verdict but based on probability contributions instead, apply a theta-dependent correction, i.e., scale the PDF probability contribution for each combination of AoA1&AoA2 by sin(qAoA1)*sin(qAoA2) or the normalized Clenshaw-Curtis weights W(qAoA1)/W(90°)*W(qAoA2)/W(90°)

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: FR2 UE RF test cases allow multiple alignment options; the selection of any of the orientations has no impact on the test results

Observation 2: FR2 NR MIMO testing where absolute probe locations are defined and where the test verdict is based on parametric testing, only a single device orientation is permitted

Proposal 1: OEMs to determine whether different device orientations/alignment options yield different multi-AoA spherical coverage test results and what augmentations of the test system/AoA directions are necessary to de-embed those differences

Proposal 2: Define just a single permitted DUT orientation/alignment option for FR2 multi-Rx testing.

Proposal 3: For FR2 multi-Rx testing not based on counting pass/fail verdict but based on probability contributions instead, apply a theta-dependent correction, i.e., scale the PDF probability contribution for each combination of AoA1&AoA2 by sin(qAoA1)*sin(qAoA2) or the normalized Clenshaw-Curtis weights W(qAoA1)/W(90°)*W(qAoA2)/W(90°)

# References

- R4-2305786, Testing Considerations for Multi AoA Rx Testing, Keysight Technologies, 3GPP TSG-RAN4 Meeting #106bis-e, April 2023

- RP-220988, New SI: Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements, Qualcomm Incorporated, 3GPP TSG RAN Meeting #95e, March 2022

- RP-221753, “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm Incorporated, 3GPP TSG RAN Meeting #96, June 2022

- R4-2302522, System Parameter Assumptions for Multi-AoA Rx Testing, Keysight Technologies, 3GPP TSG-RAN4 Meeting #106, February 2023

- R4-2220264, WF for FR2 OTA test SI, Qualcomm, 3GPP TSG-RAN WG4 Meeting #105, November 2022

- R4-2220533, WF on FR2 multi-Rx, Apple, 3GPP TSG-RAN WG4 Meeting #105, November 2022

- R4-2302923, WF for FR2 OTA test enhancement SI, Qualcomm, 3GPP TSG-RAN WG4 Meeting # 106, March 2023

- R4-2303708, WF on NR FR2 multi-Rx chain DL reception, Samsung, Apple, 3GPP TSG-RAN WG4 Meeting # 106, March 2023

- TR 38.884, Study on enhanced test methods for FR2 NR UEs

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- TR38.810, Study on test methods

- TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)

- TS 38.151, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements

- R4-2218755, Further views on multi-Rx chain DL reception in FR2, Sony, Ericsson, 3GPP TSG-RAN WG4 Meeting # 105, November 2022