3GPP TSG RAN WG4 Meeting #106bis-e	R4-2305797

Electronic Meeting, April 17-26, 2023

Agenda Item:	5.11.2.3

Source: 	Ericsson

Title:	Analysis of BWP operation without restriction option B-1-2

Document for:	Discussion

# Introduction

At RAN#99 meeting a new Rel-18 WI to complete the specification work to support BWP operation without restriction based on different solutions/options was approved [1].

In this paper we further analyze option B-1-2, which is described in the WID as follows [1]:

- BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions:

  - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and

  - UE shall support option (C) NCD-SSB (subject to IoDT availability).

# Analysis of option B-1-2

The details of the feature and the capability to support option B-1-2 will be discussed and decided by RAN1 and RAN2 respectively.

RAN4 task is to specify the requirements related to the radio link monitoring (RLM) and the link recovery procedure (LRP) for supporting option B-1-2 i.e. when the SSB is outside the active BWP with interruptions. RAN4 has to specify the following main types of core requirements to support option B-1-2:

- Evaluation periods of the respective measurements for the RLM and LRP and

- Maximum allowed interruption on the serving cell(s) during the measurements performed for the RLM and LRP

## Evaluation periods for RLM and LRP

The RLM and LRP procedures involve various type of measurements performed on the SSB during their respective evaluation periods including:

- Out-of-sync detection,

- In-sync detection,

- Beam failure detection (BFD),

- Candidate beam detection (CBD),

- L1-RSRP,

- L1-SINR.

The existing SSB based RLM and LRP requirements in TS 38.133 are applicable provided that the SSB is within the active BWP. The UE capable of supporting BM/RLM/BFD based on option B-1-2 should be able to perform the above measurements during their existing evaluation periods regardless of whether the SSB is configured within or outside the active BWP. The SSB should however be within the configured UE channel BW (CBW) i.e. UE-specific CBW. Therefore, the applicability of the existing requirements for the UE supporting option B-1-2 should be defined in the following sections in Table 1 in a way similar to those for option B-1-1.

Table 1: List of top sections on RLM/LRP measurement requirement in TS 38.133 impacted for UE supporting option B-1-2.

| No. | Section number | Section title |
| --- | --- | --- |
| 1 | 8.1 | Radio Link Monitoring |
| 2 | 8.5 | Link Recovery Procedures |
| 3 | 9.5 | L1-RSRP measurements for Reporting |
| 4 | 9.8 | L1-SINR measurements for Reporting |

## Interruption requirements

The UE supporting option B-1-2 may have to retune its receiver for obtaining SSB samples for estimating the radio link quality for RLM and LRP procedures. The RF retuning will cause interruption on other activated serving cells if the UE is configured in NR CA or DC (i.e. EN-DC, NE-DC or NR-DC). The UE is configured with different set of SSB resources for different set of RLM and LRP procedures via RRC. According to TS 38.331, the following independent set of SSB resources are configured for various RLM and LRP procedures:

- RadioLinkMonitoringRS; same for both RLM and BFD

- CandidateBeamRS; for CBD

- CSI-SSB-ResourceSet; same for both L1-RSRP and L1-SINR measurements

Furthermore, according to TS 38.331, the maximum number of SSB resources for CBD can be 64. The maximum number of SSB resources for L1-RSRP/L1-SINR can also be 64. In FR1, the number of SSBs vary between 4 and 8 depending on the carrier frequency. Typical number of SSBs configured in FR1 for CBD and L1-RSRP/L1-SINR may be 1 or 2. Also the typical number of SSBs configured in FR2 for CBD and L1-RSRP/L1-SINR may be lower than 64 but nevertheless all SSBs will be well within 1 ms e.g. 8 slots with SCS=120 kHz. The maximum SSB periodicity is 160 ms. A typical SSB periodicity configured by the network may be 20 ms or 40 ms.

Considering different set of SSB configurations for RLM/BFD, CDB and L1-RSRP/L-SINR, the overall interruption can be very large if the UE retunes its receiver outside the active BWP to monitor these SSBs.

The measurement sampling for obtaining the measurements for RLM and LRP is up to the UE implementation. Therefore, the gNB cannot be certain when the UE retunes its receiver outside the active BWP and due to which the UE cannot receive data/grants in the serving cell. Therefore, if the UE is allowed to retune for every SSB resource then there will be very frequent interruption causing substantial outage in the network e.g. drop of scheduled data, HARQ feedback etc.

The interruption requirements can be defined in terms of:

- Maximum interruption probability and

- Maximum interruption length

The above interruption metrics are described below:

Maximum interruption probability:

For example, assuming SSB periodicity of 40 ms, each interruption length of one slot for 15 kHz SCS and assuming one interruption before and one after the SSB resource, the total number of interruptions on other active serving cells during the out-of-sync evaluation period, which requires 10 SSB samples, will be 20 slots. This corresponds to 5% interruption probability. There will be similar level of interruptions on other active serving cells due to CBD and L1-RSRP/L-SINR measurements. This level of interruptions is certainly unacceptable from the network performance point of view.

We therefore suggest that the maximum allowed interruption probability is limited to around 0.5% due to the measurements for RLM/BFD, CBD and L1-RSRP/L1-SINR. Therefore, one option is to define individual interruption probability for each of the three categories of the measurements e.g.

- The rate of ACK/NACK feedback loss on any other active serving cell resulting from RLM/BFD measurements on the serving cell shall not exceed 0.5 %.

- The rate of ACK/NACK feedback loss on any other active serving cell resulting from CBD measurements on the serving cell shall not exceed 0.5 %.

- The rate of ACK/NACK feedback loss on any other active serving cell resulting from L1-RSRP/L1-SINR measurements on the serving cell shall not exceed 0.5 %.

Another alternative is to define one aggregated interruption probability covering all the three categories of the measurements. In this case in our view the aggregared interruption probability should be limited to 1%. This is because it is unlikely that the UE simultaneously (during overlapping time) performs the three categories of the measurements (RLM/BFD, CBD and L1-RSRP/L1-SINR) all the time. Therefore, we suggest that the aggregared interruption probability is less than 1% e.g.

- The rate of ACK/NACK feedback loss on any other active serving cell resulting from RLM/BFD, CBD and L1-RSRP/L1-SINR measurements on the serving cell shall not exceed 0.5 %.

Furthermore, the interruption should only be allowed in the following cases with respect to the DRX cycle on the serving cell on which the RLM/LRP measurements are performed:

- When no DRX cycle is used on the serving cell on which the RLM/LRP measurements are performed.

- When the DRX cycle used on the serving cell on which the RLM/LRP measurements are performed is shorter than or equal to 320 ms and the DRX ON duration is aligned with the SSB

Otherwise, no interruption should be allowed in the following cases i.e.

- When the DRX cycle used on the serving cell on which the RLM/LRP measurements are performed is less than or equal to 320 ms and the DRX ON duration is misaligned with the SSB.

- When the DRX cycle used on the serving cell on which the RLM/LRP measurements are performed is larger than 320 ms.

The reason is that when DRX cycle is long (i.e. ≥ 640 ms) the UE can retune its receiver between the ON duration and perform the RLM/LRP measurements without causing any interruption on other serving cells.

Maximum interruption length:

The maximum interruption length of the interruption on the other active serving cells (victim cell) should also be defined. The existing interruption lengths defined in TS 38.133 and TS 36.133 various scenarios can be reused. The RF tuning time is 0.5 ms and 0.25 ms in FR1 and FR2 respectively. The interruption length can be expressed in or rounded off to slots.

Interruption in NR CA:

When the UE is configured with NR CA then Table 8.2.2.2.2-1 defined in TS 38.133 will be reasonable proposal, which can be applicable for the inter-band CA. On the other hand, Table 8.2.2.2.2-2 defined in TS 38.133 can be reused with necessary modification for intra-band CA. In both cases in order to minimize the interruption, the UE should retune immediately before and immediately after the SSB resource i.e. slots containing the SSB(s) for the RLM or RLP. The proposals are shown in table 2 and table 3 for inter-band CA and intra-band CA respectively.

Table 2: Interruption due to RLM and RLP measurements on other active serving cell in inter-band NR CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 |

Table 3: Interruption due to RLM and RLP measurements on other active serving cell in intra-band NR CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + NSSB |
| 1 | 0.5 | 1 + NSSB |
| 2 | 0.25 | 2 + NSSB |
| 3 | 0.125 | 4 + NSSB |
| 5 | 0.03125 | 16 + NSSB |
| 6 | 0.015625 | 32 + NSSB |
| NOTE: NSSB is the number of slots containing the configured SSB(s) for RLM/BFD, CBD or L1-RSRP/L1-SINR |  |  |

Interruption in NR DC:

The NR DC supports both synchronous and asynchronous inter-band DC. For synchronous inter-band NR-DC, the interruption length shown in table 2 can be reused since the MRTD for NR CA and synchronous inter-band NR-DC are the same or in the same range.

For asynchronous inter-band NR-DC, 1 slot needs to be added to the interruption length in table 4 for SCS ≥ 60 kHz to account for the MRTD for NR DC.

Table 4: Interruption due to RLM and RLM on other active serving cell in asynchronous inter-band NR-DC

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 3 |
|  |  | Either aggressor cell or victim cell is on FR1 | 4 |
| 3 | 0.125 | Aggressor cell is on FR2 | 5 |
|  |  | Aggressor cell is on FR1 | 6 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 18 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 34 |

Interruption on LTE serving cell(s) in EN-DC and NE-DC:

In EN-DC, the interruption on the active LTE serving cell(s) depends on whether it is inter-band (which can be synchronous or asynchronous EN-DC) or intra-band EN-DC (which can be only synchronous EN-DC). The interruption on the active LTE serving cell(s) for EN-DC and NE-DC can be derived from section 7.32.2.6.1 in 36.133 as follows:

For inter-band EN-DC, the interruption on the active LTE serving cell shall not exceed:

- 1 subframe for synchronous inter-band EN-DC or

- 2 subframes for asynchronous inter-band EN-DC.

For synchronous intra-band EN-DC, the interruption shall not exceed:

- the duration of NSSB + 2 subframes.

For inter-band NE-DC, the interruption on the active LTE serving cell shall not exceed:

- 1 subframe for synchronous inter-band NE-DC or

- 2 subframes for asynchronous inter-band NE-DC.

For synchronous intra-band NE-DC, the interruption shall not exceed:

- the duration of NSSB + 2 subframes.

Where, NSSB is the number of slots containing the configured SSB(s) for RLM/BFD, CBD or L1-RSRP/L1-SINR.

Table 5 and table 6 lists high level sections in TS 38.133 and TS 36.133 where the interruption requirements on other active serving cells due to the RLM/LRP measurements performed based on option B-1-2 should be defined.

Table 5: List of top sections on interruption requirements in TS 38.133 impacted for UE supporting option B-1-2:

| No. | Section number | Section title |
| --- | --- | --- |
| 1 | 8.2.2 | SA: Interruptions with Standalone NR Carrier Aggregation |
| 2 | 8.2.4 | NR-DC: Interruptions |

Table 6: List of top sections on interruption requirements in TS 36.133 impacted for UE supporting option B-1-2:

| No. | Section number | Section title |
| --- | --- | --- |
| 1 | 7.32 | 7.32 Interruptions with EN-DC |
| 2 | 7.36 | 7.36 Interruptions with NE-DC |

## Specification changes to support option B-1-2

### Evaluation periods of RLM and LRP

The applicability of the existing RLM/LRP measurement requirements for option B-1-2 can be defined in the top-level and necessary sections which contain conditions related to the RLM/LRP within the active BWP. The proposed updates to these sections are shown below with revision marks.

## 8.1	Radio Link Monitoring

### 8.1.1	Introduction

The requirements in clause 8.1 apply for radio link monitoring on:

-	PCell in SA NR, NR-DC and NE-DC operation mode,

-	PSCell in NR-DC and EN-DC operation mode,

-	Deactivated PSCell in NR-DC and EN-DC operation mode, when configured.

The UE shall monitor the downlink radio link quality based on the reference signal configured as RLM-RS resource(s) in order to detect the downlink radio link quality of the PCell, PSCell and deactivated PSCell if configured with bfd-and-RLM with value true as specified in TS 38.213 [3]. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. UE is not required to perform RLM outside the active DL BWP except for the UE capable of [BM/RLM/BFD based on SSB outside the active BWP with interruptions] performing SSB based RLM provided that no CSI-RS, NCD-SSB or CD-SSB is within the DL BWP.

### 8.1.2	Requirements for SSB based radio link monitoring

#### 8.1.2.1	Introduction

The requirements in this clause apply for each SSB based RLM-RS resource configured for PCell, PSCell or deactivated PSCell, provided that the SSB configured for RLM is actually transmitted within UE active DL BWP except for the SSB configured for RLM for the UE capable of [BM/RLM/BFD based on SSB outside the active BWP with interruptions] provided that no CSI-RS, NCD-SSB or CD-SSB is within the DL BWP, during the entire evaluation period specified in clause 8.1.2.2.

## 8.5	Link Recovery Procedures

### 8.5.1	Introduction

The UE shall assess the downlink radio link quality of a serving cell based on the reference signal in the set  as specified in TS 38.213 [3] in order to detect beam failure on:

-	PCell in SA, NR-DC, or NE-DC operation mode,

-	PSCell in NR-DC and EN-DC operation mode,

-	SCell in SA, NR-DC, NE-DC or EN-DC operation mode,

-	Deactivated PSCell in NR-DC and EN-DC operation mode

The RS resource configurations in the set  on PCell, PSCell or deactivated PSCell (if configured with bfd-and-RLM with value true) can be periodic CSI-RS resources and/or SSBs. RS resource configuration in the set  on SCell shall be periodic CSI-RS. UE is not required to perform beam failure detection outside the active DL BWP except for the UE capable of [BM/RLM/BFD based on SSB outside the active BWP with interruptions] performing SSB based beam failure detection provided that no CSI-RS, NCD-SSB or CD-SSB is within the DL BWP. UE is not required to meet the requirements in clause 8.5.2 and 8.5.3 if UE does not have set . UE is not required to perform beam failure detection on a deactivated SCell, and also not required to perform beam failure detection on resources which is implicitly configured for a deactivated SCell. When more than 2 periodic CSI-RS resources on a CC are configured in the set  for current SCell or implicitly configured in the set  for other SCell, it is up to UE implementation to select two of CSI-RS resources in active BWP in current CC to perform beam failure detection. UE is not required to perform beam failure detection on a SCell on which  is not configured.

#### 8.5.2.1	Introduction

The requirements in this clause apply for each SSB resource in the set  configured for a serving cell, provided that the SSB configured for beam failure detection is actually transmitted within the UE active DL BWP except the SSB configured for beam failure detection for the UE capable of [BM/RLM/BFD based on SSB outside the active BWP with interruptions] provided that no CSI-RS, NCD-SSB or CD-SSB is within the DL BWP, during the entire evaluation period specified in clause 8.5.2.2. The requirements in this clause could not be applicable if UE is required to perform beam failure detection on more than 1 serving cell per band.

#### 8.5.5.1	Introduction

The requirements in this clause apply for each SSB resource in the set  configured for a serving cell, provided that the SSBs configured for candidate beam detection are actually transmitted within UE active DL BWP during the entire evaluation period specified in clause 8.5.5.2 except for the UE capable of [BM/RLM/BFD based on SSB outside the active BWP with interruptions]. The requirements in this clause apply when UE is required to perform beam failure detection on no more than 1 serving cell per band.

## 9.5	L1-RSRP measurements for Reporting

### 9.5.1	Introduction

When configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP. The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements within the active BWP except on the SSB configured for L1-RSRP measurements for the UE capable of [BM/RLM/BFD based on SSB outside the active BWP with interruptions] provided that no CSI-RS, NCD-SSB or CD-SSB is within the DL BWP.

The UE shall be able to measure all CSI-RS resources and/or SSB resources of the nzp-CSI-RS-ResourceSet and/or csi-SSB-ResourceSet within the CSI-ResourceConfig settings configured for L1-RSRP for the active BWP, provided that the number of resources, including the number of SSB resources of the cell with PCI different from serving cell configured for L1-RSRP measurements in 9.13, does not exceed the UE capability indicated by beamManagementSSB-CSI-RS.

The UE shall report the measurement quantity (reportQuantity) and send periodic, semi-persistent or aperiodic reports, according to the reportConfigType according to the CSI reporting configuration(s) (CSI-ReportConfig) for the active BWP.

## 9.8	L1-SINR measurements for Reporting

### 9.8.1	Introduction

When configured by the network, the UE shall be able to perform L1-SINR measurements with the measurement resources configured as the selection of:

-	CSI-RS based CMR and no dedicated IMR configured;

-	SSB based CMR and dedicated IMR configured;

-	CSI-RS based CMR and dedicated IMR configured.

The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-SINR measurements within the active BWP except on the SSB configured for L1-SINR measurements for the UE capable of [BM/RLM/BFD based on SSB outside the active BWP with interruptions] provided that no CSI-RS, NCD-SSB or CD-SSB is within the DL BWP.

The UE shall be able to measure all CSI-RS resources and/or SSB resources and/or CSI-IM resources of the nzp-CSI-RS-ResourceSet and/or csi-SSB-ResourceSet and/or CSI-IM-ResourceSet within the CSI-ResourceConfig settings for L1-SINR for the active BWP and measure interference on corresponding NZP CSI-RS or CSI-IM resources if configured, provided that the number of resources does not exceed the UE capability indicated by beamManagementSSB-CSI-RS.

The UE shall report the measurement quantity (reportQuantity) and send periodic, semi-persistent or aperiodic reports, according to the reportConfigType according to the CSI reporting configuration(s) (CSI-ReportConfig) for the active BWP.

# Summary

This paper has further analyzed the requirements for RLM and LRP for the UE capable of option B-1-2 i.e. for the UE supporting BM/RLM/BFD based on SSB outside the active BWP with interruptions. The following are the observations and proposals:

- Observation #1: The option B-1-2 means the UE meets the RLM/LRP requirements provided that the SSB is anywhere within the UE-specific channel bandwidth.

- Observation #2: The SSB being outside the active BWP will not impact the evaluation periods of the measurements performed for radio link monitoring (RLM) and link recovery procedure (LRP).

- Proposal #1: The existing evaluation periods of the following measurements related to the SSB-based RLM and the SSB-based LRP defined in TS 38.133 shall also apply for the UE supporting [BM/RLM/BFD based on SSB outside the active BWP with interruptions] regardless of whether the SSB is outside the active BWP or not, and provided that the SSB is within the UE-specific channel BW:

  - Out-of-sync detection,

  - In-sync detection,

  - Beam failure detection (BFD),

  - Candidate beam detection (CBD),

  - L1-RSRP,

  - L1-SINR.

RLM/LRP measurement period requirements:

- Proposal #2: The existing SSB-based RLM and the SSB-based LRP requirements in clauses 8.1, 8.5, 9.5 and 9.8 in TS 38.133 are made applicable for UE supporting [BM/RLM/BFD based on SSB outside the active BWP with interruptions] without the restriction (for this UE capability) that the SSB has to be within the active BWP.

- Proposal #2: The existing SSB-based RLM and the SSB-based LRP requirements in clauses 8.1, 8.5, 9.5 and 9.8 in TS 38.133 are made applicable for UE supporting [BM/RLM/BFD based on SSB outside the active BWP with interruptions] without the restriction (for this UE capability) that the SSB has to be within the active BWP.

Interruption due to RLM/LRP measurements:

- Observation #3: According to TS 38.331, the following independent set of SSB resources are configured for various RLM and LRP procedures:

  - RadioLinkMonitoringRS; same for both RLM and BFD

  - CandidateBeamRS; for CBD

  - CSI-SSB-ResourceSet; same for both L1-RSRP and L1-SINR measurements

- Observation #4: When DRX cycle of the serving cell on which the RLM/LRP measurements are performed, is long (i.e. ≥ 640 ms) then the UE can retune its receiver between the successive ON durations and perform the RLM/LRP measurements without causing any interruption on other serving cells.

- Proposal #3: The maximum interruption probability on other active serving cells due to RLM/LRP measurements performed on the serving cell are defined using either option 1 or option 2; we prefer option 1:

  - Option 1:

    - The rate of ACK/NACK feedback loss on any other active serving cell resulting from RLM/BFD measurements on the serving cell shall not exceed 0.5 %.

    - The rate of ACK/NACK feedback loss on any other active serving cell resulting from CBD measurements on the serving cell shall not exceed 0.5 %.

    - The rate of ACK/NACK feedback loss on any other active serving cell resulting from L1-RSRP/L1-SINR measurements on the serving cell shall not exceed 0.5 %.

  - Option 2:

    - The rate of ACK/NACK feedback loss on any other active serving cell resulting from RLM/BFD, CBD and L1-RSRP/L1-SINR measurements on the serving cell shall not exceed 0.5 %.

- Proposal #4: The maximum length of each interruption is defined as follows:

Interruption length in NR CA:

Table 2: Interruption due to RLM and RLP measurements on other active serving cell in inter-band NR CA

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 17 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 33 |

Table 3: Interruption due to RLM and RLP measurements on other active serving cell in intra-band NR CA

|  | NR Slot length (ms) | Interruption length (slots) |
| --- | --- | --- |
| 0 | 1 | 1 + NSSB |
| 1 | 0.5 | 1 + NSSB |
| 2 | 0.25 | 2 + NSSB |
| 3 | 0.125 | 4 + NSSB |
| 5 | 0.03125 | 16 + NSSB |
| 6 | 0.015625 | 32 + NSSB |
| NOTE: NSSB is the number of slots containing the configured SSB(s) for RLM/BFD, CBD or L1-RSRP/L1-SINR |  |  |

Interruption length in NR DC:

- For synchronous inter-band NR-DC, the interruption length is the same as in table 2.

- For asynchronous inter-band NR-DC, the interruption length is shown in table 4.

Table 4: Interruption due to RLM and RLM on other active serving cell in asynchronous inter-band NR-DC

|  | NR Slot length (ms) of victim cell | Interruption length X2 (slots) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 3 |
|  |  | Either aggressor cell or victim cell is on FR1 | 4 |
| 3 | 0.125 | Aggressor cell is on FR2 | 5 |
|  |  | Aggressor cell is on FR1 | 6 |
| 5 | 0.03125 | Aggressor cell is on FR1 | 18 |
| 6 | 0.015625 | Aggressor cell is on FR1 | 34 |

Interruption length in EN-DC and NE-DC:

For inter-band EN-DC, the interruption on the active LTE serving cell shall not exceed:

- 1 subframe for synchronous inter-band EN-DC or

- 2 subframes for asynchronous inter-band EN-DC.

For synchronous intra-band EN-DC, the interruption shall not exceed:

- the duration of NSSB + 2 subframes.

For inter-band NE-DC, the interruption on the active LTE serving cell shall not exceed:

- 1 subframe for synchronous inter-band NE-DC or

- 2 subframes for asynchronous inter-band NE-DC.

For synchronous intra-band NE-DC, the interruption shall not exceed:

- the duration of NSSB + 2 subframes.

Where, NSSB is the number of slots containing the configured SSB(s) for RLM/BFD, CBD or L1-RSRP/L1-SINR.

Proposal #5: The interruption requirements in proposals # 3 and # 5 are allowed only under certain conditions wrt the DRX cycle:

- The interruption is allowed only when any of the following conditions are met:

  - When no DRX cycle is used on the serving cell on which the RLM/LRP measurements are performed.

  - When the DRX cycle used on the serving cell on which the RLM/LRP measurements are performed is shorter than or equal to 320 ms and the DRX ON duration is aligned with the SSB

- Otherwise, no interruption is allowed in any of the following cases:

- When the DRX cycle used on the serving cell on which the RLM/LRP measurements are performed is less than or equal to 320 ms and the DRX ON duration is misaligned with the SSB.

- When the DRX cycle used on the serving cell on which the RLM/LRP measurements are performed is larger than 320 ms.

# References

- RP-230805, New WID: Complete the specification support for BandWidth Part operation without restriction in NR, Vodafone, RAN#99, Rotterdam, March 2023.