3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2305801

Online, April 17th – April 26th, 2023

Agenda item:	5.15.5

Source: 	Qualcomm Incorporated

Title: 	RRM requirements for NR support for less than 5MHz spectrum

Document for:	Discussion

# Introduction

Agreement:

- For the 3MHz channel bandwidth in band n100 (max channel utilization 15 PRBs as already agreed in RAN1/RAN4)

  - PBCH transmission bandwidth is 12 PRBs

  - CORESET#0 transmission bandwidth is to be decided by RAN1

- RAN1 is requested to consider whether the above also applies for other bands with 3MHz channel bandwidth, or whether the PBCH transmission bandwidth is 15 PRBs for such bands.

- For the 5MHz channel bandwidth:

  - PBCH transmission bandwidth is 20 PRBs

  - CORESET#0 transmission bandwidth is to be decided by RAN1

- Other details (including sync raster details) are to be progressed in the WGs.

- The less-than-5MHz WI in Rel-18 should consider single-carrier operation, excluding RedCap. In addition, UE speeds up to 500km/h should be targeted for Band n100 without impact to RAN1.

- It is assumed that UE support of the <5MHz feature is band-specific and optional

RAN4 started discussing RRM requirements for NR support for dedicated spectrum less than 5MHz [1] in the last meeting and WF [2] from RAN4#106 lists the agreements and open issues. During RAN#99, LSs from RAN1[3] and RAN4[4] were discussed and LS responses [5][6] were provided. A summary of the agreements is outlined below:

In this contribution paper, we present our views in the scope of the RRM requirements.

# Scope of RRM requirements

As agreed in RAN#99, a 5MHz channel shall have the legacy PBCH of 20PRBs, but a 3MHz channel shall have a PBCH transmission bandwidth of 12PRBs. The reduced PBCH bandwidth for 3MHz channel bandwidth will have an impact on RRM requirements that depend on PBCH DMRS detection, PBCH decoding etc.

Observation 1: RANP agreed to use a PBCH transmission bandwidth is 20 PRBs for 5MHz channel bandwidth and a PBCH transmission bandwidth is 12 PRBs for 3MHz channel bandwidth.

Proposal 1: RAN4 to study impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on at least the following RRM requirements:

- Handover

- RRC Re-establishment

- RRC Connection Release with Redirection

- RLM/BFD/CBD

- Intra frequency/Inter-frequency measurements

# Conclusion

Observation 1: RANP agreed to use a PBCH transmission bandwidth is 20 PRBs for 5MHz channel bandwidth and a PBCH transmission bandwidth is 12 PRBs for 3MHz channel bandwidth.

Proposal 1: RAN4 to study impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on at least the following RRM requirements:

- Handover

- RRC Re-establishment

- RRC Connection Release with Redirection

- RLM/BFD/CBD

- Intra frequency/Inter-frequency measurements

# References

[1] RP-230186, “Revised WID: NR support for dedicated spectrum less than 5MHz for FR1”, Nokia, Anterix, T-Mobile USA, RAN#99

[2] R4-2303191, “WF on NR_FR1_lessthan_5MHz_BW”, Nokia, RAN4#106

[3] R1-2302186, “LS on transmission bandwidths for NR on dedicated spectrum less than 5 MHz”, RAN1, RAN1#112

[4] R4-2303713, “LS for spectrum less than 5 MHz”, RAN4, RAN4#106

[5] RP-230780, “Response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz”, RAN, RAN#99

[6] RP-230780, “Response to LS from RAN4 on spectrum less than 5 MHz”, RAN, RAN#99