3GPP TSG-RAN WG4 #106-e	R4-2305809

# Electronic Meeting, April 17th - 26th, 2023

Agenda item:	5.11.2.1

Source: 	Spreadtrum Communication

Title: 	Discussion on option A for BWP operation without restriction

Document for:	Discussion

Introduction

The new WI RP-230805[1] has been created in RAN99 following several rounds of discussion about resolutions of bandwidth part operation without restriction. Following approaches are identified to further analyze specification impacts.

Excerpt from RP-230805[1] - For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to IoDT availability). The interruption related requirements will be decided and specified in RAN4.

The interpretations of the above options are depicted in [4], this discussion paper presents our observations on time and frequency tracking alternatives in case of option A.

Discussion

As we are aware, UE acquires the initial transmission timing based on SSB detection during cell searching procedures. The relevant RRM requirement for initial transmission timing is defined in 3GPP TS 38.133 [2] section 7.1.2.

Excerpt from TS 38.133 [2] section 7.1.2 The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies: - when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission, or it is the first transmission sent on the PSCell for activating the deactivated SCG without RACH. - when it is the transmission for PUSCH on CG resources for SDT in RRC_Inactive. When the UL SCS is 120 kHz or smaller, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. When the UL SCS is 480 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 80 ms. When the UL SCS is 960 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 40 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna. NTA for PRACH is defined as 0.

In that context, a few companies showed concerns on the timing requirements in case of Option A where SSB is outside the active BWP. At RAN4 106 meeting, it’s agreed the existing timing requirements apply for Option A and they are complete. The agreement is captured in RAN4 LS of [3]

Excerpt from RAN4 LS [3] If CSI-RS based RLM/BFD/BM requirements are complete for Option A <Agreement> - For the UE performing the BM/RLM/BFD based on CSI-RS within the active BWP (Option A), existing BM/RLM/BFD requirements defined in TS 38.133 is complete and no new or additional requirements are needed. Whether timing requirements should be enhanced for the case when CD-SSB is outside active BWP <Agreement> - The existing UE transmission timing error requirements based on the SSB defined in clause 7.1 of TS 38.133 shall apply for the UE performing the BM/RLM/BFD based on CSI-RS within the active BWP (Option A), i.e., no additional timing requirements are needed. FFS if any clarifications on the existing requirements is needed, e.g., applicability of requirements, conditions of gap configuration etc.

As far as option A is considered, besides the aforementioned initial timing requirements, we would like to make discussion on timing and frequency tracking alternatives when UE operates in a BWP without SSB.

Timing and frequency tracking are crucial for OFDM system, due to there is mismatch between the oscillator frequency at the receiver and at the transmitter, which then translates to ISI and ICI interferences to impair the overall performance. Practically, the CFO and timing offset are estimated and corrected to some extent by using specific synchronization algorithms, which relies on the phase offsets observed on consecutive signals in both frequency domain and time domain. Hence one symbol CSI-RS or DMRS is not suitable for CFO estimation. While a cross-correlation between the received signal and pre-stored signal pattern is performed for fine timing recover, DMRS usually prone to precoding is not suitable to implement timing offset estimation.

Observation 1: One symbol CSI-RS or DMTS is not suitable for CFO estimation.

Observation 2: Pre-coded DMRS is not suitable to implement timing offset estimation

Proposal 1: It’s proposed to take the above observations into account when making specification clarifications for the Option A.

Proposal 2: It’s proposed to consider alternative T/F tracking signals other than MG based SSB for the Option A, such as TRS, for the sake of

finer T/F tracking performance while avoiding frequent MG based SSB reading.

Conclusion

This discussion paper presents our observations on time and frequency tracking alternatives in case of option A. We made the following observations:

Observation 1: One symbol CSI-RS or DMTS is not suitable for CFO estimation.

Observation 2: Pre-coded DMRS is not suitable to implement timing offset estimation

Proposal 1: It’s proposed to take the above observations into account when making specification clarifications for the Option A.

Proposal 2: It’s proposed to consider alternative T/F tracking signals other than MG based SSB for the Option A, such as TRS, for the sake of finer T/F tracking performance while avoiding frequent MG based SSB reading.

References

- RP-230805, “New WID: Complete the specification support for BandWidth Part operation without restriction in NR”, TSG-RAN4

- TS 38.133 “NR; Requirements for support of radio resource management”

- R4-2303314, “LS on additional agreements for BWP_withoutRestriction for option A,” Apple, RAN4#106, March 2023

- RP-223506, “Summary for [98e-30-BWP-WithoutRestriction],” Vodafone, RAN #98e, December 2022