3GPP RAN4 Meeting #106-bis-e	R4-2305816

# Online, April 17th – 26th, 2023

Title: 	Sidelink UE TX requirements in unlicensed spectrum

Agenda Item:	5.31.2.1.2

Source: 	Qualcomm Inc

Document for:	Approval

# Introduction

According to the work plan we are to consider MPR and A-MPR scenarios during this meeting.

2) RAN4 #106bis (Apr 2023) • For FR1 single carrier SL in unlicensed bands o Agree on targeted FR1 unlicensed bands, CBWs and discuss other system parameters o Agree on targeted power classes and MIMO/TxD scenarios o Agree on the specification structures, e.g., separate from current NR-U and SL requirements or nested in them o Discuss the UE RF emission requirements and consider MPR/AMPR scenarios and evaluation assumptions • For Uu+SL concurrent operation o Agree on the targeted Uu+SL band combinations, e.g. Uu at licensed bands and SL at unlicensed bands o Discuss and agree on the targeted power classes and CBW combinations o Continue discussing other RF requirements o Initially discuss the MPR/AMPR scenarios • Agree on the necessity and scenario of coexistence evaluations

# NS cases for sidelink in n46/96/102

In the NRU WI, the following NS values were specified to inform the UE of county-specific conditions related to regulatory rules. Our view is RAN4 should develop sidelink requirements to allow operation in the unlicensed spectrum for these regions, as was NRU. We should use the same NS values and develop sidelink A-MPR as needed.

### 6.1.1	Transmitter characteristics

This section details specific transmitter characteristics for a UE operating in the 6 GHz NR band.

Table 6.1.1-1: Summary of NS values.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | NS_58 | TBD |
| Region 2 |  |  |  |
| US | NS_54 | NS_53 | N/A |
| Canada | NS_54 | NS_59 | TBD |
| Brazil | N/A | NS_53 | TBD |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | N/A |
| Costa Rica | N/A | NS_01 | TBD |
| Colombia | N/A | NS_53 | N/A |
| Region 3 |  |  |  |
| South Korea | N/A | NS_60 | [NS_61] |

Proposal: RAN4 to use NS_29-31, 53-54, 58-6 to signal additional requirements for sidelink in bands n46/n96/n102.

# Conclusions

Proposal: RAN4 to use NS_29-31, 53-54, 58-6 to signal additional requirements for sidelink in bands n46/n96/n102.

# References

- R4-2301182, R18 workplan for NR SL evolution WI

- 3GPP TR 38.849 V17.1.0 , Introduction of 6GHz NR unlicensed operation

- TS 38.101-1-1 V.17.7.0