3GPP TSG-RAN WG4 #106-bis-e	R4-2305818

# Online meeting , April 17th- 24th, 2023

Agenda item:	5.31.2.2

Source: 	Qualcomm Incorporated

Title: 	Concurrent operation and Uu and sidelink UE considerations

Document for:	Approval

# Introduction

The work plan for this meeting included discussion of concurrent operation of sidelink aspects.

- RAN4 #106bis (Apr 2023) - For FR1 single carrier SL in unlicensed bands - Agree on targeted FR1 unlicensed bands, CBWs and discuss other system parameters - Agree on targeted power classes and MIMO/TxD scenarios - Agree on the specification structures, e.g., separate from current NR-U and SL requirements or nested in them - Discuss the UE RF emission requirements and consider MPR/AMPR scenarios and evaluation assumptions - For Uu+SL concurrent operation - Agree on the targeted Uu+SL band combinations, e.g., Uu at licensed bands and SL at unlicensed bands - Discuss and agree on the targeted power classes and CBW combinations - Continue discussing other RF requirements - Initially discuss the MPR/AMPR scenarios - Agree on the necessity and scenario of coexistence evaluations

# Discussion

IN the last RAN4 meeting we has some discussion on concurrent operation on Uu@Licensed and SL@Un-licensed

#### Issue 1-4-1: Concurrent operation on Uu@Licensed and SL@Un-licensed The exact frequency band combinations for both Uu and SL-U are to be determined based on company input during the WI. Agreement: - Inter-band combinations for concurrent operation for Uu @Licensed and SL @Un-licensed are to be studied based on company inputs. In order to meet the WI objectives and schedule it is requested that combinations are decided in the next RAN4 meeting. - One example band combination is needed. FFS on which band combination is chosen. #### Issue 1-4-2: Concurrent operation on Uu@Un-licensed and SL@Un-licensed Proposed way forward: FFS

V2X already has concurrent for n47 = PC5 paired with Uu n1, n5, n8, n39, n40, n41, n71, n78, and n79. Our preference is to develop these same concurrent combinations.

Proposal: Develop concurrent operation on Uu@Licensed and SL@Un-licensed for n46/n96/n102 with n1,5,8,39,40,41,71,78, and 79.

And on LTE sidelink and NR sidelink co-channel we have sent LS to RAN1 describing TDMing the transmissions as one condition we will specify and leaving other conditions as FFS.

1. Overall Description: RAN4 discussed the WI objective on co-channel coexistence for LTE sidelink and NR sidelink, i.e. Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] RAN4 will study the LTE sidelink and NR sidelink co-channel coexistence scenarios in Rel-18 only the case where LTE sidelink and NR sidelink transmit via TDM manner but could receive simultaneously in the same channel for a single UE. Whether RAN4 will specify the corresponding requirements and requirements for other possible scenarios depends on the study outcome and further inputs from RAN1 and RAN2.

Proposal: Focus on the co-channel LTE sidelink and NR sidelink mode where both could receive simultaneously but transmit in a TDM manner.

# Conclusion

In this paper

Proposal: Develop concurrent operation on Uu@Licensed and SL@Un-licensed for n46/n96/n102 with n1,5,8,39,40,41,71,78, and 79.

Proposal: Focus on the co-channel LTE sidelink and NR sidelink mode where both could receive simultaneously but transmit in a TDM manner.

# References

- R4-2303568, WF on NR sidelink evolution

- RP-230077, WID revision: NR sidelink evolution, 3GPP TSG RAN Meeting #99

- R4-2301182, R18 workplan for NR SL evolution WI

- R4-2303718, LS on co-channel coexistence