3GPP TSG-RAN WG4 #106-bis-e	R4-2305821

April 17th ‒ 26th, 2023

Electronic Meeting

Agenda item:	6.4.2

Source: 	Qualcomm Incorporated

Title: 	900 MHz band coexistence with Band 5/26

Document for:	Discussion

# Introduction

A work item has been agreed to define new LTE [1] and NR [2] bands in the 900 MHz frequency range for use in the US.  One of the issues to be resolved is the UE coexistence requirement between this new band and Band 5 and Band 26.  This contribution provides measured results on the power backoff needed to meet a conventional -50 dBm/MHz emission requirement for protection of Band 5 and Band 26.

# Discussion

The 900 MHz band configuration is shown below as provided in [1] but amended to illustrate the coexistence scenario with the downlink for Band 5 and Band 26.

The 900 MHz band configuration is shown below as provided in [1] but amended to illustrate the coexistence scenario with the downlink for Band 5 and Band 26.  After discussion during RAN4 #106 meeting, the following agreements were captured in [3] regarding UE coexistence

- With the removal of 5 MHz CBW from the WID and the move of the center channel for 3 MHz shifted to 899 MHz it is not expected that A-MPR (to be verified) will be necesssary and the use of the 3GPP general SEM or FCC SEM will provide enough protection.

- FFS to see if the -50 dBm/MHz coexistence requirement can be relaxed or removed. This includes imposing only the FCC emission requirement or general SEM, RB allocation restriction, or assuming improved UE performance.

The work items were modified to remove the 5 MHz channel bandwidth so only 3 MHz and 1.4 MHz channels remain in scope for LTE.

Laboratory bench measurements were conducted to evaluate the power backoff needed to meet -50 dBm/MHz, recognizing applicability of this value is not yet agreed according to the second bullet of the way forward listed above.  For the measurements, five 3 MHz LTE waveforms were measured for emissions at 893.5 MHz with center frequencies of 897.5 MHz and 899 MHz.  The waveforms were all LTE signals with single RB at the upper and lower edge of the channel as well as full allocation with 15 RB’s.  The modulation was QPSK, DFT-S encoded, and the measurement was calibrated to 30 dB ACLR with 1 dB MPR for full allocation.  LO and image were added at -28 dBc and CIM3 at -60 dBc.  The required backoff results are as follows

| Waveform | Description | Power backoff to meet -50 dBm/MHz from 893 – 894 MHz |  |
| --- | --- | --- | --- |
|  |  | FTx = 897.5 MHz | FTx = 899 MHz |
| 1 | LTE_3MHz_1RB0_QPSK_Clipped | 10.2 | 0 |
| 2 | LTE_3MHz_1RB0_QPSK_Unclipped | 9.7 | 0 |
| 3 | LTE_3MHz_15RB0_QPSK_Clipped | 9.8 | 4.8 |
| 4 | LTE_3MHz_15RB0_QPSK_Unclipped | 9.8 | 4.8 |
| 5 | LTE_3MHz_1RB_14_QPSK_Unclipped | 12.3 | 0 |

From these measurements, it can be seen that the 3 MHz channel would suffer significant power backoff if placed at the lower edge of the band.  Fortunately, the agreement has already been captured that the 3 MHz channel shall be shifted to 899 MHz.  This is also in accordance with the Post-Transition Band Plan as outlined in the FCC Report and Order [4] which was specifically designed to align with 3GPP’s LTE channel raster.  This shift should be enforced in the specifications, perhaps by disallowing the EARFCN corresponding to channel locations below 899 MHz or at any other location than as identified in [4].

Proposal:  EARFCN for 3 MHz channels other than 899 MHz shall be disallowed in the specification.

The second observation is even with the 3 MHz channel placed at 899 MHz, for fully allocated waveforms the power backoff required is 4.8 dB (1 dB MPR + 3.8 dB A-MPR) to meet -50 dBm/MHz.  Therefore, if the zero A-MPR assumption from the way forward is to be fulfilled, the emission requirement must be relaxed from -50 dBm/MHz.  The LTE SEM requirements for a 3 MHz channel with 4 MHz offset (edge of the 3 MHz channel is 897.5 MHz and the center of the nearest 1 MHz emission region is 893.5 MHz) is -10 dBm/MHz.  The FCC [4] requires out of band emissions by attenuated by at least 43 + 10 log (P) dB for uplink operations in 897.5 – 900.5 MHz.  This amounts to an emission limit of -13 dBm/MHz.

In recognition of the small separation between the 900 MHz band and the downlink of Band 5 and Band 26 as well as the fact that networks are already operating in this frequency band requiring no more than -13 dBm/MHz emissions, it is proposed that a value of -30 dBm/MHz could be an appropriate middle-ground.  A value of -30 dBm/MHz sits between the conventional UE coexistence emission requirement of -50 dBm/MHz, the LTE general SEM of -10 dBm/MHz, and the FCC mandated emission limit of -13 dBm/MHz.  A value of -30 dBm/MHz could be met without A-MPR.

Proposal:  An emission limit of -30 dBm/MHz is specified for UE coexistence to protect the downlinks of Band 5 and Band 26 when the 3 MHz LTE channel is centered at 899 MHz.  No A-MPR is allowed.

# Conclusion

In this contribution, the UE coexistence emissions from the uplink in the 900 MHz band falling into the downlinks of Band 5 and Band 26 are studied.  It was found that for a 3 MHz channel centered at 899 MHz, the conventional UE coexistence requirement of -50 dBm/MHz could not be met without A-MPR power backoff.  Additionally, taking into account the LTE general SEM and the FCC mandated out-of-band emission limit as well as the observation that proprietary LTE-based networks are already operating in this band based solely on the general SEM and FCC OOBE, it is proposed to define the coexistence requirement as a middle ground at -30 dBm/MHz.  The proposals in this contribution are reiterated below.

Proposal:  EARFCN for 3 MHz channels other than 899 MHz shall be disallowed in the specification.

Proposal:  An emission limit of -30 dBm/MHz is specified for UE coexistence to protect the downlinks of Band 5 and Band 26 when the 3 MHz LTE channel is centered at 899 MHz.  No A-MPR is allowed.

# Reference

- RP-230722, “Revised WID: Introduction of 900 MHz LTE Band in the US,” Anterix

- RP-230723, “Revised WID: Introduction of 900 MHz NR Band in the US,” Anterix

- R4-2303486, “WF on US 900 MHz bands,” Qualcomm Incorporated

- FCC 20-67.  https://docs.fcc.gov/public/attachments/FCC-20-67A1.pdf