3GPP TSG-RAN WG4 Meeting # 106-bis-e												R4-2305915

Electronic Meeting, 17 April –26 April, 2023

Agenda item:			5.19.3

Source:	Intel Corporation

Title:	WF on absolute physical layer throughput requirement

Document for:	Approval

# <Test parameters and simulation assumptions >

Issue 1-1: Maximum number of HARQ transmission

- Agreement: Define ATP requirements with HARQ ReTx enabled

  - Further effort on the alignment of simulation results can be performed.

Issue 1-2: Precoding for HARQ retransmission

- Agreement: Tentative offline agreements confirmed as below

o For retransmission number 4 including initial transmission, RV {0,2,3,1} with same MCS and rank as initial transmission; follow the latest UE reported PMI whose rank is same as the initial transmission

Issue 1-3: Understanding of TX EVM for ATP simulation

- Agreement: RAN4 clarify the Tx. EVM as below

  - If BS support Tx with 256QAM, Tx EVM should be 3.5% or less, according to BS RF requirements (TS 38.104 Table 6.5.2.2-1).

For ATP requirements, RAN4 assume the following Tx EVM. Note this is for simulation purpose. - FR1 - TX EVM = 6% for QPSK/16QAM/64QAM, when 64QAM CQI Table is configured - TX EVM = 3% for QPSK/16QAM/64QAM/256QAM, when 256QAM CQI Table is configured - FR2 - TX EVM = 6% for QPSK/16QAM/64QAM, when 64QAM CQI Table is configured.

Note) RAN4 don’t need to rerun the simulation with this statement.

- Interested company may update the simulation results in the next meeting

# <Phy Layer TP requirement specification >

Issue 2-1 Phy Layer TP test metric

- Average SNR of impairments results to achieve T% of maximum throughput + X dB margin

  - Use Gspan = [2.5] dB to check if the results are aligned

  - The maximum throughput is defined as with TBS corresponding to CQI index 15 with rank Y for 2Rx/4Rx UE, e.g., Y=2 for both 2Rx/4Rx UEs.

Issue 2-1-1: Test point T (%) selection

- Candidate test pool based on alignment simulation results in R4-2304257

| Test Case | Candidate test point 1: T % (SNR, Gspan) | Candidate test point 2: T % (SNR, Gspan) |
| --- | --- | --- |
| FR1 FDD 2x2 | 10% (3.2 dB, 1.6 dB) 15% (6.5 dB, 2.1 dB) | 40% (18.5 dB, 2.4 dB) |
| FR1 TDD 2x2 | 10% (3.6 dB, 1.9 dB) 15% (7.1 dB, 1.8 dB) | 35% (17.5 dB, 2.4 dB) |
| FR1 FDD 2x4 | 15% (3.3 dB, 1.9 dB) | 50% (15.2 dB, 2.5 dB) |
| FR1 TDD 2x4 | 15% (3.6 dB, 2.2 dB) | 50% (15.4 dB, 2.8 dB) 35% (10.7 dB, 2.3 dB) |
| FR2-1 (TDD 2x2) | 15% (4.4 dB, 2.1 dB) | 40% (15.1 dB, 1.8 dB) |

- Issue A. # of test point(s)

  - Option 1. Two test points for 2Rx and 4 Rx.

  - Option 2. Two test points for 2Rx and 1 test point for 4 Rx.

  - Option 3. Single test point for 2Rx and 4 Rx

- Issue B. T(%) and outlier removal for Gspan <= 2.5 dB

Issue 2-1-2: X dB margin

  - Option 1: Apply X dB margin per modulation order to impairment results

X = [0.5] dB for QPSK, X = [0.5] dB for 16QAM

X = [0.8] dB for 64QAM, X = [0.8] dB for 256QAM

Note that the modulation order would be based on median value over companies’ own median statistics on CQI.

  - Option 2: Apply single X dB margin to impairment results (e.g. X=0.5 dB or else)

WF for Issue 2-1-1 and 2-1-2

- Discuss in the next meeting with the following aspect based on the updated results if any.

- SNR options considering uniqueness of test SNR coverage

- T (%) based on alignment results considering Gspan

- Final SNR with X dB margin is within range of [0 20] for FR1 and [0 16] for FR2.

- It does not preclude the possibility of T(%) adjustment with [+- 5% steps] or removal of outlier results from Gspan perspective.

# <Applicability and release independence >

Issue 2-2: Applicability and release independence

- Option 1: The requirement with link adaptation should be applicable for all NR UEs without any new applicability rules, and the requirement should be release independent from Rel-15

- Option 1.a: Proposal considering declaration

- Optional for Rel-15 and Rel-16 UEs based on declaration - Mandatory for all Rel-17 and forward UEs.

- Option 2: The absolute physical layer throughput requirement with link adaptation should be applicable from Rel-18 and not release independent from Rel-15.

# <Work plan and CR work split >

Issue 2-3: Work plan and CR work split

- Consider Split 2 and prepare CR text in the next meeting

- Moderator will communicate for CR TP alignment before the next meeting

| Section |  |  | Split 1 | Split 2 |
| --- | --- | --- | --- | --- |
| FR1 (5.X) | 1 Rx | Void | NA | NA |
|  | 2 Rx | FDD | Company A | Company A [Intel] |
|  |  | TDD |  | Company D [Samsung] |
|  | 4 Rx | FDD | Company B | Company E [CMCC] |
|  |  | TDD |  | Company B [CTC] |
| FR2 (7.X) | 1 Rx | Void | NA | NA |
|  | 2 Rx | TDD | Company C | Company C [Nokia] |