3GPP TSG-RAN WG4 Meeting # 106bis-e												R4-2305935

Electronic Meeting, April 17 – April 26, 2023

Agenda item:			5.34.5

Source:	Moderator (Qualcomm Incorporated)

Title:	WF on mIAB RF requirements and coexistence

Document for:	Approval

# Introduction

This document provides way-forward on mobile IAB (mIAB) based on the outcomes of “Topic summary for [106-bis-e][314] NR_mobile_IAB_RF.

# RF requirements

## mIAB output power and power control

Agreement:

- Postpone the discussion on this issue till the outcome of the RAN4 co-existence study

# Coexistence study

## Carrier aspects

Carrier and frequency configuration

Agreement:

- Focus on case 1a below for Rel-18 mIAB coexistence study.

| Case | MT Carrier | DU Carrier | RAN carrier |
| --- | --- | --- | --- |
| 1a | Same carrier(s) |  |  |
| 1b | MT-dedicated carriers | Same carrier(s) |  |
| 1c | Same as RAN carriers | DU-dedicated carriers | RAN carriers |
| 1d | IAB-dedicated carriers |  | Dedicated carrier(s) |
| 2a | Band A |  | Band B |
| 2b | Band A | Band B |  |

Multi-band operation

Agreement:

- Multi-band cases are not relevant for Rel-18 mIAB.

Multiplexing configuration

Agreement:

- Only TDM operation between IAB-MT and IAB-DU is to be considered in co-existing simulation.

## Scenarios

Interference scenarios

Agreement:

- Consider the scenarios listed in Table 1 and 2 below as a starting point.

Table 1:  Scenarios simulated for layout 1&2 between IAB <-> NR

| Deployment Scenario (Aggressor->Victim) | Case No. | Baseline | Aggressor | Victim | Comemnts (related RF parameter) |
| --- | --- | --- | --- | --- | --- |
| IAB -> NR Macro | 1 | NR DL active:UE receiving IAB (unenabled): access link ( IAB-DU) DL inactive, backhaul link ( IAB-MT) inactive | IAB: access link (IAB-DU) inactive, backhaul link ( IAB-MT) DL active : IAB donor transmitting | NR DL: UE receiving | IAB donor ACLR is the same with normal BS, this case could be skipped. |
|  | 2 | NR DL active: UE receiving IAB (unenabled): access link ( IAB-DU) DL inactive, backhaul link ( IAB-MT) inactive | IAB: access link (IAB-DU) inactive, backhaul link ( IAB-MT) inactive : IAB-DU transmitting | NR DL active: UE receiving | IAB-DU Tx power IAB-DU ACLR |
|  | 3 | NR UL active: BS receiving, UE transmitting IAB(unenabled) : access link (IAB-DU) DL inactive, backhaul link (IAB-MT) inactive | IAB: access link (IAB-DU) inactive, backhaul link (IAB-MT) UL active: IAB-MT transmitting | NR, UL | IAB-MT Tx power IAB-MT ACLR |
|  | 4 | NR UL active: BS receiving, UE transmitting IAB(unenabled) : access link (IAB-DU) DL inactive, backhaul link (IAB-MT) inactive | IAB: access link (IAB-DU) active, backhaul link (IAB-MT) UL inactive: IAB-DU receiving | NR, UL | This is legacy scenario, could be skipped. |
| NR Macro -> IAB | 5 | IAB: access link ( IAB-DU) DL inactive, backhaul link DL( IAB-MT) active: IAB-MT receiving NR DL and UL inactive: | NR DL active: BS transmitting | IAB backhaul link: IAB-MT receiving | IAB-MT ACS |
|  | 6 | IAB: access link (IAB-DU) inactive, backhaul link (IAB-MT) UL active : IAB-Donor receiving, IAB-MT transmitting NR UL and DL inactive; | NR UL active: UE transmitting, BS receiving | IAB backhaul link: IAB-Donor receiving | IAB donor ACS is the same with normal BS, this case could be skipped. |
|  | 7 | IAB: access link (IAB-DU) UL active, backhaul link (IAB-MT) inactive: IAB-DU receiving NR UL inactive; | NR UL active: UE transmitting, BS receiving | IAB access link : IAB-DU receiving | IAB-DU ACS |
|  | 8 | IAB: access link ( IAB-DU) DL active, backhaul link DL( IAB-MT) inactive: IAB-DU transmitting, UE receiving NR DL and UL inactive: | NR DL active: BS transmitting | IAB access link: UE receiving | UE and BS are legacy, this scenario can be skipped. |

Table 2:  Scenarios simulated for layout 1&2 between IAB <-> IAB

| Deployment Scenario (Aggressor->Victim) | Case No. | Baseline | Aggressor | Victim | Comemnts (related RF parameter) |
| --- | --- | --- | --- | --- | --- |
| IAB Agressor -> IAB Victim | 9 | IAB victim DL,backhaul link active: IAB donor transmitting, IAB-MT receiving IAB aggressor (unenabled): access link ( IAB-DU) DL inactive, backhaul link ( IAB-MT) inactive | IAB Agressor: access link (IAB-DU) inactive, backhaul link ( IAB-MT) DL active : IAB donor transmitting | IAB victim DL: IAB-MT receiving | IAB donor ACLR is the same with normal BS IAB-MT ACS |
|  | 10 | IAB Victim DL active; IAB agressor (unenabled): access link ( IAB-DU) DL inactive, backhaul link ( IAB-MT) inactive | IAB: access link (IAB-DU) active: IAB-DU transmitting backhaul link ( IAB-MT) inactive : IAB-DU transmitting | IAB victim DL: IAB-MT receiving | IAB-DU Tx power IAB-DU ACLR IAB-MT ACS |
|  | 11 | IAB victim UL: IAB-MT transmitting, IAB-Donor receiving IAB agressor (unenabled) : access link (IAB-DU) DL inactive, backhaul link (IAB-MT) inactive | IAB: access link (IAB-DU) inactive, backhaul link (IAB-MT) UL active: IAB-MT transmitting | IAB victim UL: IAB-MT transmitting, IAB-Donor receiving | IAB-MT Tx power IAB-MT ACLR |
|  | 12 | IAB victim UL: IAB-DU receiving IAB agressor (unenabled) : access link (IAB-DU) DL inactive, backhaul link (IAB-MT) inactive | IAB: access link (IAB-DU) inactive, backhaul link (IAB-MT) UL active: IAB-MT transmitting | IAB victim UL: IAB-DU receiving | IAB-MT Tx power IAB-MT ACLR IAB-DU ACS |

mIAB Antenna installation location

Agreement:

- RAN4 to focus on antenna installation with IAB-MT antenna outside (e.g., rooftop) the vehicle and IAB-DU antenna inside the vehicle.

- FFS on the MT /DU antenna modeling

## Coexistence parameters

Network layout

Agreement:

- In initial co-existence study considering both layout 1 and layout 2, it’s not precluded to do prioritization in later stage.

Inter-mIAB minimum distance

Agreement:

- 5m following RAN4 inter-UE distance assumptions.

- Other values not precluded.

mIAB-BS minimum distance

Agreement:

- 35m along the ground between mIAB and macro-BS for layout 1 and 5m along the ground between mIAB and micro-BS.

Number of mIAB nodes per macro-BS

Agreement:

- 1 mIAB node per macro-BS, following Rel-16 for static IAB.

mIAB antenna height

Agreement:

- 4m.

FR2 EIRP assumptions

Agreement:

- For normal ground UEs, the FR2 UE Tx power could be 22.4 dBm EIRP (13.4 dBm conducted).

- Other options not precluded.

Impacted RF requirements from coexistence study

Agreement:

- Agree on the following preliminary list of impacted RF requirements:

  - Output power

  - ACLR/ACS

  - Other requirements are not precluded.

Remaining system parameters

Agreement:

- Agree on the below table for the remaining system parameters.

| Parameters |  |
| --- | --- |
| Duplex mode | TDD |
| Frequency range | FR1: 4.9GHz – FR2: 30GHz |
| Beamforming | FR1: Yes – FR2: Yes |
| Simulation bandwidth | 100MHz for FR1 200MHz for FR2 |
| Number of UEs in the network | FR2: 1 active UE/sector FR1: 3 active UEs/sector |
| gNB Tx power | 33 dBm for FR2 macro and micro 46 dBm for FR1 |
| IAB node Tx power | TBD for MT and DU link |
| gNB antenna height | 25m for macro cells and 10m for micro cells |
| IAB node antenna height | 4 m for macro cells and 4m for micro cells |
| gNB receiver noise figure | 10dB for FR2 5dB for FR1 |
| IAB node receiver noise figure | 10dB for FR2 5dB for FR1 |
| UE Tx power (dBm) | FR2: 22.4dBm EIRP (13.4dBm conducted) FR1: 23dBm (conducted) |
| UE noise figure | 9dB for FR1 10 dB for FR2 |

## UAV operation

Agreement:

- No need to consider UAV in Rel-18 mIAB RAN4 requirements and co-existence study.

# References

- R4-2305861,” Email discussion summary for [106-bis-e][314] NR_mobile_IAB_RF” Qualcomm Incorporated.