3GPP TSG-RAN WG4 Meeting # 106bis-e													R4-2306173

# Online, April 17 – April 26, 2023

Agenda item:			5.31.4

Source:	Moderator (OPPO)

Title:	Topic summary for [106bis-e][225] NR_SL_enh2_part2

Document for:	Information

Introduction

Briefly introduce background, the scope of this summary (e.g. list of treated agenda items).

This email discussion summary includes RRM Core requirements for R18 SL evolution (5.31.3.1 & 5.31.3.3).

- RRM requirements for SL CA

- RRM requirements for Co-channel coexistence for LTE SL and NR SL

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| LG Electronics | Jin-Yup Hwang | jinyup.hwang@lge.com |
| Ericsson | Santhan Thangarasa | Santhan.thangarasa@ericsson.com |
| Nokia | Lars Dalsgaard | Lars.dalsgaard@nokia.com |
| Intel | Ian Hwang | Ian.hwang@intel.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: RRM requirements for NR SL CA

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| Tdoc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304929 | MediaTek Inc. | Proposal 3: Postpone the discussion on RRM impacts due to NR sidelink CA until further checking in RANP #100. |
| R4-2304701 | LG Electronics Inc. | - Proposal 1: Reuse LTE sidelink CA operation requirements as a baseline - Proposal 2: Introduce interruption to WAN when sidelink UE performs sidelink carrier addition/release as mode 2 operation. - Proposal 3: Do not introduce carrier addition/release delay requirement in NR sidelink CA operation - Proposal 4: Introduce selection/reselection of V2X synchronization reference source in NR sidelink CA operation |
| R4-2304781 | Xiaomi | Proposal 1: The requirements specified for LTE V2X CA could be used as baseline for NR sidelink CA and could be further discussed based on progress in RAN1. |
| R4-2305245 | OPPO | Observation 1: RRM requirements for LTE SL CA can be used as baseline for NR SL CA. Observation 2: The delay requirements for SL component carrier addition/release are not needed for Mode 2 operation and up to UE implementation. Observation 3: The procedure and configuration of Selection / Reselection of Synchronization Reference Source under SL CA operation need to be firstly decided in RAN1/2. Proposal 1: Consider to specify at least the following RRM requirements for NR SL CA - Interruption requirements (e.g., length and/or location) for SL carrier (re-)selection (component carrier addition/release) - Detection and measurement requirements for (re-)selection of synchronization reference source for NR SL CA |
| R4-2305545 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 shall wait for RAN1 to progress on NR sidelink CA feature so that based on RAN2 lead discussion can provide inputs to RAN4 impacts/requirements to be taken up. Proposal 2: RRM requirements in TS 36.133 sec. 13.9 - Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation and sec. 13.10 - Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation for LTE sidelink CA, can be used as baseline for NR sidelink CA in Rel-18. |

The moderator can suggest a limited number of papers which could be presented.

Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: General and scope

Sub-topic description

Open issues and candidate options before f2f meeting:

- Issue 1-1-1: Scope of RRM requirements for NR SL CA

- Proposals

  - Option 1 (Xiaomi, OPPO, LGE, Nokia):

    - The requirements specified for LTE V2X CA could be used as baseline for NR sidelink CA and could be further discussed based on progress in RAN1.

  - Option 1a (Nokia):

    - RRM requirements in TS 36.133 can be used as baseline for NR sidelink CA in Rel-18.

      - sec. 13.9 - Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation

      - sec. 13.10 - Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation for LTE sidelink CA,

- Recommended WF

  - Can we agree on both option 1 an 1a?

| Company | Comments |
| --- | --- |
| QC | Support option 1. |
| LGE | Support option 1 |
| OPPO | Support option 1 and 1a. |
| Xiaomi | Support option 1 |
| Huawei | We can agree with option 1. |
| Ericsson | We suggest alternative wording as follows: “The requirements specified for LTE V2X CA could be used as baseline for NR sidelink CA, and RAN4 to further investigate if there is any NR specific impact for NR SL CA. The requirements could be further discussed based on progress in RAN1.” We disagree to option 1a because it is too early to draw the conclusion on the exact requirements from LTE SL. More discussions are needed. |
| MTK | We are not opposing Option 1. As discussed in issue 1-1-2, RAN4 should not start the work on CA before RAN1 and RAN2. |
| Nokia | Support option 1 and 1a. |
| Intel | Support Option 1 and 1a with some clarification on Sec. 13.9 1) In TS 36.133, there is a Note: For UE configured in sidelink transmission mode 4, the delay is up to UE implementation. 2) For NR SL CA, only Mode 2 (which is equivalent to Mode 4 in LTE SL) is in work scope. Thus, NR SL CA addition and Release Delay is expected to be up to UE implementation if we follow LTE principle. |

- Issue 1-1-2: How to organize RRM discussion for NR SL CA in RAN4

- Proposals

  - Option 1 (MTK): Postpone the discussion on RRM impacts due to NR sidelink CA until further checking in RANP #100.

  - Option 2 (Nokia): RAN4 shall wait for RAN1 to progress on NR sidelink CA feature so that based on RAN2 lead discussion can provide inputs to RAN4 impacts/requirements to be taken up.

  - Option 3 (OPPO): At least the procedure and configuration of Selection / Reselection of Synchronization Reference Source under SL CA operation need to be firstly decided in RAN1/2.

- Recommended WF

  - Other solutions are not precluded. Need more discussion.

| Company | Comments |
| --- | --- |
| QC | All the options are similar, and we don’t see the necessity to make agreement here given that the progress is pending RAN1/2 which is captured in is 1-1-1 already. |
| LGE | For option 1, based on WID, RAN4 discussion is started in this meeting. So RAN4 can discuss NR SL CA based on LTE SL CA principle since NR SL CA would be not much different from LTE SL CA based on WID. |
| OPPO | Agree with QC and LGE. |
| Xiaomi | Agree with QC and LGE. We also think RRM discussion for NR SL CA need more RAN1/RAN2 progress. |
| Ericsson | We have similar view as Qualcomm. We also prefer to postpone the discussions on the requirements until more RAN1/RAN2 progress is reached on this topic. |
| MTK | Fine with Option 1 and Option 2. According to the following guidance from RANP#99, RAN1 will treat SL CA in May only if SL Co-Ex is completed and RAN will check the status in RANP#100 with possible down scoping. We don’t think RAN4 should start the discussion on SL CA before RAN1. |
| Nokia | Agree with QC. Options are very similar and main point is to let RAN4 waith until further RAN1/RAN2 progress. |
| Intel | Same view as QC and LTE. It seems that one of main principle in TS 36.133 13.10 is scale up of required processing time by N where N is the number of aggregated carriers configured as synchronization carrier. |

Sub-topic 1-2: Detailed RRM requirements for NR SL CA

Sub-topic description

Open issues and candidate options before f2f meeting:

- Issue 1-1-3: Interruption requirements when NR SL UE performs SL carrier addition/release

- Proposals

  - Option 1 (LGE):

    - Introduce interruption to WAN when sidelink UE performs sidelink carrier addition/release as mode 2 operation.

  - Option 1a (OPPO): specify at least the following RRM requirements for NR SL CA

    - Interruption requirements (e.g., length and/or location) for SL carrier (re-)selection (component carrier addition/release)

- Recommended WF

  - Need discussion.

| Company | Comments |
| --- | --- |
| QC | Our understanding of option 1 is reusing LTE SL CA requirement with possible modifications due to SCS, under this assumption we can support option 1. |
| LGE | Since mode 2 operation is available for NR SL CA, interruption length can be defined, but it seems to be difficult to define interruption location as mentioned by option 2. |
| OPPO | We can start form option 1, and the details of interruption requirements can be further discussed. |
| Xiaomi | Fine with option1. |
| Huawei | We are fine with option 1 |
| Ericsson | Given that this is the first meeting to discuss NR SL CA, we are fine to have high-level discussions on the RRM impact. We can agree that interruption requirements are needed. But scenarios and details of the interruption requirements e.g. interruption probability, interruption length are FFS. But it is too early to agree on the detailed requirements. For example, there are differences between LTE and NR, when interruptions are needed and length/location of such interruptions need more study. Therefore we don’t support option 1 or 1a. |
| MTK | Pending on issue 1-1-2. |
| Nokia | OK to start with option 1 (same assumption as QC wrt. understanding of option 1) |
| Intel | For Option 1, we are not sure whether WAN is involved in SL CA in mode 2. For Option 1a, is it new requirement introduced in NR SL CA? or is there any similar requirement defined in LTE SL CA in UE autonomous allocation mode? |

- Issue 1-1-4: Delay requirements of carrier addition/release for NR SL CA

- Proposals

  - Option 1 (LGE):

    - Do not introduce carrier addition/release delay requirement in NR sidelink CA operation

  - Option 1b (OPPO)

    - The delay requirements for SL component carrier addition/release are not needed for Mode 2 operation and up to UE implementation.

- Recommended WF

  - Can we agree on option 1b?

| Company | Comments |
| --- | --- |
| QC | Support option 1 and 1b. |
| LGE | Support both options. Option 1 is the same intention with option 1b. |
| OPPO | Option 1 and 1b are similar. ‘Up to UE implementation’ can be explicitly stated in the spec. |
| Xiaomi | Fine with both option1 and 1b. Option 1b is more aligned with delay requirement for LTE SL CA . |
| Huawei | WE agree with option 1b. |
| Ericsson | What does it mean delay requirements are up to UE implementation? Our view is that even for mode 2, the requirements need to be clear. Without clear requirements, the performance cannot be guaranteed. But we are also fine to keep the exact requirements FFS to allow more time for companies to check, e.g. RAN1/RAN2 progress. |
| MTK | Pending on issue 1-1-2. |
| Nokia | Option 1&1b are not contradicting. Fine with both. |
| Intel | Support option 1 and 1b. 1) In TS 36.133, there is a Note: For UE configured in sidelink transmission mode 4, the delay is up to UE implementation. 2) For NR SL CA, only Mode 2 (which is equivalent to Mode 4 in LTE SL) is in work scope. Thus, NR SL CA addition and Release Delay is expected to be up to UE implementation if we follow LTE principle. |

- Issue 1-1-5: Requirements of Selection / Reselection of Synchronization Reference Source under NR SL CA operation

- Proposals

  - Option 1 (LGE):

    - Introduce selection/reselection of V2X synchronization reference source in NR sidelink CA operation

  - Option 2 (OPPO):

    - The procedure and configuration of Selection / Reselection of Synchronization Reference Source under SL CA operation need to be firstly decided in RAN1/2.

    - Specify at least the following RRM requirements for NR SL CA:

      - Detection and measurement requirements for (re-)selection of synchronization reference source for NR SL CA

- Recommended WF

  - Need discussion.

| Company | Comments |
| --- | --- |
| QC | Option 2 aligns with issue 1-1-1 option 1 better. |
| LGE | Fine with both options. |
| OPPO | Agree to start the discussion on requirements of Selection / Reselection of Synchronization Reference Source for SL CA. Option 2 is slightly preferred. |
| Xiaomi | Fine with both options. |
| Huawei | We agree to wait RAN1/RAN2 progress on the procedure of synchronization reference source selection/reselection under NR SL CA operation. |
| Ericsson | We are fine with option 2. Since this objective has just started in RAN1/RAN2, it may be premature to identify the exact requirements. But we can agree that at least the detection and measurements requirements shall be defined and RAN4 to further discuss other impacted requirements. |
| MTK | Pending on issue 1-1-2. |
| Nokia | Both options are fine. Option 2 is slightly more detailed. |
| Intel | Fine with Option 1 and Option 2. Option 2 seems to be more specific. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1: General and scope | - Issue 1-1-1: Scope of RRM requirements for NR SL CA Tentative agreements: - RRM requirements specified for LTE V2X CA could be used as baseline for NR SL CA. The requirements could be further discussed based on progress in RAN1/2. - FFS: Interruption requirements of SL carrier addition/release for NR SL CA - FFS: Delay requirements of carrier addition/release for NR SL CA - FFS: Requirements of Selection / Reselection of Synchronization Reference Source under NR SL CA operation - RAN4 to further investigate if there is any NR specific impact for NR SL CA. - Issue 1-1-2: How to organize RRM discussion for NR SL CA in RAN4 Tentative agreements: N/A Candidate options: - Option 1: Postpone the discussion on RRM impacts due to NR SL CA until further checking in RANP #100. - Option 2: Start RAN4 discussion on RRM impact due to NR SL CA from this meeting (RAN4#106bis). Recommendations for 2nd round: Depending on the discussion of updated Work Plan. |
| Sub-topic #1-2: Detailed RRM requirements for NR SL CA | - Issue 1-1-3: Interruption requirements when NR SL UE performs SL carrier addition/release Tentative agreements: - RAN4 to specify interruption requirements for NR SL CA - FFS scenarios and details of the interruption requirements e.g. interruption probability, interruption length, interruption location. - Issue 1-1-4: Delay requirements of carrier addition/release for NR SL CA Tentative agreements: - FFS: RAN4 not to specify delay requirements for SL component carrier addition/release for Mode 2 operation in this WI and it is up to UE implementation. - Issue 1-1-5: Requirements of Selection / Reselection of Synchronization Reference Source under NR SL CA operation Tentative agreements: - The procedure and configuration of Selection / Reselection of Synchronization Reference Source under SL CA operation need to be firstly decided in RAN1/2. - Specify at least the following RRM requirements for NR SL CA: - Detection and measurement requirements for (re-)selection of synchronization reference source for NR SL CA - RAN4 to further discuss if any other impacted requirements. |

Topic #2: RRM requirements for Co-channel coexistence for LTE SL and NR SL

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| Tdoc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2304703 | LG Electronics Inc. | Proposal: Wait for further RAN1 progress and conclusion for co-channel coexistence for LTE SL and NR SL. |
| R4-2304783 | Xiaomi | Proposal 1: The existing RRM requirements are applicable for the co-channel coexistence solution of TDM-based semi-static resource pool partitioning. Proposal 2: For co-channel coexistence solutions of dynamic resource sharing and FDM-based semi-static resource pool partitioning, RAN4 to wait for further progress in RAN1. |
| R4-2305247 | OPPO | Proposal-1: Postpone the discussion on co-channel coexistence between LTE sidelink and NR sidelink until significant progress is reached in RAN1. |
| R4-2305547 | Nokia, Nokia Shanghai Bell | - RAN4 awaits RAN1 outcome for solutions for synchronization between LTE and NR for co-channel coexistence. - RAN4 to wait for RAN1 to complete the resource (re-)selection procedure for a Type-A device before RAN4 continues to discuss the impact to the test for V2X UE autonomous Resource Selection/Reselection. - RAN4 to determine a set of evaluation parameters for NR SL and LTE SL coexistence. This can be with two perfectly aligned resource pools (in time and frequency) as well as equal subchannel sizes. - No enhancements for the IDC framework from Rel-16 is needed to support Type A devices for LTE and NR co-channel coexistence. |
| R4-2305324 | Huawei, HiSilicon | Proposal 6: The existing scheduling availability requirements for UE switching between LTE SL and NR SL can be reused for co-channel coexistence in Rel-18. |
| R4-2304929 | MediaTek Inc. | Proposal 2: LTE SL and NR SL coexistence has no impact on RRM core part requirements yet. FFS whether to specify test case for NR SL resource (re)selection procedure with dynamic resource sharing. |

The moderator can suggest a limited number of papers which could be presented.

Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1: General aspects for co-channel coexistence for LTE SL and NR SL

- Issue 2-1-1: General

- Proposals

  - Option 1 (LGE, Xiaomi, OPPO, Nokia):

    - Wait for further RAN1 progress and conclusion for co-channel coexistence for LTE SL and NR SL.

  - Option 1a(Nokia):

    - RAN4 awaits RAN1 outcome for solutions for synchronization between LTE and NR for co-channel coexistence.

    - RAN4 to wait for RAN1 to complete the resource (re-)selection procedure for a Type-A device before RAN4 continues to discuss the impact to the test for V2X UE autonomous Resource Selection/Reselection.

  - Option 2 (MTK):

    - LTE SL and NR SL coexistence has no impact on RRM core part requirements yet. FFS whether to specify test case for NR SL resource (re)selection procedure with dynamic resource sharing

- Recommended WF

  - Need discussion.

| Company | Comments |
| --- | --- |
| QC | We see impact of dynamic resource sharing between LTE and NR, but agree that it is pending RAN1 agreement. |
| LGE | It depends on RAN1 design, so we prefer option 1. |
| OPPO | Option 1. |
| Xiaomi | Support option 1 |
| Huawei | We can agree with option 1. Currently, no RRM impact is observed. |
| Ericsson | We also support option 1. |
| MTK | No RRM impact has been observed yet. But we are fine to wait for further RAN1 progress. |
| Nokia | Both Option 1 and Option 1a are fine for us. |
| Intel | Support Option 1 |

- Issue 2-1-2: Whether to reuse existing RRM requirements for Co-channel coexistence for LTE SL and NR SL

- Proposals

  - Option 1 (Xiaomi):

    - The existing RRM requirements are applicable for the co-channel coexistence solution of TDM-based semi-static resource pool partitioning.

  - Option 2 (Huawei):

    - The existing scheduling availability requirements for UE switching between LTE SL and NR SL can be reused for co-channel coexistence in Rel-18.

- Recommended WF

  - Need discussion.

| Company | Comments |
| --- | --- |
| QC | Option 1 and 2 are not contradictory. Option 1 is valid given that the RRM requirement apply to the partition of the resource pool to NR. |
| LGE | Fine with both options. For dynamic resource pool case, we need to wait for RAN1 conclusion. |
| OPPO | Suggest to wait for RAN1’s conclusion on co-channel coexistence solutions. |
| Xiaomi | Support option 1.Also fine to wait for RAN1 conclusion |
| Huawei | Support both options. |
| Ericsson | As commented in issue 2-1-1, RAN4 shall wait for further RAN1 progress and conclusion for co-channel coexistence for LTE SL and NR SL. The detailed requirements can be discussed after that. |
| MTK | Both the options are ok to us. |
| Nokia | Would prefer to await RAN1 to conclude. |
| Intel | Same view with OPPO. Suggest waiting for RAN1’s conclusion on co-channel coexistence solutions. |

- Issue 2-1-3: Others

- Proposals

  - P1: RAN4 to determine a set of evaluation parameters for NR SL and LTE SL coexistence. This can be with two perfectly aligned resource pools (in time and frequency) as well as equal subchannel sizes.

  - P2: No enhancements for the IDC framework from Rel-16 is needed to support Type A devices for LTE and NR co-channel coexistence.

- Recommended WF

  - Need discussion.

| Company | Comments |
| --- | --- |
| QC | It’s not obvious to us why we need to discuss these proposals. |
| LGE | We are not sure whether both proposals are related to RRM requirements. |
| Ericsson | The rationale for these proposals should be clarified. As they are now, we cannot agree to those. It seems P1 is more RAN1 related since RAN1 is doing LTE SL and NR SL co-channel coexistence study. P2 is also not RAN4 related. |
| MTK | P2 is right to us. But as pointed by other companies, this is not RRM related. If P1 refers to the parameters for test cases, this should be discussed when defining test cases. |
| Nokia | These can be discussed when RAN1 has completed to SL-CoEx work. |
| Intel | Need further clarification on the purpose of this topic. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1: General aspects for co-channel coexistence for LTE SL and NR SL | - Issue 2-1-1: General Tentative agreements: - Wait for further RAN1 progress and conclusion for co-channel coexistence for LTE SL and NR SL. - Issue 2-1-2: Whether to reuse existing RRM requirements for Co-channel coexistence for LTE SL and NR SL Tentative agreements: N/A Candidate options: - Option 1: The existing RRM requirements are applicable for the co-channel coexistence solution of TDM-based semi-static resource pool partitioning - Option 2: Wait for RAN1’s conclusion on co-channel coexistence solutions, e.g., for dynamic resource pool. Recommendations for 2nd round: N/A - Issue 2-1-3: Others Tentative agreements: No more discussion. Candidate options: RAN4 to discuss the following proposals when RAN1 has completed to SL-CoEx work. - P1: RAN4 to determine a set of evaluation parameters for NR SL and LTE SL coexistence. This can be with two perfectly aligned resource pools (in time and frequency) as well as equal subchannel sizes. - P2: No enhancements for the IDC framework from Rel-16 is needed to support Type A devices for LTE and NR co-channel coexistence. Recommendations for 2nd round: N/A |

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on NR_SL_enh2_part2 | OPPO | To capture the agreements and open issues |

Existing tdocs

| Tdoc number | Revised to | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-22xxxxx |  | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2304929 |  | MediaTek Inc. | Noted |  |
| R4-2304701 |  | LG Electronics Inc. | Noted |  |
| R4-2304781 |  | Xiaomi | Noted |  |
| R4-2305245 |  | OPPO | Noted |  |
| R4-2305545 |  | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2304703 |  | LG Electronics Inc. | Noted |  |
| R4-2304783 |  | Xiaomi | Noted |  |
| R4-2305247 |  | OPPO | Noted |  |
| R4-2305547 |  | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2305324 |  | Huawei, HiSilicon | Noted |  |
| R4-2304929 |  | MediaTek Inc. | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents