3GPP TSG-RAN WG4 Meeting # 106-bis-e	R4-2306340

Electronic Meeting, 17 April –26 April, 2023

Title: 	WF on simultaneous multi-panel operation for train roof-mounted FR2 high power

devices

Agenda Item:	5.13.4

Source: 	Samsung

Document for:	Approval

# Way Forward on simultaneous multi-panel operation for train roof-mounted FR2 high power

## Sub-topic 1-1 General discussion on RRM requirement impact

Agreement:

- Measurement based on multi-panel RX:

  - Simultaneous L1 (L1 + L1) measurements

    - FFS the conclusion from Rel-18 FR2 multi-Rx can be reused, and check if any enhancements are needed for HST FR2 scenario

  - Simultaneous L1 + L3 measurements

    - Simultaneous L1 on one panel and L3 measurements on the other panel (i.e., L1 +L3)

      - FFS, whether to reuse the conclusion from Rel-18 FR2 multi-Rx: i.e., simultaneous L1 and L3 measurements is not supported for Rel-18 FR2 HST.

    - Simultaneous L3 on one panel and L1 measurements on the same panel

      - Simultaneous L1 and L3 measurement on the same panel is not supported for Rel-18 FR2 HST

    - FFS scheduling restriction relaxation for L3 measurements

  - Simultaneous L3 + L3 measurements

    - FFS, whether L3+L3 measurement on different panels shall be supported in Rel-18 FR2 HST enhancement WI.

Agreement:

- Measurement and data reception based on multi-panel RX:

  - Simultaneous data reception and L1 measurement on different panels

    - FFS the conclusion from Rel-18 FR2 multi-Rx can be reused, and check if any enhancements are needed for HST FR2 scenario

  - Simultaneous data reception and L3 measurement on different panels

    - FFS the conclusion from Rel-18 FR2 multi-Rx can be reused, and check if any enhancements are needed for HST FR2 scenario

Agreement:

- Simultaneous data reception from both panels

  - Simultaneous data reception shall be supported in Rel-18 FR2 HST

Agreement:

- Scheduling restriction

  - Data+ L1 measurement

    - FFS the conclusion from Rel-18 FR2 multi-Rx can be reused, and check if any enhancements are needed for HST FR2 scenario

  - Data+ L3 measurement

    - FFS the conclusion from Rel-18 FR2 multi-Rx can be reused, and check if any enhancements are needed for HST FR2 scenario

Agreement:

- UE capability

  - The Rel-17 group-based reporting may be be considered in the FR2 HST:

    - FFS the necessity of Rel-17 group-based reporting in the FR2 HST

    - FFS the applicability of Rel-17 group-based reporting in the FR2 HST

## Sub-topic 1-2 MRTD and panel switching for FR2 HST multi-panel simultaneous reception

### Issue 1-2-1: MRTD for FR2 HST multi-panel simultaneous reception

Agreement:

- Rel-18 FR2 PC6 UE should support simultaneous data reception from two panels with MRTD more than the CP length.

### Issue 1-2-2: Turning on/off the multi-panel reception operations in FR2 HST

Agreement:

- FFS a mechanism for turning on/off the multi-panel reception operation in FR2 HST:

  - FFS the necessity of such a mechanism

### Issue 1-2-3: Discussion on the necessity of signalling to UE used by the UE to perform the switching

Agreement:

- FFS the necessity of new signalling to UE used by the UE to switch between multi-panel and single-panel simultaneous reception with respect to NW configuration

## Sub-topic 1-3 RX beam sweep number for FR2 HST multi-panel simultaneous reception

### Issue 1-3-1: Rx beam sweeping factor reduction

Agreement:

- FFS whether the sweeping factor N1 can be reduced

- How to reduce the sweeping factor N1

  - FFS for L1-RSRP measurement requirements, for set1 configuration the value can be further reduced to 1, for set2 configuration the value can be further reduced to 3.

## Sub-topic 1-4 DL Transmission Scheme for FR2 HST multi-panel simultaneous reception

### Issue 1-4-1: Discussion on DL transmission scheme

Agreement:

- Preclude the SFN-based PDCCH transmission scheme for open space deployments in Rel-18 FR2 HST work item.

## Sub-topic 1-5 UL and DL beam switching for FR2 HST multi-panel simultaneous reception

### Issue 1-5-1: Discussion on UL transmission scheme

Agreement:

- Simultaneous UL transmissions with 2 panels is not supported in Rel-18 FR2 HST WI.

### Issue 1-5-2: Whether enhancements are needed in uplink spatial relation switch delay requirement

Agreement:

- Whether enhancements are needed in uplink spatial relation switch delay requirement

  - Option 1: RAN4 not to consider UL spatial relation switch to the target associated to the unknown DL RS in HST FR2 scenarios

  - Option 2: Keep current uplink spatial relation switch delay requirement

  - Other options are not precluded

## Sub-topic 1-6 Deployment scenarios and RF chains configuration

### Issue 1-6-1: CPE configuration

Agreement:

- FFS the RRM requirement impact due to the configuration of Rel-18 FR2 HST PC6 UE:

  - RAN4 to assume that HST FR2 enhanced CPE supports 4 layers (Rank 4) in DL

  - [To support rank 4 in DL, RAN4 to assume that HST FR2 Enhanced CPE is equipped with at least 2 simultaneous active Rx panels (each with dual polarized reception i.e., Rank-2)]